## CHAPTER 1

## Quality in Manufacturing –

## General Concepts

Chapter 1 Quality in Manufacturing. General Concepts

1.1 Introduction

Every company entrusts its managers to define its goals.  Typically, goals are established for sales levels, profits, products, service to the customers, finances, personnel, strategic planning, and the like.  Establishing goals for the quality of a company’s products or services is absolutely essential to assuring its future.  Products and services are your company’s connection to its customers and the only reason for its existence.  Without customers – satisfied customers – a company will not survive

This Manual describes a three or a five-day Training Program for Quality Management in Photovoltaics, focused on helping PV manufacturers adopt methods leading to consistent quality in products or services (see Appendix VI, Training Schedule). The methodology is based on a concept of “total quality,” which requires commitment at all levels of a company to deliver quality products to its customers all the time. The Training Program further explains how your PV company can achieve and maintain recognition of the quality of its products and  certification by world-recognized authorities and how your company could profit from it.

1.1.1 The Photovoltaic (PV) Industry

If they are to achieve their full market potential, photovoltaic (PV) systems must be cost effective and must also meet and, ideally, surpass customer expectations. Most older applications for telecommunications and space use have proven the cost-effectiveness of PV technology, because of their high reliability and performance.  New large scale terrestrial PV applications must similarly demonstrate reliability and long life to prove cost-effectiveness; therefore the PV industry will rise or fall, based on the quality of PV products.  Ultimately, consumers must see economic value from their investment in PV systems, above and beyond the satisfaction from opting for a form of renewable energy with no detrimental impact on the environment.

Manufacturers that do not produce quality products cannot survive long in the marketplace. Therefore, every manufacturer must be concerned with the quality of  company products. Quality management processes vary from manufacturer to manufacturer, but the goal is always the same: to ensure that products and processes meet specified expectations.

Photovoltaic products are manufactured in many countries and sold all over the world. They are also supplied in response to tenders, such as those issued for projects sponsored by the World Bank. Putting in place global standards and quality procedures that are accepted everywhere will  assure a uniform quality of PV products.  Equally important, it will also benefit the individual manufacturer by broadening  markets and making quality products competitive everywhere.

Manufacturing a quality product requires designing quality into the product from the outset.  Assuring a quality product requires more than post-production inspection, which simply separates acceptable products from unacceptable ones or “rejects.” Rejects at the end of the production line are really added expenses for the manufacturer.  Instead, a consistent quality manufacturing process, based in statistical measures, maintains quality and allows for improved customer satisfaction as well as added profits for the manufacturer.

As a participant in this Training Program, you will be given tools your company can use to manage and improve the quality of its products and, thereby, enhance the value provided to the customer.  When  value surpasses  cost, the market will support the development and promotion of hundreds of new PV applications. In such a scenario, the initial expense of putting quality systems in place will be recouped many times over, thereby confirming the wisdom of investing in quality.

  - Elements of a Quality Manufacturing System

The elements of a globally acceptable quality system are:

- A quality manufacturing process based on ISO 9001:2000 Standard

- Products manufactured according to a standard

- Products tested and the same models periodically re-tested in an approved testing laboratory

- Auditing by an outside organization

- Products that provide customer satisfaction

The Photovoltaic Industries have identified these elements  as the basic components for a program to guide end users towards selection of companies producing quality products.  In order to establish a quality system for PV with easy  recognition of quality products and worldwide acceptance, the Global Approval Program for PV (PV GAP) was set up to incorporate these elements and issue a Mark or a Seal of distinction.  Information of the PV GAP Program is given in the following chapters.

1.3 Global Approval Program for PV (PV GAP)

### 1.3.1  Why is there a Need for a Global Approval Program for PV?

Photovoltaics have proven to be one of the most reliable sources of electricity.  Practically all satellites in orbit are powered reliably by PV systems and, for the past 30 years PV has been successfully used for many terrestrial applications.  Some PV modules are now sold with a 25-year warranty.

PV system reliability is a sine qua non for ensuring confidence in the PV industry.  Lack of consistency of quality in PV products could jeopardize the credibility of both the industry and its underlying technology. It could also dampen customer acceptance and limits the interest of potential financiers.

This has happened in recent years in terms of inconsistencies in the quality of terrestrial PV components, systems, installations, and after-sales maintenance, especially in off-grid rural electrification projects deployed by local manufacturers and integrators in developing countries.

1.- The virtual absence of standards for anything but PV modules is a major cause of quality inconsistency in solar home systems,

2.- Manufacturers lack uniform guidelines on how to produce reliable systems or components and on how to install or service them.

3.- The lack of accredited testing laboratories has also complicated product testing for many small and medium manufacturers.

4.- Even when testing has occurred, problems still arise. After submitting samples of their PV products to a testing laboratory and receiving a test report indicating compliance, some manufacturers have labeled all of the same type of product as tested by the testing lab, with no assurance that the tested product was truly representative of the manufacturer’s production, since no audit of the manufacturer’s production process had been carried out.

Given these serious quality issues, the global PV industry, supported by financial institutions and government agency funding programs jointly agreed on the need for a global PV quality assurance program.    The result, with the guidance of the PV industry and the PV community, was the formation of the Global Approval Program for Photovoltaics ( PV GAP) in 1997.

BOX 1.1

The PV GAP Mission Statement PV GAP, a not-for profit international organization, is dedicated to the sustained growth of global photovoltaics (PV) markets to meet energy needs world-wide in an environmentally sound manner. Our mission is to promote and encourage the use of internationally accepted standards, quality management processes and organizational training in the design, fabrication, installation, sales and services of PV systems. To this end, we partner with PV and related industries, international organizations, testing laboratories, government agencies, financing institutions, non-governmental organizations, and private foundations, in developing and developed countries

1.3.2  The Elements of the PV GAP Approval System

The PV GAP approval system includes the elements of a globally acceptable quality system as noted in section 1.2.  It also provides the approved manufacturer with a distinctive PV Quality Mark for PV components and a  Seal for systems.  The Quality Mark and Seal serve to help customers  recognize a quality product and distinguish it from a product of unknown quality. The elements of the PV GAP approval system are summarized in Figure 1.1:

PV GAP APPROVAL SYSTEM

Figure 1.1

### PV GAP

Requirements for approval:

- Quality Manufacturing (ISO 9001:2000)

- Product manufactured and tested according to International Standards (IEC or PVRS) by the manufacturer testing facility (included in his ISO 9001:2000 certification) or by an accredited  (ISO/IEC 17025) Testing Laboratory

- “Periodic Audit” of the manufacturer and the product

PV GAP function:

- Based on IECQ approval, PV GAP licenses the use of the Mark and/or Seal

The following is a brief description of the requirements for approval of a quality management  system for manufacturers:

1.3.3 ISO Standards for Quality in Manufacturing

The ISO 9001:2000 standard is an international quality management system for any organization. It is applicable to PV manufacturers as well as installers, service providers, distributors and the like.  The system is a proven methodology to enhance the performance of an organization that delivers products or services to its customers and is trying to enhance customer satisfaction and improvement of the quality system.

Several years ago, the application of this standard was somewhat complex, and only large companies, with extensive quality departments initiated its use.  Today, many companies are initiating its use due to the benefits they receive.  The design and implementation of the quality management system varies according to the needs and objective of the organization, and also it’s size and structure. PV GAP developed a detailed but simple Training Manual which simplified and provided forms and methods to benefit  small Photovoltaic organizations. The PV GAP “Manufacturing Quality Process Manual” (see the Bibliography) was written and reviewed by many organizations, including manufacturers in both industrialized and developing countries.  They found the Manual, with its examples and instructions, to be very effective in helping small and medium-size companies to develop a quality manual, and  implement the quality management process. The World Bank sponsored PV GAP’s preparation of this Training Manual in 1999. The Manual was tested in several training courses held for PV manufacturers.

A number of consulting organizations have recently developed programs to help small and medium size manufacturers  implement the ISO 9000:2000 quality system. Because  several such consulting organizations now exist, and the process is simplified and better understood, the cost of  implementation of the ISO 9000:2000 system has become very affordable.

The revision of ISO 9000 and the 9001 standards in  2000, necessitated an update of the Manufacturers Training Manual developed for the World Bank so as to incorporate all the changes introduced by the new ISO 9001:2000 version as well as changes in the PV GAP approval process since release of the Manual in 1999.  This current Manual  incorporates all the above changes.

1.3.4  Standards for the Performance of Products

Why is a standard needed for product performance, when a quality manufacturing process already assures that every product is manufactured within an acceptable range?

The answer is simple. A quality manufacturing process assures that every manufactured product falls within acceptable limits.  But alone it does not assure that the resulting product is functional.  For example, one might manufacture life jackets from concrete. The quality manufacturing process assures that they are identical in quality.  But in order to be functional, the life jackets have to be made to certain standards.  One standard could be that the life jacket has to keep a 100-kg person afloat. The life jacket then has to be tested according to that standard.

Standards will be discussed extensively in Chapter 5 of the course and in  associated sections of this Manual.  As indicated above, the development and utilization of globally accepted PV standards is a pressing need in the PV industry today. The IEC (International Electrotechnical Commission) standards are  accepted worldwide and already include some standards relevant to PV. More are needed.  As a bridging mechanism, PV GAP has established the mechanism of “PV GAP Recommended Specifications” for use until final IEC standards can be established.

BOX 1.2 Quality manufacturing of products according to standards results in quality products that are useful and durable and are capable of meeting requirements

# 1.3.5  Testing Laboratories

Normally, a manufacturer  sends a product to an approved testing laboratory. If it passes the test, the manufacturer will receive a test report approving the product.

What is an approved testing laboratory? The worldwide accepted criteria for the approval of a testing laboratory is compliance with ISO/IEC 17025, whose rules have been adopted by both the ISO and IEC.  This Standard establishes the quality process and procedures for a testing laboratory.

At present there are only a few testing laboratories in the entire world for PV, even though testing laboratories are ideally needed in every country where PV products are being manufactured. A key PV GAP task is to help establish and approve as many testing laboratories as possible. The names of approved PV testing laboratories are listed on PV GAP’s web site (www.PVGAP.org).

1.3.6  Auditing by an Outside Organization

The requirement that a manufacturer’s quality system be reviewed and its products verified and tested by an independent organization and testing laboratory does not stem from   mistrust of the manufacturer. The main purpose of this requirement is to assure reciprocity.  Reciprocity ensures that  products are accepted everywhere in the world and that no re-testing will be needed in other countries. Naturally, the outside audit also safeguards honest manufacturers and customers from unscrupulous operators.

An outside organization must approve the manufacturer’s compliance with the ISO 9001:2000 standard. Furthermore, in order to maintain an ISO 9001:2000 approval, periodic auditing of the manufacturer’s quality system by an outside organization is required.  The audit process should also verify that the manufacturer has a PV product  tested in an approved testing laboratory, and that it is going to be re-tested after a certain time or when the manufacturer makes certain changes in the product.

The purchase or acquisition of a solar PV system by a customer is a significant event. The customer is putting himself, or an organization or company, at the forefront of wise and environmentally friendly use of energy.  But the reliability of the system is equally significant for the customer. He or she must be assured that the whole solar PV system, including all  components, have been made and tested according to international standards for reliability and durability.

How can the individual customer be reasonably sure that the performance of manufacturers, distributors, dealers and installers concerned meet these standards?  Should they merely be taken at their word? For everyone's peace of mind, an independent check or audit is essential to verify that standards have really been followed. That independent activity is called certification of conformity. Without periodic outside audits, the door would be left open for unscrupulous competitors to disappoint customers and damage the reputation of the entire PV industry.

### 1.3.7  Globally Utilized “PV Quality Mark and Seal”

When samples are tested by an approved testing laboratory, can the manufacturer print the words “Tested by XY Testing Laboratory” on every product of the same type?

The answer is “no.” Since only some samples were tested, labeling every product of the same type, “Tested by XY Testing Laboratory” would be false and misleading unless the samples are representative of the manufacturing quality and the manufacturer has proven a consistent quality process which has been established and verified. Customers need to be able to easily distinguish “quality products” from products of unknown quality.  For PV products, PV GAP has adopted a labeling system similar to that used successfully in other industries (e.g. UL for electrical safety, the “Wool Mark” for wool products, the Green Seal for environmental friendliness, and the like). The PV Quality Mark is used for PV components, while the PV Quality Seal covers complete PV systems.  (More information about the Mark and Seal will be provided in the section 6-4).

1.3.8 PV GAP Program Administered by a Globally Accepted Approval Body

PV GAP has elected to use an existing and internationally accepted approval process, the IECQ (International Electrotechnical Commission Quality Assessment System for Electronic Components), which is administered by the International Electrotechnical Commission (IEC).  PVGAP has arranged for IECQ  to administer the PV GAP approval program. PV GAP’s responsibility is to implement the program and coordinate worldwide efforts to ensure PV system reliability and durability and, if the manufacturer’s product is approved by IECQ, to issue a license to the company to authorize  display of the PV GAP PV Quality Mark and/or Seal on the product.

The IECQ system:

- requires no maintenance fee from the PV industry,

- has an approval system for testing laboratories and establishes global reciprocity of all test results

- establishes a global auditing procedure, with which manufacturers are periodically checked.

1.3.9 How Expensive is it to Set up and to Operate a Quality System?

Obvious benefits accrue to the manufacturer in being able to display the PV GAP PV Quality Mark or Seal on a product.

1.- It distinguishes the manufacturer’s quality product from products of unknown quality not having the Quality Mark or Seal.

2.- Besides  the marketing advantage gained by increased customer confidence in the products,

3.- Another important advantage is that products displaying the Quality Mark or Seal are accepted in every country, while those not  qualified with the Mark or Seal must gain independent credentials in every country.

4.- Allows the purchase of system components tested under known conditions, specifications and standards, therefore assuring a good quality and reliable performance

As an example, the World Bank has advised its clients (see www.worldbank.org/energy) to  accept products displaying the PV GAP Mark and Seal.  Those not having the Mark or Seal must be re-certified country by country according to the country’s requirements.

Table 1.1  THE REWARDS OF QUALITY

| Leveling the playing field | - Customers can differentiate between quality products and products of poor or unknown quality. - The same quality standard is used globally. |
| --- | --- |
| Reliability and durability of PV products | - The reputation of PV products is important. - Financing of PV products will become easier. |
| Customer satisfaction | - Market expansion will be a natural result. |
| Participation in tenders | - Tenders require quality products. - Globally uniform quality standards make it possible for tenders to specify the same testing. |
| Opening export markets | - Reciprocity of testing will open export markets. |
| Customer recognition of quality products by the “PV Quality Mark and Seal” | - The “PV Quality Mark and Seal” will help all companies, large or small, differentiate their products from products of inferior quality. |
| Cost reduction by manufacturing improvements | - The reduction of inline rejects and the finished product non-conformity has been reduced by 10 % in most companies where the system is established. |
| Cost reduction for system companies buying PV GAP marked products | - Purchasing components from a company that is approved to use the Mark and Seal do not have to retest these products, because they are identified as good quality and meeting all specified requirements. - Significant savings and confidence in the products purchased gives a significant advantage |

What costs does a PV  manufacturer incur in obtaining the PV GAP approval to  display the PV Quality Mark and/or Seal on  products? Can a small or medium size company afford these costs?

Apart from testing expenses, the cost of setting up a quality process that meets requirements for a manufacturer,  varies with the size of the company. Appendix 1 provides an example for a  company producing 1MW/year and having 50 employees..  The example assumes that the manufacturer has no established system and that the modules will have to be tested in an accredited testing laboratory.  The calculation indicates that the total cost is $ 0.06 per Watt in the first year and $ 0.0025 per watt in the second year.  As the savings from the quality system are an order of magnitude higher, the implementation of the system is a significant saving for the organization rather than a cost in the long term.

ELEMENTS OF THE GLOBAL APPROVAL PROGRAM FOR PV

(PV GAP)

Figure 1.2

Washington DC and Geneva. The World Bank now recognizes the approval of products displaying the Global Approval Program for Photovoltaics (PV GAP) Quality Mark (for PV components) and Seal (for PV systems) for Bank sponsored projects. The announcement encourages Bank staff to inform its clients undertaking PV projects about this decision (http://www.worldbank.org/energy).

The decision of the World Bank states

“PV components or systems eligible for use in Bank-assisted projects should either comply with: (a) relevant standards issued by IEC, ISO or similar standards/certification organizations of international standing, (b) appropriate national standards, or (c) bear the Global Approval Program for Photovoltaics (PV GAP) Mark or Seal”

“This visual recognition simplifies for customers the selection of quality products. It also eliminates the need of country-by-country re-certification, as the credentials of the PV manufacturer and the product were established by PV GAP”.

“This eliminates, for products so marked, the review of the credentials of each product a PV manufacturer wants to sell for a project supported by the World Bank. This simplifies administrative procedures, is less costly to suppliers, gives the buyers a greater confidence in the quality of the products, and can encourage inter-country trade”.

Manufacturers may therefore choose

- to have their credentials re-certified country by country or

- (simpler and less expensive) to obtain a PV GAP license to display the Mark or Seal and receive global acceptance for their products.

The World Bank Group is supporting PV projects in India, Indonesia, Sri Lanka, China, Vietnam, Philippines, Lao PDR, Morocco, Argentina, Mexico, Honduras, Togo, Benin, Cape Verde, Uganda, Kenya, and elsewhere.

The Global Approval Program for Photovoltaics (PV GAP) (http://www.pvgap.org), established in 1997 by three PV associations (EPIA, JEMA, SEIA), is a not-for-profit international organization, registered in Geneva, Switzerland. It promotes the utilization of quality PV products and distinguishing them from PV products of unknown quality. PV GAP is achieving this through a close working relationship with the International Electrotechnical Commission (IEC), (http://www.iec.ch). PV GAP has formal liaison with IEC's Quality Assessment System for Electronic Components (IECQ) (http://www.iecq.org), which is currently providing supplier approval, product approval and laboratory accreditation services for PV GAP.

## Box 1.3   WORLD BANK RECOGNITION OF PV QUALITY MARK AND SEAL

1.4  The Evolution and Merits of Quality Systems

Since the early 1970’s, manufacturers in the United States have used a variety of quality improvement systems (QIS) to improve product and service quality as a means of reducing product costs and gaining market share.  Their initiative was a response to growing competition from Japanese manufacturers, who were producing products that were leading the world in quality, reliability and cost.  The basic philosophy of QIS emphasizes control of the manufacturing process with the aim of being statistically capable of producing products within established performance limits, thereby avoiding defective products.

QIS represented a significant departure from prior quality efforts in which manufacturers used end-of-line inspection techniques to prevent defective products from reaching customers.  End-of-line inspection is costly due to the high value of products at that point, and not effective enough since some defective products still reach customers.  End-of-line inspection of products could not compete with manufacturing processes that were under statistical process control.

The Japanese manufacturers were the first to implement QIS or Total Quality Control (TQC) on a large scale as part of a strategy to gain competitive advantage in world markets.  Even though many of the QIS procedures and the concepts of statistical process control were developed in the United States in the 1930’s, Japanese manufacturers in the 1960’s and 1970’s were the first to embrace these techniques. Their success in capturing world market share demonstrated the value of QIS and the need for change.

To regain market share, United States and European manufacturers started to introduce their own QIS.  For example, American car manufactures have made significant improvements in the quality of cars produced in the United States and have regained market share over the past twenty years.

Following the success in manufacturing, service companies such as banks, insurance companies, hotels have implemented QIS in order to gain a competitive advantage.  In addition to commercial entities, governments and international organizations have also implemented QIS, although at a slower pace. QIS applies equally well to small manufacturers and large ones, since customers expect and deserve quality products and services regardless of size.  The lesson learned by the United State car manufacturers is that quality products and services are necessary to survive in global markets.

In most manufacturing companies, QIS starts with the quality manual.  The quality manual establishes the policies, procedures and control for the manufacturing organization, as well as the documentation and audit system to ensure compliance.   The main elements of QIS are described in the quality manual and include organization charts, job descriptions, product and process specifications, process control statistics, production flow diagrams, use of shop floor teams for problem solving, and training of the entire organization in QIS, just to name a few.

Any discussion of quality must start with a common definition, which must be easily understood by the entire organization and must be measurable.  A good exercise to demonstrate different perceptions of quality is to ask members of an organization their definition of quality products.   Some usual responses are:

- Quality products are reliable.

- Quality products produce value.

- Quality products have many features.

- Quality products perform well.

- Quality products have long warrantees.

- Quality products have a good appearance.

All of these are correct responses and can be classified into product attributes that satisfy customer needs. These attributes are considered part of a transaction between a supplier (manufacturer, integrator or installer) and a customer where the customer establishes the criteria for the transaction through a negotiation with the supplier. A simple definition of quality is “satisfying customers’ needs.”  In essence, the transaction becomes a quality contract specifying the customer’s needs and the supplier’s deliverables.

A similar list could be developed for quality services provided by a manufacturer.  These include order information, complaints, order fulfillment and other requests coming from customers.  A separate exercise should be done for quality services.

Since needs vary from customer to customer, it is important to establish each customer’s needs through direct negotiation.  The needs must be quantitative and well documented, in order to prevent confusion about when the transaction is completed and satisfied; or, if a need is not satisfied, to clarify what corrective actions are required.  Based on many surveys and interactions with customers, manufacturers have learned that all customer product-related needs fit into the following three major categories:

- price and terms

- product or service definition and performance

- time schedules.

A customer will be satisfied, if all three category needs are satisfied by the transaction.  Only by satisfying all three simultaneously, can the manufacturer be considered to have provided a quality product.  Customer needs must be measurable, so that the performance of a manufacturer can be quantified and tracked.  This promotes comparisons among competitors.

Using the definition of quality as satisfying customer needs, many examples of both good and poor quality products and services can be found.  These include a piece of furniture that is not the same color as the one seen in the showroom, a restaurant that serves consistently delicious meals, or the computer program that does not operate properly.   A good exercise to do with a group is to identify examples of poor and good quality based on personal experiences and the specific customer needs that were involved.

True quality manufacturers who embrace QIS can consistently produce quality products and services over a long period of time.  These world-class manufacturers provide product designs and production processes that are capable of meeting the quality criteria of customers.  It is not sufficient to have a good design for the engineering sample, if the production processes are unable to reproduce the performance of the design on a predictable basis. For example, manufacturers that have unreliable equipment with frequent breakdowns cannot meet customer’s time schedules.  Manufacturers that have high reject rates leading to high production costs cannot consistently meet the customer’s price.  And manufacturers that have uncontrolled production processes cannot meet customers’ product performance specifications.

A manufacturer can achieve world-class status by motivating and training the entire organization in the principles of QIS, including compliance with the policies and procedures listed in its quality manual.  ISO 9000 can help establish the framework for the documentation system used in the quality manual to assure compliance with QIS.

The implementation of QIS for an organization must be comprehensive and involve all employees, from management to operators.  Resources must be committed to training the organization about QIS and the need for a team-based organization.  While small companies are more likely to function in a team environment, it is nonetheless important to emphasize the critical need for team participation by the workforce in any successful QIS process.

Training in statistics, control charts, cause and effect diagrams, and brainstorming are important elements of an effective QIS process. Frequent internal and external auditing of procedures, as required by ISO 9000, are essential for a successful QIS process.  In addition, the workforce must understand that customers, subcontractors and vendors also exist within the organization and that the same quality criteria used for external customers apply to interactions between departments and employees.

And lastly, clear accountability and responsibility for all activities in the organization must be established through job descriptions and organization charts.  These, in turn, must be communicated and made available to each employee. QIS and ISO 9000 can be effective tools in producing products of consistent high quality, but they require management’s commitment to develop an organization to achieve and accept nothing less than doing the job right the first time.

1.5 Quality Manufacturing Process.

Introducing the IEC, IECQ, and ISO

The IEC (International Electrotechnical Commission) and the ISO (International Organization for Standardization), together with the ITU (International Telecommunication Union), are the three bodies responsible for standardization and related matters at the international level.  They work closely together and occupy adjacent premises in Geneva, Switzerland.  Figure 1.3 shows the relationship between IEC and ISO.

The IEC, through more than one hundred technical committees, prepares international standards for the electrotechnical field and is also parent to several autonomous certification schemes, one of which is the IECQ (IEC Quality Assessment System for Electronic Components).

The ISO and ITU take care of all other fields (some jointly with the IEC).  The ISO, however, also has a special committee for developing jointly with the IEC what are called ISO/IEC Guides, many of which are fundamental to the whole activity of conformity assessment.  With the existence of these internationally accepted guides, the rules for an approval scheme like the IECQ can be concise.  In many cases, such as approval of testing laboratories, reference can simply be made to the relevant guide (in that case, Guide 17025).

The ISO is a worldwide federation of national standards bodies (ISO member bodies). The edition of the ISO 9001 standard, which is discussed in this course and manual, was issued in the year 2000.

Instituting the ISO 9001:2000 standard in manufacturing assures that every product will have similar quality. This means that each product falls within an acceptable range, which in turn means there are potentially no rejects at the end of the production line. If there are any rejects, corrective action is taken immediately and rejects are not allowed to leave the factory.  It also means that a continuous evaluation process will take place for the review of the process and its potential improvement.  The goal of the standard is also to achieve customer satisfaction at all times.

Figure 1.3 How ISO, IEC and IECQ are Related

IEC

ISO

NATIONAL

STANDARD

BODIES

ISO/IEC 17025

Testing Laboratories

Also

Guide 62, 65, etc.

Conformity Assessment

Board

Standardization Management Board

NATIONAL

TECHNICAL

COMMITTEES

Technical committees

TC176

TECHNICAL

COMMITTEES

TC 82

Photovoltaics

Subcommittee

SC2

Quality systems

handbook

Subcommittees

IECQ

WORKING GROUPS

(7)

ISO 9001:2000

QUALITY SYSTEMS

STANDARDS

STANDARDS

PRODUCT APPROVAL / CERTIFICATION SYSTEM

1.6  ISO Standards for Quality In Manufacturing

The quest for quality is neither new nor a passing fad.  It is an essential need for any business in today’s competitive global economy.  Customers expect that the products they buy will perform as advertised and meet their expectations. To meet customer expectations, a company must first know what its customers want and then do its very best to deliver it.    Otherwise, the customer may choose to buy from a competitor the next time.

Many companies sincerely want to deliver a quality product, but they do not plan their activities in a way that makes it possible to meet this goal.  If you want quality, you have to implement it in a consistent way.  The quality effort cannot be left to chance; it must be managed very carefully and persistently.  Only through continuous effort and planning can the quality of products be improved.

Other companies may have already an existing Quality system, but it may be not well documented or organized, therefore leading to undetected deficiencies that curtail an easy correction of problems.

This training program provides the framework and tools for a system to produce quality products and services and its proper review and audit of activities to enhance its performance. The objective is not to make a complex system that will generate lots of paperwork.  The objective is to develop a system as simple as possible that will generate quality products and services and will protect the company’s interest and make it a financial success.  The successful experience with quality systems and its benefits and costs will be studied.

In 1979, the ISO (International Organization for Standardization), with the support of several industries and quality organizations, established a series of international standard practices with the goal of setting up generic quality systems applicable everywhere.  The ISO standards are used to manage quality once methods and procedures are developed. The past methodology of inspecting every final product to assure quality was changed into a process in which quality considerations are introduced during the design of the product or service.  With this new method in mind, quality is managed at every stage in the production process, and the company does not depend solely on the inspection of its products after completion.

Once designs and procedures are established that can render a quality product that meets the desired specifications, the quality system takes over to assure that the processes are strictly observed to maintain a consistent quality level. The objective is to be able to make a good product the same way thousands of times.

1.6.1  What is ISO 9000?

ISO 9000 is a series of generic quality management standards that have the goal of organizing the way a company carries out the quality functions.  It has been designed to be flexible for a wide range of requirements and situations.  Whether a business is large or small, the ISO 9000 system allows the organization to manage the business in a systematic and documented way that prevents inconsistencies caused by individual deviation from accepted methods and procedures.

The ISO 9000 family of standards promotes the responsibility of all the Company’s personnel in establishing processes for the quality of the end product or service. It includes all  personnel, including the management and all those involved in the manufacturing activities and development and design.  The process includes reviewing the resources available and the planning of the quality functions.  It promotes the establishment of quality goals, responsibilities and measurements as well as general quality management improvements.

The ISO 9001:2000 standard has been recently released. It changed the 1994 version in the approach to management of the system.  It has similar elements of the quality system (applicable to any business) but its approach has been changed into a process methodology to enhance its efficiency.  In general, the standard tells a company what it must do to implement a quality system without dictating how it should do it. It does not specify what kind of processes must occur, but it does require that appropriate quality activities be defined and documented and that the company proves its consistent adherence.  Guidelines explain which elements are necessary for maintaining certification. The implementation process is intended to be as simple as possible, in order to minimize paperwork and records, while ensuring that essential records are kept to verify that quality is being maintained. Processes must be documented and consistent to maintain quality.

The ISO 9000 methodology leads to the measurement of performance, therefore promotes the use of statistical measurements and controls to document and measure performance of the quality system.  The data is used for the continuous analysis and management review of the methods and procedures that potentially can improve quality and performance. The rest is up to the company. Using this quality philosophy, a company will achieve consistent quality, as well as the ability to continuously improve it.

1.6.2 Quality System Requirements

The ISO 9000:2000 standard includes a set of concepts, principles and the vocabulary common to all quality management systems.  All the requirements for certification are specified in ISO 9001:2000.  In the 1994 version of the ISO 9000 standards there was three different series, the 9001, 9002 and the 9003, which were  applicable to companies that deal with design, manufacturing and services (9001), companies that only deal with manufacturing and services (9002) or companies that only deal with final testing and inspection (9003) of their products or services.

This distinction has been eliminated from the new ISO 9000:2000 version.  There is now a unique set of requirements for all companies irrespective of  products, service, or size. The requirements listed in ISO 9001:2000 apply to all companies. Those elements found irrelevant for the type of business can be excluded from the process, but they must be identified.

The ISO 9001:2000 contains all the normative references and is the basic requirement for obtaining certification. It demonstrates the capability of a company to satisfy the customer needs.  The ISO 9000:2000 also contains two important separate standards: the ISO 9004: 2000 which is strongly recommended in order to pursue the continued improvement process that will make a business a world class business. This standard is not essential to achieve the certification.

It is also related to  ISO 19011, a standard that provides guidance in audits for quality and environmental management systems.  All these standards are part of the ISO 9000:2000 standard system and show the same basic principles.

TABLE 1.2-   ISO 9000: 2000 FAMILY OF STANDARDS

| ISO 9000-2000 | Quality Management Systems Fundamentals and Vocabulary | This Standard describes the concepts of the Quality System and establishes a process approach to the pursuit of quality management. |
| --- | --- | --- |
| ISO 9001-2000 | Quality Management Systems Requirements | This Standard addresses the requirements for an organization to demonstrate its capability to meet customer needs all the time |
| ISO 9004-2000 | Quality Management Systems Guidelines for Performance Improvements | This Standard addresses the need for a Continuous Improvement Process in order to Enhance Customer Satisfaction |
| ISO 19011-2002 | Guidelines for Auditing Quality and Environmental Systems | This Standard is currently under review. It addresses the need for Audits of the Quality and Environmental Systems. |

This Training Manual is designed to help your company meet the requirements of the ISO 9001:2000. The Manual reviews the implications and philosophy of the ISO 9001:2000 System and the way to implement the various requirements of ISO 9001:2000 and to achieve Certification.

1.6.3  General Description of ISO 9000:2000

The fundamental concept of the new Quality Management System is the approach to Quality Management as a process model, in which several processes are interlinked to achieve the business goals as well as  satisfying customer needs and  possibly exceeding customer expectations.

The Standard develops a series of elements that are required to achieve  customer satisfaction, but which are dependent on the type of business, either manufacturing or service, and which your organization must define in its Quality Manual as necessary or not.  The system is very flexible and adaptable to all sorts of businesses; large or small and either manufacturing or service organizations.  The only requirement is that the Quality Manual must justify any of the elements that are not necessary to achieve the service or product.

The new Standard is based on Eight Quality Management Principles that are the guidelines for the Management of the Organization to achieve the quality goals:

1.-Customer focus: The main goal of a business is to understand and fulfill the customer’s expectations.  The success of a business is based on customer satisfaction.

2.- Leadership: The company management must establish the direction of the organization. They must foster an environment where the people see their interest and involvement in the quality and other goals of the organization by participation, review and support.

3.- Involvement of people: All members of the organization must be unified and involved in the pursuit of the goals within their operating parameters.  All employees should identify their contribution and responsibility towards achieving customer satisfaction.

4.- Process approach:  The desired service or product requires several activities, which are always intended for a customer, internal or external. The management of resources and time for the required activities is optimized when this approach is taken.

5.- System approach to management:  Each individual is responsible to add some value to the previous step and fulfill the expectations of the next.  This approach allows management to more efficiently and effectively understand and identify the needs and problems and achieve the goals and objectives of the organization.

6.- Continual improvement:  This is a major goal since the organization benefits  by  continuous improvement efforts.

7.-  Factual approach to decision making:  Decisions and  changes shall be based on factual data or information in order to be effective and reproducible.

8.- Mutually beneficial supplier relationship:  Good supplier relationships are a key organization goal. A close relationship and quality performance is beneficial and indispensable for the ability to create value and satisfy  customer needs.

Process approach:  Under this approach, any activity that uses resources to transform inputs to outputs is identified as a process.  Several of these processes are linked to each other until the output is the desired product or service.  Identifying processes and managing the interactions among processes allow for their improvement, thus making output efficient  and assuring a more effective use of resources.

The terminology in this standard has changed in order to adapt it to the “process” approach.  The organization is the entity that is intending to use this certification.  The “supplier” is the sub-contractor or the entity that supplies materials or services to the organization, which then are processed and transformed into the delivered product or service to the “customer”.

These terms differ considerably from all previous issues of the Standard and must be revised carefully if previous documentation exists.

Supplier  		Organization		Customer

Continual Improvement of

The quality management system

Customers

And other interested parties

Customers

And other interested parties

Management responsibility

Satisfaction

Measurement analysis and improvement

Resource management

Requirements

Product

realization

Product

Input						          Output

Value adding activities

Information flow

Figure 1.4- MODEL OF A PROCESS-BASED QUALITY MANAGEMENT SYSTEM

Source: ISO 9001-2000

The process starts by a full understanding of the customer needs and requirements, which is labeled as the “input” in the chart, and is linked by the product realization process or processes which result in a product or “output” which is delivered to the customer; therefore the process starts at the customer and is FOR the customer.  The information flow loop is the feedback that the customer makes regarding satisfaction with the results of the organization effort in providing the “product”.

The quality management approach emphasizes the control and evaluation of the effectiveness and efficiency of the various processes that changed the input into the output and utilizes the customer feedback to implement corrective actions if necessary.  Each process is evaluated based on its added value, and emphasis is made in the interfaces between the various processes and their impact in the resultant product.

The methodology typically used for this evaluation is called PDCA, which means Plan, Do, Check and Act.  It proposes that every action should be planned, implemented, review the details and data of the implementation and then act accordingly and consistent with the results of the action taken.  The purpose of this methodology is a continual evaluation of the performance of the processes and their interfaces so as to  improve the output quality continuously.  The use of ISO 9004:2000 in this effort is recommended.

The model indicates an internal flow of information coming from the customer to the box labeled Measurement, analysis and improvement.  This information and data must be presented to the Management and proper decision should be made based in the information obtained.  If flaws are found in the quality system, corrective action is taken and if additional resources are needed in the form of equipment, space or human resources these should be provided or planned.  Changes in the process are typically identified in this feedback.

These activities provide management with the information and tools needed to improve the quality management system, which is represented with the outside arrow focusing in the Continual Improvement Box.

1.6.4 Components of the Quality Management System

The ISO 9001:2000 has changed the previous 20 elements and 18 documented procedures of the quality management system to 5 processes and six basic documentation requirements.  This does not mean that other documentation may not be useful; but only that it is not a requirement.

The five basic processes and their supporting functions are summarized as follows:

- General Requirements	(PLAN and ACT)

- Definitions

- Documentation requirements

- Quality Manual

- Control of documents

- Control of Records

- Management Responsibility		(PLAN  and  ACT)

- Management commitment

- Quality Policy

- Quality planning

- Responsibility, Authority and Communication

- Management Review

- Human and other Resources		( DO)

- Resource Management

- Human resources

- Infrastructure

- Work environment

- Planing and Product Realization	(DO)

- Customer related processes

- Design and development

- Purchasing

- Production and service provisions

- Control of Measuring devices

- Monitoring and Measurement		(CHECK and ACT)

- General considerations

- Customer satisfaction

- Internal audit

- Control of non-conforming products

- Analysis of data

- Improvement

All these processes and their components are analyzed in the following chapters of this Manual.  Where documentation is a requirement, Forms will be supplied in addition to typical documentation forms used in the past for the convenience of the process.

1.6.5  Definitions:  (From ISO 9000:2000)

Organization	A group of people and facilities with an arrangement of responsibilities, authorities and relationships.

Supplier	An organization or person that provides a product. This product is transformed by the organization into its output or product.  In general terms the supplier could be internal or external to the organization. Sometimes this is called a contractor.

Process	A set of interrelated activities which transforms inputs into outputs.

Customer	An organization or person that receives a product.  Can also be defined as a client, end-user or purchaser.  These are external customers.  In the process approach there are internal customers, which are the recipients of the output of the various processes that integrate the system.

Customer Satisfaction  Refers to the perceived degree of fulfillment of  customer needs and requirements.  This is the best measure for the efficiency and effectiveness of the Quality system.

Contract	This term applies to all transactions between a customer and an organization that provides a product or service. It may be verbal or documented.  For example:

A purchase order

A telephone order or a verbal order in a restaurant

A written agreement to deliver a new product with specified requirements

ISO Section 3.3 defines a contract as “agreed requirements between supplier and customer transmitted by any means.”

Product	Any form by which a business can provide a saleable outcome of its activities to meet a customer requirement.

ISO defines it as the result of interacting activities and processes of the supplier that transforms input into output.

Non-conformity	The non-fulfillment of a specified requirement.  It could be internal (during processing) or external at or after delivery.

Words Used in the ISO Standards

“Should/May”	Used with reference to things that are recommended to make the quality system effective. They are never used to indicate a requirement that must be fulfilled.

“Shall”	Used with reference to things that must be followed or carried out, whenever they occur in the standards.

Glossary of terms  An extensive number of terms and definitions are included in Appendix 9.

1.6.6  ISO Documentation Levels

This is a basic and fundamental step in the conformance with the Standard.  All auditors, internal or external will evaluate the performance from its adherence to the documentation available.  It is very important therefore that  documents reflect what is being done and not what is desired.  The documentation should be basic and simple.  Most companies have existing processes and procedures that their experience indicates work adequately; those should be described in a most simple and clear but effective way.  There is no need to create lots of paper work and new procedures.  The system should be as simple and brief as possible but consistent with the methods and processes used.

Documentation is of key importance in the quality system. At the top of the hierarchy of documentation is the Quality Manual. It must contain references to all processes and documents and indicate where they can be found; it must also explain the procedures for changing processes and documents.  It does not need to contain all the technical or processing details, but it must include references to them. The various kinds and levels of documentation are briefly described below.

Level A - Quality Manual

This document reflects the company’s quality policy.  It includes the reference to the operating processes and their interaction.  During external audits, this is the most critical document to follow. It should provide a comprehensive view of the overall system and is frequently used to show customers the quality procedures and philosophy of the company.

Level B - Quality Operating Procedures

Quality Operating Procedures  (QOPs) explain exactly how a company’s quality management plan and policies are implemented. They describe each operating procedure, stating its purpose, who is responsible for it, when and how it should be done, and the associated quality records.  The operating procedures are audited frequently, especially in critical operations. Usually these audits are carried out by observation or interview.  The operating procedures aim at assuring that the person doing a task will always do it according to a pre-determined procedure, which prevents a  non-conformity of the product.

Level C – Operator Work Instructions

Work instructions are usually written in narrative format, but are sometimes presented as charts or flow diagrams.  These instructions are descriptions of activities that take place at each workstation or work center.  They detail how and when the product arrives and what to do with it prior to sending it to the next processing step.  Work instructions may or may not include the details of how to perform a given activity, which typically are part of level D.

Level D - Other Instructions,  Specifications and Records

This level includes the records and results of activities achieved or provides evidence of the activities that have been performed.  It also includes additional documentation required for training and certification of operators, task instructions, maintenance instructions, job descriptions, technical specifications, standards, fire and safety codes, regulatory requirements, equipment operation training, etc.

Figure 1.5 Typical Documentation Levels for ISO 9001:2000

1.6.7  ISO Quality Definitions

A number of definitions differentiate the meaning of the quality terminology used by the ISO.9000:2000 Standard

Quality	Degree to which a set of  characteristics of a product or service fulfills requirements.  It includes the ability to satisfy stated or implied needs.

Quality Policy	The overall intentions and direction of an organization with regard to quality as formally expressed by top management.

Quality Management	That aspect of overall management that coordinates activities, and directs the organization and  implements the company’s quality function.

Quality System	The organizational structure, responsibilities, processes, procedures, and resources for implementing quality management.

Quality Control	The operational techniques and the activities used to fulfill the requirements of quality.

Quality Plan	A document from management, which sets out quality objectives and identifies the specific quality practices, resources, and activities relevant to a particular product, process, service, contract, or project.

Quality Assurance	All those planned or systematic actions necessary to provide adequate confidence that a product or service will satisfy given requirements for quality.

IMPORTANT NOTE: All the forms in the following sections of this Training Manual have been numbered according to the appropriate clause of the ISO requirement followed by a sequential number for the various forms that correspond to that clause.

1.7  The Quality Management System (Clause 4.0)

1.7.1  General Requirements	(Clause 4.1)

What Does Quality Mean for Your Business?

Quality means something different for every business.  In general, quality is typically defined with reference to the manufacturing of products to meet customer expectation for performance, cost, effectiveness and on-time service.  However, your business must define quality for itself.  It should begin to do so by identifying specific customer expectations for products or services. This can be done by customer surveys and interviews, focus groups, market studies and other means of gathering both qualitative and quantitative information.  These expectations can be stated or implied such as anticipated regulatory requirements or meeting established product standards.

Before we start the establishment of the system, the organization has to decide why they want to implement the system.  This is a strategic decision and must be taken based on a benefit for the performance of the organization from a commercial or a competitive point of view.  This decision is of great importance to convince internal and external customers of the advantages of a certification and facilitate continuous support in the future.

Quality objectives should be established only once the company is clear about customer needs and expectations.   Every aspect of the business should have measurable quality objectives.  Management should ensure that every employee understands the quality objectives and precisely how his or her job contributes to their accomplishment.  In short, quality should not be a vague or abstract concept, but a set of clear expectations and actions that contribute directly to customer satisfaction.

The system shall also demonstrate that in order to preserve the level of customer satisfaction a continuous review of the performance is established to maintain a proper quality improvement process of the system.

Reviewing the General Requirements

The Standard ISO 9001:2000 requires here that an overall description of the system be outlined.  This step requires several actions that shall be made by the management of the organization.

The intent is to identify the processes that will be the basic components of the service or product to be delivered to the customer.  Once the processes or steps are identified and its input and output defined, the organization must proceed with the following actions:

- Identify processes.

- Determine their sequence and their interaction; such as requirements for inputs and outputs.

- Define documentation, criteria and methods of control and measurement of each process.

- Provide the resources and information required by each of the process to achieve success.

- Set-up a system to audit, review and monitor progress and results of these processes.

- Implement any corrective action required for the proper operation and the improvement of each process, specially the most important ones.

- Make sure that any process performed outside the organization is properly documented and controlled.  Set up a process for control of non conforming materials or products purchased.

The easiest way to this step is to create a flow chart that uses task blocks and decision diamonds.  It should be made based on the present steps taken in your business and the interface between them.  The critical points are the interfaces between the steps and the requirements between them, such as booking of a sales order and providing a manufacturing group with the details of the product required, standard or special, and the delivery date, special requirements for delivery or measurements, etc.  All these items are further defined in the subsequent steps of the standard.  At this point an overall chart is sufficient.  More detailed charts can be developed to describe further needs such as for a Purchasing Agent:

- Supplier identification

- Supplier selection

- Purchasing procedures

- Supplier evaluation and certification

- Verification of purchased product

- Procedures for rejection of materials that do not meet specifications

It is most important that a continuous review and up-date of these processes be made to maintain a good control of:

- How things are made

- Who makes them and

- When they are made.

Updating the system allows the continuous improvement desired by this Standard but proper communication and documentation of these changes is indispensable for the smooth operation of the system.

A simple overview of a Work Flow Diagram (Form 4.1) could be used in a typical manufacturing operation to start the development of the Process Flow Diagram

WORK FLOW DIAGRAM  (Form 4.1-1)

Customer Related Process

Product Design Review Process ——>Yes——>Design Process

If No Product Design, then

Manufacturing Process

Purchase Components ——>Yes——>Outside Vendor/Product Assessment

If Company does not Purchase Finished Components, then

Collect Components From Inventory/Storage

Product Assembly		    Yes		Assembly Process Steps

Product Testing Process      Yes       Equipment Calibrated      No     Calibration Process

Yes		Test Products		Yes

Product Testing Not Carried Out

Delivery Process:

Product Packaging (Include Manual/Product Specifications)

Product Shipment/Delivery

Customer Receipt/Approval and Customer Feedback

Customer Support and Education. Service Agreement

Documentation of Product Quality and Management review

Corrective and Preventive Actions for future orders

## Homework Assignment – Day One

YOUR NAME:  _______________________________

YOUR COMPANY:  ___________________________

Today you began to learn about the general requirements of quality systems and the requirements for PV GAP.  Ultimately, you will need to tailor a system that meets your company’s unique needs and creates a framework and structure within which it will operate to ensure that its results or products are known, consistent and repeatable.  The homework assignments during this training are designed to help you think about how to apply what you learn here when you go back to your company and be able to organize the effort to qualify for ISO 9001:2000 and for PV GAP Mark or Seal achievement.

The first step in designing a quality system is to step back and carefully consider exactly what happens in your business and what the implications are for the type of quality system it should implement.   Your first homework assignment is to do just that in two steps, as follows:

Determining why the ISO standard applies to your business is essential.  Why do you feel receiving certification can improve your Company.  To do so, answer the following questions:

- Does your company do design work?  This entails taking raw concepts and – through drawings, computer designs, or academic thought processes – developing a product, service or project plan to meet the needs of your customers?

___  Yes

___  No

- (If you do not design) Does your company manufacture products or perform services under existing standards and specifications?

___  Yes

___  No

- Does your company deliver a very simple, fully verified product or service for which it only provides final tests and visual inspection?

___  Yes

___  No

Does your company deliver custom designs based on detailed customer specifications?

____.Yes

____ No

Does your company have to meet special statutory and regulatory requirements?

____.Yes

____ No

### Based on your responses above, state if your company needs to fulfill all the requirements of ISO 9001:2000 and if not give a detailed explanation of where you feel there should be exceptions.

On a separate sheet of paper, make a detailed flow chart showing the sequence of the processes involved in what your company manufactures, services or sells.  For each item on the flow chart, indicate the specific steps taken, workflow, paperwork and other activities involved in its accomplishment.  Make sure that you not only capture what happens within the company, but what happens with regard to outside suppliers and subcontractors.  Put question marks beside the areas where you do not have the information.

This could be the first draft of the process flow.  You may highlight the interfaces where most likely troubles or difficulties may develop.

Chapter 2 -  ISO 9001:2000 Standard

2.1  Documentation Requirements  (Clause 4.2)

2.1.1  General Requirements (Clause 4.2.1)

The ISO 9001:2000 Standard requires that a Documentation system shall be established.  It has reduced the minimum required procedures from the 1994 version.  The main reason behind the changes is to make it easier to apply to various sizes or types of organizations.  The organization therefore is supposed to implement as many procedures as required to make its processes efficient and effective but must maintain a basic list of documented procedures.  The need for extra documented procedures will be dependent of the complexity of the processes and the training and skills of its employees.

The required documented procedures and the sections that describe them are:

- Document control						4.2.3

- Control of records						4.2.4

- Internal quality audits						8.2.2

- Control of nonconforming products and materials		8.3

- Corrective action						8.5.2

- Preventive action						8.5.3

A documented procedure implies that the procedure has been established, documented, implemented and is being maintained and reviewed for potential  improvements.

There are also several basic documents that are required independent of the size or type of organization, they are:

- Quality policy

- Quality objectives

- Quality manual

- Documents needed by the organization to ensure effective planning, operation and control of its processes

- Documented procedures (including those listed above) as required by the Standard

- Records required

These documents should be as simple and effective as possible.  Extensive paperwork should be avoided when it is not directly contributing to the effectiveness and efficiency of the quality system.  This is one of the main goals of the revision of the Standard.

Several activities are important and they may be included as part of your documented system.  These could be work practices, work procedures, operating instructions, maintenance records etc.  These  procedures  are included as information and instructions for the personnel so they can do their job consistently, or in some cases, like production schedules or audit schedules, serve as means of communication with the various departments in a larger organization.

In general, the documented procedures should indicate who does what, where and when, and if possible why and how.  They should be based on what really is happening and not on what is desirable.  If the process requires something different from what is happening it should be worked out through a change and the personnel involved in the process be part of this change, therefore providing ownership of the process to those that have to execute it.

2.1.2  Quality Manual (Clause 4.2.2)

The first step in establishing a quality system is for company management to issue a Quality Policy Statement that defines the quality goals and commits all the resources necessary to ensure that the product or service conforms to  specific customer requirements. This will be discussed in paragraph 2.2.3.  It is important to post this Statement throughout the various departments to signal the managers’ commitment to ensuring that the policy is understood, implemented and maintained at all levels.

The second step is to develop a written Quality Manual. The Quality Manual is a controlled document that should describe what is being done and the procedures used to make sure that things are done properly.  The Quality Manual  is the place where you establish, document and maintain the processes that ensure that quality goals are met at all times. This Manual usually incorporates all the applicable sections of the ISO 9001:2000 Standard. The Manual  is focused on preventing quality problems rather than detecting them after they have already occurred.  If ISO certification is the company’s goal, its processes and procedures should be able to be audited by internal or external sources; they should also be clearly correlated to ISO sections.  If some of the ISO sections are deemed irrelevant for your business, they may be excluded, but the quality manual must identify the reason why each section is not relevant.  There shall be noted why the section is not relevant and also demonstrated that the sections not included have no effect on the quality of the product or service being provided.

The quality system should be explained in detail in the Quality Manual.  The main purpose of the Manual is to assure that all relevant processes are under control at all times. The most important processes and procedures to specify are those that assure consistency in the way of doing things.  The Manual is the main source of guidance and contains references to the other levels of documentation.  The Manual is the “road map” of the quality system and should reflect all the activities in the processes and their interfaces necessary to make the final products or service delivered.  The Manual documents all its reviews and should be maintained and kept current at all times.

The quality system should be described in the various sections of the Quality Manual, which contains the way the business is conducted, the processes utilized and the way they provide assurance that the organization will meet  customer requirements and will improve continuously  customer satisfaction.

The system documentation should be continuously updated to reflect current working conditions.  It should state what is actually being done, rather than what should be done. The system should be flexible and have clearly established procedures to change the processes as required in order to optimize the quality of the products.   Manufacturing or design processes could be part of secondary documentation. In that case, the Manual should indicate where they could be found.

The Quality Manual should be concise and straightforward with all processes explained in as simple terms as possible. The use and maintenance of the system is highly dependent on its simplicity.

The usual contents of the Manual are:

- Quality policy and objectives;

- Scope of the quality management system and justification for any exclusion

- Business processes or activities, such as sales, manufacturing, design, purchasing, shipping, servicing, and the like;

- Document flow and processes interactions, maintenance and information about how; to do things;

- Outline of procedures for documenting quality requirements;

- Organizational charts;

- Definition of terms that are unique for your business;

- Statements of responsibility and authority

The Manual should be a living document that is meaningful for the organization and not a showpiece to fulfil a requirement.

2.1.3 Control of Documents  (Clause 4.2.3)

The ISO standard requires a procedure to control and maintain the documentation and data related to products, services, suppliers, materials, and the like, as well as some key external documents, such as industry standards and customer drawings. This documentation shown in Figure 4.2.3-1 concerns all processes and acceptance criteria for the intermediate steps or the finished product, as well as general business processes that could affect the end service of the supplier.

To maintain the proper documentation system, the following steps may be necessary:

- Set up a document control procedure to release documents or data in use at  present.

- Compile a master list of all documents and data. The master list includes specifications for materials and processes, as well as controlled data such as stock numbers, customers list, and the like.  (Form 4.2.3-2)

- Establish a master list of all personnel that received copies and the issue date in all the organization’s facility or supplier.  This list shall be updated as needed and will be used to control all copies released. The number of copies issued should be minimized as much as possible.  When changes are made old versions of a document must be removed and replaced.

- Set up a signature authorization process by which all necessary approvals are obtained prior to the release of all new documents. The manager should always authorize and sign all changes before implementation. Data should be recorded and submitted to the authorized personnel to justify the changes proposed and predict the impact for further analysis after implementation.

- Establish a distribution system that will confirm removal of all previous issues of the document and their replacement with the new revised copy.

- Advise all personnel involved in the process about the new release.  This will  minimize errors in processing that can occur when changes in procedures take place.

- If a copy of the old document is retained, make an appropriate marking to distinguish it from the current document in use.

- Keep properly marked copies of old documentation and record all changes in the system for future legal or reference needs.

- Do not confuse controlled data with data that is the result of some activity or its measurement.  Take, for example, the output test of a solar module or solar cell. Those data, or the statistical average of test results, are fixed and are the result of a test.  In comparison, controlled data are variable such as: customer listings, supplier listings, materials specifications, industry standards etc. Controlled data should be stored separately and someone should be assigned responsibility for controlling changes and distributing them to all affected parties.

Documentation control is frequently hampered by lack of communications or the assumption of knowledge by all parties involved. The establishment of controls should not limit the free flow of information.  It is intended to provide a formal way of communicating the changes, which might not be noticed by all parties involved, if they are not properly notified.  Even in small companies where communications are relatively uncomplicated, the use of controls is important to keep records for future reference about when specific changes took place.

Documents can be maintained by a paper system or by a computerized system.  In either case provisions have to be made that the only copy accessible at the system is the latest version of the document.  In case of the computer version of the system, it is imperative that the system has a back up in case of failure and that control by password is made for the updating capability.  In this case it shall be stated that the paper versions of the documents are not “controlled” copies and therefore are not officially acceptable and only used as references.  It is the duty of the personnel to have access to the network or computer issue to verify that the approved and latest copy is being utilized.

The documentation procedure is one of the most important and necessary procedures for the ISO 9000 system.  It has been in the past the most frequent reason for noncompliance; therefore it should be handled with rigor and accuracy.  The listed requirements are:

- All documents shall be approved for accuracy and adequacy;

- All documents shall be reviewed, up dated and re-approved as necessary;

- Documents must have a revision status visibly noted;

- Only the latest revised and approved version should be available;

- The use of obsolete documents must be prevented internally and externally;

- Documents of external origin shall be controlled;

- All documents must remain legible and identifiable.

External documents such as standards, statutory regulations, codes, specifications from the customer or international agencies are examples of the external documents that should be controlled.

The proper authority for the approval sequence and final approval must be established in the documentation control procedure.

Documented

Request

Specifications

Procedures

Documentation Control

Data

Approved Document

Management

Approval Process

Manufacturing

New!!

Engineering

Operator

?

Auditor

Old

###### Figure 2.1 - EXAMPLE OF A DOCUMENTATION CONTROL CYCLE

DOCUMENT FLOW AND CONTROL FORM (Form 4.2.3-1)

The purpose of this form is to ensure that the documents and forms in use in the Quality Process System are the correct and most current forms, and that these forms have been reviewed and approved by the appropriate authorities. In addition, this form is to ensure that, in the appropriate cases, documents are modified, withdrawn, or classified properly. NOTE: All changes MUST be dated and numbered in the specifications or product documentation.

- Issuance of Documents and Data

- Document Developed in Accordance with Requirements

- Document Reviewed and Approved by Appropriate Manager

- Document Title and Designation Added to Master List of Documents

- Document Distributed to Appropriate Staff and Placed in Appropriate Files

- Modification of Documents and Data

- Document Reviewed by Appropriate Manager and Staff

- Document Changes Approved by Appropriate Manager

- New Document Developed

- Document Designation Modified (new date, new revision number, new title, etc.)

- Final Document Reviewed and Approved by Appropriate Manager

- Master Document List Updated

- Withdrawal of Documents and Data

- Document Reviewed by Appropriate Manager and Staff

- Document Changes Approved by Appropriate Manager

- New Document Developed

- Final Document Withdrawal Approved by Appropriate Manager

- Master Document List Updated (note date of withdrawal and superseding document)

- Classification of Documents and Data

- Document Reviewed by Appropriate Manager

- Document Classification Approved by Appropriate Manager

- controlled

- proprietary

- public

- other

- Master Document List Updated (note classification)

# MATERIALS CONTROL DATA (Form 4.2.3-2)

| Stock Number | Drawing or Specification Number | Revision Number and Date | Material Description | Acceptance Test Requirements | Engineering Control |
| --- | --- | --- | --- | --- | --- |

NOTES:

1.   The Purchase order must be in agreement with the latest release of the specification; otherwise it must have engineering approval. Material received must specify revision level.

2.   The acceptance requirement should be detailed in the Purchase order. A qualified vendor or a certificate of compliance could be a substitute if vendor is compliant with ISO requirements

3.   The engineering control column indicates that this is a controlled substance or product that must be evaluated by engineering staff upon arrival and prior to use or stock.

###### 2.1.4  Control of Records	(Clause 4.2.4)

All businesses keep records to some extent of their various operations.  Typically, records are kept on such things as contracts, purchase orders received, special requirements for products, deliveries, and meeting notes (Clause 7.2.2). Records are the result of some activity and as such are means of providing information to management on how the system is performing.

ISO section 4.2.4 requires the establishment of procedures to ensure the safe storage of records, their future availability, and also their ease of retrieval.

Every product should have an identifier that allows traceability to the significant parameters during manufacture and also its destination or contract.   Records shall be kept for a specified period of time to demonstrate that the product conforms with the specified contract performance, warranty periods, and final manufacturing tests conducted prior to shipment (Clause 7.5.3).

The quality control aspect of the business goes further.  It maintains manufacturing records of the quality process, including tests performed to check product operating parameters, records of the results of these tests, stock records, shipping, and the disposition of non-compliant products. These records are important in case  a calibration defect is found.

It is important to avoid maintaining records that are not significant to the quality or performance of the product.  Records can be kept in any media – computerized, written, or otherwise -- but a methodology and index should assure availability, good maintenance and prevention of loss.  Electronic media is particularly sensitive to the electronic programs and equipment needed, therefore changes or upgrades of the equipment or software must guarantee that the data is not lost or difficult to read.

Examples of the types of quality records that are usually kept are:

- Design files and calculations (Clause 7.3)

- Customer’s order or contracts and specified requirements (7.2.2)

- Meeting notes for contracts and management reviews.

- Validation of processes for production and service provision (Clause 7.5.2)

- Internal audit reports

- Non-conforming products and disposition

- Service reports, warranty complains, product returns and complaints

- Process control records. Traceability of product (7.5.3)

- Supplier evaluation or customer supplied product records

- Inspection and test records

- Final acceptance record and approval authority

- Education, training and skills certification

- Equipment maintenance and calibration records

- Delivery date records, shipping records, paperwork, and insurance, labeling and crating

- Management review

- Management representative reports

- Records of corrective and preventive actions

2.2  Management Responsibility 			   (Clause  5)

2.2.1 Management Commitment      (Clause 5.1)

In any company, large or small, quality in all its forms is attainable only with the total commitment of management.  Management must first establish quality as a fundamental principle of doing business and ensure that the entire workforce understands that quality is essential for business survival and growth. Second, management must take responsibility for mobilizing all personnel and company resources around the goal of quality in all the processes, products and services;  that is, meeting customer needs and expectations in all the areas of interface and services, including products that meet all the specifications and regulatory and statutory requirements. .   Managers should ensure that official company quality policy reflects the high priority given to quality.  But that is by no means enough.  The responsibility of management extends to making a habit of  quality commitment  by establishing goals and  infusing quality throughout the company’s processes through norms and culture.

For this to happen, management must be consistently involved and uncompromising in  commitment to quality.  Rhetoric alone is not enough.   A significant portion of managers’ time should be devoted to initiating quality-related actions, identifying quality systems requirements, delegating authority and providing resources in support of quality systems, and constantly reviewing performance and monitoring customer satisfaction.   In this way, managers demonstrate the importance of quality and instill a positive attitude towards quality throughout the workplace. They can further improve employees’ attitude towards this crucial function by acknowledging and rewarding employees who uphold and enhance the manufacturing of products of highest quality.

The end result of management commitment to quality is a company aligned around a consistently effective quality management system.  In other words, the end result is a company that is well positioned to be competitive and profitable.  The standard asks for a continued effort in internal communication within  the organization with the aim of establishing a quality policy, quality goals, and then following up with reviews of performance to assure that there are enough resources to meet the established goals.

This effort is sub-divided in several sections that can build the framework required to streamline and focus the management responsibility in meeting the objectives of the standard.  They are summarized as:

- Customer Focus

- Quality Policy

- Quality Objectives

- Quality Planning

- Responsibility, Authority and Communication

- Management Representative

- Internal Communication

- Management Review

2.2.2 Customer Focus  (Clause 5.2)

This clause centers on the previously discussed quality concept:  Good quality means customer satisfaction.  In order to achieve this, management must set-up a customer- related process which starts by  understanding  customer needs and  expectations through the contract or purchase orders.  These needs and expectations have to be communicated to all the organization. The resources required and product qualifications or service needs have to be evaluated, prior to accepting the order or contract.

The final steps are the evaluation of customer satisfaction after the order is completed.  Feedback and management evaluation of the quality system performance shall be performed at this time.  Corrective actions shall be taken to promote improved performance whenever necessary.  Customer satisfaction is sometimes difficult to measure; therefore frequent questionnaires and review of customer complaints are very useful tools to assert this information.  This process was outlined with the Graph in Figure 1.4 in Chapter 1.

The products of the organization have to reflect the needs of the customers. These are usually determined by means of industry reports or marketing surveys and analysis.

2.2.3 Quality Policy  (Clause 5.3)

Creating a quality policy statement or document is a basic and very important step for the organization.  The quality policy shall indicate what basic commitments are pursued and specific objectives for all the efforts in the delivery of services and products.  The new ISO standard also requires  a management commitment to pursue quality improvement goals.

As the basic quality objective is always customer satisfaction, the senior management of the organization shall list all the key parameters required to meet customer needs, and establish a plan to fulfill them.  It is important that the quality policy address the appropriate issues related to the organization’s business; and that the  policy and goals are realistic and can be carried out.  Management should provide the required resources and training, and establish measurable goals.  Each distinct objectives to be pursued should be monitored by the top management and periodically reviewed to assert progress and adjust the goals as appropriate to promote  continuous improvement.

These goals should be set in conjunction with all personnel to enlist the buy-in of all employees and managers.  Goals should be achievable within a set period of time.  They should be communicated and discussed internally to indicate a firm commitment of the senior managers to the policy.  The Quality Policy is not a static statement.  It is a strategic document that requires constant review, validation and enhancement as the goals are being met.

The Quality Policy  indicates the vision of the organization and its goals in conducting business.  The document or statement must contain measurable indicators.  All employees should  know its contents and understand  their role is this pursuit.

STATEMENT OF COMMITMENT TO QUALITY

The company, including its owners, managers, and staff, believe that quality is the primary means by which the company will succeed, and so have agreed to make it a critical priority. To this end, the company expects quality within the organization and from those organizations and individuals with which it does business.

Through meeting quality goals, the company expects to provide its customers with products that are of excellent quality, reliable, and cost-effective.  These products will be delivered on time, and will be backed by a level of service that is second to none in the industry.

__________________________________

Managing DirectorDate

###### Form 4.5.3-1 Example of Statement of Commitment to Quality

2.2.4 Planning  (Clause 5.4)

2.2.4.1 Quality Objectives  (Clause 5.4.1)

The quality policy identifies the purpose of the organization.  In order to fulfill this purpose, the senior management of the organization must develop a strategic plan and identify associated goals needed to achieve and improve its performance.  For example in a manufacturing company, reduction of scrap, reduced cycle time, delivery on schedule etc. are goals that enhance the business performance and the customer satisfaction.  They are measurable and could be analyzed for trends and consistency of the performance.

Achievement of the goals or objectives of the organization can be hindered by noncompliance issues or products that limit the efficiency and the effectiveness of the delivery of services and/or products.  These issues are usually identified through the management review process and through the customer satisfaction surveys.  In order to improve (and sometimes to maintain) the quality level it is necessary to establish goals and objectives for the specific functions or processes that are identified as the source of nonconformance. These objectives must be measurable and set up for a specific period of time.  After that period of time, progress is evaluated by a management review and a new set of goals set-up.  It is important to recognize that quality improvements come in small steps, which steadily advance towards the top of  performance.

Frequently the relevant processes are broken down into specific key objectives that are then improved to enhance the performance of the quality system.  It is important to identify the resources and the skills of the operators required in order to avoid pitfalls during the implementation process of these changes.  Management must monitor changes in processes very closely, to ensure the integrity of the quality management system.

2.2.4.2  Quality Planning  (Clause 5.4.2)

The supplier is responsible for ensuring that the products or services to be rendered are consistent with the specific customer requirements and statutory and regulatory requirements at all times.  Therefore, on projects involving either standard or non-standard products there should be a plan that assures the final product meets all those requirements as shown in Form 5.4.2-1. A sample quality plan is included in Appendix 2 as a typical document for a manufacturing company.

In addition, a strategic quality plan should be in place for the continuous evaluation of what is needed to maintain and to improve the quality of products or services. The plan should include attention to both short-term and long-term quality goals.  Short-term goals typically:

- Ensure compatibility with present processes, equipment, procedures, inspections, tests etc. on new orders or projects for special products

- Detail the action plan requirements for delivery. (Frequently several departments are necessary in the design, manufacture and implementation of special products. An action plan is needed to assert the proper documentation and test requirements.)

- Verify project milestones during schedule

- Clarify the needs and expectations of the customer and the acceptance criteria (including those requirements that may be subjective)

- Evaluate how to meet all the statutory and regulatory requirements

- Document and maintain quality records and problem resolution procedures

Long-term goals typically:

- Identify responsibility and authority for the implementation of any process improvement plans

- Identify the need to acquire controls, processes or new equipment, including tools and fixtures needed to achieve the desired quality or performance

- Identify the capability of state-of-the-art equipment and technology and develop the skills required and training or technology development required prior to accepting orders that demand such skills

- Update test equipment, inspection methods and techniques, instrumentation and quality checks, as required

- Identify operator training requirements through employee involvement and participation in the planning phase

- Identify the cost and verify the product performance at various stages of the project

In order to achieve these goals, the plan must address:

- Senior executive leadership

The management of the company must lead the effort to plan ahead to meet the company’s short-term and long-term requirements.  Significant investment may be called for new equipment, and financial analysis may be needed to justify certain courses of action.  It is important to utilize existing resources fully before considering  requests for  new ones. The goal of a good plan is not to add resources immediately but to use all existing resources more effectively.

- Customer satisfaction, requirements or complaints

The driving force in all decisions should be the customers and their needs and requirements. Customers might want improvements in quality, reductions in cost, improvement in performance or reliability, or changes in shipping methods and delivery time.  Frequent surveys are one means of staying close to the customer and are an important source of input to the strategic plan.

QUALITY PLANNING CHECKLIST  (Form 5.4.2-1)

The Management Representative and the Quality Team* should engage in quality planning that considers the best way to:

Prepare quality plans

Identify and acquire any controls, processes, equipment (including inspection and test equipment), fixtures, resources, and skills that may be needed to achieve the required quality

Ensure the compatibility of the design process, production process, installation, servicing, inspection, and test procedures and the applicable documentation

Update, as necessary, quality control, inspection, and testing techniques, including the development of new instrumentation

Identify any measurement requirement involving a capability that exceeds the known state of the art, in sufficient time for the needed capability to be developed

Identify suitable verification at appropriate stages in the manufacturing process

Clarify standards of acceptability for all features and requirements, including those that contain a subjective element

Prepare quality records

*Note:  A “quality team” is an organization’s team of managers and staff responsible for developing and reviewing the organization’s quality systems.  It is a good way for the management representative to involve other employees in the planning process.

2.2.5 Responsibility, Authority and Communication                (Clause 5.5)

2.2.5.1 Responsibility and Authority (Clause 5.5.1)

This clause requires the identification of the responsibility of every employee and the level of authority.  This is particularly important when analyzing non-conformance issues and implementing corrective or preventive actions.  The operator and the manager or the process owner has the duty to seek signatures of the authorized personnel to implement the changes required.  The documentation shall identify such responsibility to maintain an orderly system of operation.  This is applicable to a service or a manufacturing organization as well.

In order to identify the authority, the organization usually issues an organizational chart delineating the reporting authority throughout the organization.  This chart is not mandatory for the ISO9001: 2000, but it is the most frequently used vehicle to achieve the goal.  In very small organizations it may not be necessary, but in intermediate and larger organizations it is indispensable.

The following chart shown in Form 5.5.1-1 is an example of how an organization chart for a small manufacturer might look. A company using this Manual should develop its own organizational chart, or use its existing organizational chart to identify the personnel, their positions, and the reporting structure.

In order to identify the responsibility within the quality system, the most frequently utilized vehicle is a job description form.  This form is sometimes based on some specific operational requirements. However, it should also incorporate the quality implications of the specific function and the expectations for such function as shown in form 5.5.1-2.  For example a salesperson will be responsible for accurate delivery schedule, accurate description of the customer requirements and communication to the manufacturing or service department of any special need that the customer has requested.   The salesperson should be responsible for following up on the results after completion of the work and ascertaining the level of customer satisfaction.

These quality functions and feedback are elements that should appear in the salesperson’s job description along with clear indication of responsibility for achieving them.

Managing Director

Quality Manager

Customer

Service

Sales and

# Marketing

## Operations

## Operator 1

Drafting & Design

Order Entry

## Maintenance

## Manufacturing

Delivery and Warehouse

Design &

Development

Management

Representative

Installation

& Service

Audit &

Quality Teams

Process Development

## Product Development

## Quality Assurance

Quality Control

Incoming Inspection

Shipping Inspection

## Operator 2

Documentation Control

Process Control

Engineering & Design

### Purchasing

SAMPLE ORGANIZATION CHART

FORM 5.5.1-1

JOB DESCRIPTION 
(Form 5.5.1-2)

Job Title:

Employee Name:

Date Started on Job:

Description of Work:		Module Manufacturing Engineer

Description of Responsibilities:

Review and issue process specifications. Evaluate manufacturing needs to introduce new products. Issue & review materials specifications. Review all non-conforming products or materials. Request capital equipment required for manufacturing.

Description of Reporting Authority:

Reports to manufacturing manager and line manager. Functional support for  engineering manager and process development.

List of Required Qualifications:

Bachelor degree in Engineering. Mechanical know-how. Ability to program electronic controllers or computerized systems. Materials science background and experience.

List of Equipment Responsibilities:

Controllers, computers, laminators, automation units. Calibration of sun simulator. Capable of designing jigs and fixtures.

Training Completed: (note: only personnel with appropriate training may perform some tasks)

Direct training in trouble shooting electronic controllers and automation equipment

How does your job function affect the quality of the products/services of the company?

My job maintains quality of the assembly and calibration of the module line. I am responsible to maintain yields equal or better than goals for the year. Training of new employees is very important.  Evaluation of non-conforming products and corrective actions assures the quality of the company’s products.

- If additional space is required, attach additional sheets.

## 2.2.5.2 The Management Representative (Clause 5.5.2)

Quality should be the concern of everyone in the company.   At the same time, however, the design and day-to-day coordination of the quality system should be the responsibility of one clearly authorized management representative. In small organizations, the general manager or even the person with highest authority may be the Management Representative.  It is essential that the manager be able to carry out the responsibilities associated with the job.  The greater the authority of the Management Representative is, the greater the commitment to quality will be throughout the company.

The company’s quality goals should be posted, and the managing director should commit in writing to the goals of the program.  Form 4.5.3-1 is an example of a Statement of Commitment. The managing director may be the Management Representative, but in case this is not possible he or she should name one. The Management Representative not only sets up and oversees the quality system within the company, but also is the company’s representative to any auditors or standard bodies, such as the ISO or the IEC, under which the quality system will be certified.

The Management Representative is responsible for establishing quality goals. Quality goals should be realistic and achievable.  Milestones should be set to measure and monitor progress; they are the basis for reporting how the program is helping to improve quality or eliminate complaints from internal or external customers. The Management Representative represents the involvement and commitment of the top management in the quality system of the organization. As original goals are achieved and confidence is developed, new and more challenging goals can be set.  In this way, there can be continuous quality improvement.

It is important to recognize that small steps can improve quality  and steadily advance the company to the top of its performance.  All goals should be measurable.  Employees who contribute to their enhanced quality may be rewarded for their efforts and their commitment.  The representative is responsible for frequent rewards and recognition of efforts, so that other employees understand that quality is a top concern of everyone in the business.

The Management Representative ensures that the responsibilities for carrying out the tasks related to accomplishing quality objectives are appropriately distributed throughout the company and that specific quality control tasks are carried out by the persons whose jobs are most directly involved with the corresponding part of the production process.

Once the individual quality job descriptions are completed, the Management Representative may find it useful to develop and post a master organizational chart which shows employees how their tasks fit into the larger quality system and helps them better understand how their co-workers contribute to product quality.  (Form 5.5.1-1)

Part of the Management Representative’s role is to make sure that each employee sees clearly how his or her work affects the quality of the product or service that the company provides and that he or she understands and commits to specific quality control actions.  One of the tasks of the Management Representative or other designee is to document every employee’s role and responsibilities in a quality job description. (Form 5.5.1-2)

The Management Representative should organize a quality team (or various teams from various departments or functions) that will assert that the quality goals are understood and maintained as a main objective of the company.

These teams should have members from all levels of the company, should be cross- functional, and should be responsible for bringing the quality objectives to the operator level through the use of participative dialogue and feedback.

The establishment of system procedures may entail additional work for some individuals or may call for new skills for which training will be required.   Management must identify training needs and allocate the resources needed to address them, such as equipment, added labor, jigs, measuring equipment, etc.

Form 5.5.2-1 outlines the requirements and documentation that assure the integrity of the quality system and that need management implementation and outlines the authority and responsibility of the Management Representative.

The Management Representative is responsible for the operation and performance of the quality system, including the performance of sub-contractors, documentation and record-keeping of all quality processes, the verification and validation of quality, the quality audit process, the documentation of results, and the use of statistical analysis of data to facilitate a continuous improvement process.

In order to assure the continuous operation and success of the quality system, the Management Representative must conduct regular management reviews. (A quorum should be established for such reviews. Their frequency is not fixed; it depends on the company and could be weekly, by-weekly, monthly, quarterly, etc.)   A standardized procedure and agenda should be used to study and evaluate the following:

- Overall system effectiveness

- The system’s conformance to the company’s stated quality policy

- Documentation and records

- Problems encountered and corrective or preventive actions taken

- Making sure that customer needs are communicated and understood

- Customer complaints

- Training needs, and equipment or maintenance problems

- Sub-contractor performance

- Resource allocation for quality related tasks

Reviews should be aimed at assessing the effectiveness of the system.   The purpose is not to present a lengthy report, but rather to summarize how the system is functioning to improve the customer satisfaction. (Form 5.6.3-1) can be used to assure that all elements are being reviewed and that reports are issued as needed.

Once the quality policy is well known and accepted throughout the company, the focus of the management review should shift to reviewing information that best illuminates the factors that contribute to non-conforming products.  This information typically includes documented failures, predominant problem areas, corrective actions taken, training needs, and equipment or maintenance problems. (Form 5.5.2-2)

The central part of any review should always be to evaluate all aspects of customer satisfaction related to product, performance, and supply, using the most relevant documentation and information.

Specifically, it is the responsibility of the Management Representative to set up a quality management system, based on this Manual and the forms it contains, which will:

- Assure that customer needs are identified and understood

- Initiate actions that prevent defective product by reviewing the process, material and quality system requirements;

- Identify and keep record of any problems found with the product, its process or components;

- Initiate and recommend solutions by using the proper channels and authority as outlined in a documentation approval process;

- Verify that corrective action is being taken;

- Control the disposition of any non-conforming products; and,

- Monitor effectiveness of the system.

##### MANAGEMENT REPRESENTATIVE (Form 5.5.2-1)

- The Management Representative has authority and responsibility for:

- ensuring that the quality system is developed, implemented, and maintained,  in accordance with the requirements set out in this document;

- making sure that all customer needs and requirements are understood and satisfied

- reporting on the performance of the quality system to management;

- creating quality teams and internal audit teams to support the quality effort at all levels of the company, and

- constant review of the system effectiveness.

- Name of Management Representative:

MANAGEMENT REPRESENTATIVE RESPONSIBILITY CHECKLIST (Form 5.5.2-2)

- Check off  the items on the following list as each is completed.

- 1) Quality System Development

- Quality Requirements of this Manual are Reviewed

- Ask Customers about their Requirements and Expectations and communicate internally

- Ask Employees for Input on Improving/Implementing Quality Processes

- Adapt Sample Manual to Company Specifics

- Adapt/Develop Appropriate Reporting/Tracking/Auditing Forms*

- Adapted Quality Manual is Reviewed and Accepted by Management

- 2) Quality system implementation

- Equipment is Checked Against the Operating Parameters Specified in Its 	Documentation

- ______  Internal Communication Methods and Channels are established

- Employees Receive Training in Job Functions

- Employees Receive Training in Quality Program

- Documentation and Audit Materials are used to Track Quality

- Management Receives Product Quality Reports and Customer satisfaction reports

- Quality Audit and Documentation are Filed and Accessible

- 3) Quality system MAINTENANCE

- Quarterly Audit of Quality Procedures

- 4) Reporting to management

- Quarterly Report to Management

- 5) Review of the quality system

- Review of the Quality System and its Impacts on the Company and the Company’s Products and Customer Relations by the Management Representative, the Quality Team (if one has been assembled), and Company Management, including

Quality problems and actions taken

Customer complaints

How the quality system is working/if objectives are being met

Quality audit reports (internal and external)

______ Corrective action reports

Areas for improvement/changes needed

Outstanding actions

Relevance of quality policy and objectives to current needs

Training needs

Vendor problems

Equipment, working environment, and maintenance problems

- *Remark

- Sample forms are included in this Manual.

2.2.5.3 Internal Communication (Clause 5.5.3)

This internal communication requirement is new in ISO9001:2000.  Although it has been always recognized as a good method to achieve results, it was not postulated as an element in the previous issues of the Standard.  It does not require a special documentation, but records and the Quality Manual should indicate how the internal communications will take place and with what frequency.

In small companies, internal communications are generally  easy because people  interface more often.  In medium and large companies interface becomes more occasional.  Therefore a plan is usually made for assuring that important reports are communicated and the results of audits or management reviews of the quality system are  followed up.

Usual meetings for departments, strategic discussions, non-conformance reviews, and the like are typical means of communicating.  In many companies, quarterly or monthly meetings are organized to review with all employees the financial, marketing and sales, and quality issues for the month.

Typically, department reviews, design reviews, engineering meetings and sales and marketing reviews are the best means to communicate the status of new projects, new products and the performance of the organization and discuss with the employees of the various departments their needs for training or equipment.	Employee reviews offer excellent opportunities for suggestions and analysis of quality problems and prevention of nonconformance.

Suggestion boxes and bulletin boards for posting news that affect the company in general can also facilitate internal communication.

A systematic information derived from various departments performance and the posting of quality results provides a continuous data flow  and is an  excellent way to analyze trends and promote employee awareness of potential problems before they cause a non-conformance.

The most important communication is usually the customer requirements and, after delivery, any  indications of customer satisfaction or complaint.  Records should be kept of this data to gauge  the level of  customer satisfaction.

2.2.6 Management Review (Clause 5.6)

2.2.6.1 General (Clause 5.6.1)

The organization’s top management  shall participate in the quality effort by a scheduled number of reviews during the year.  Even though the top management has named a Management Representative for day-to-day executive decisions, the top managers responsible for all functions must be involved in the quality system operation.

The Management Reviews are fundamentally directed at the performance of the quality management system, and to oversee if it is effective and adequate for achieving customer satisfaction and seeking new opportunities for improvement.  During the Management Reviews the Quality Policy and the Quality goals are evaluated and changed if pertinent.

In order to perform the Review the top management and the participants must rely on inputs from the various activities and then provide solutions or outputs for further action.

2.2.6.2 Review Input  ( Clause 5.6.2)

The agenda for the Management Review should start by analyzing the action items from previous Review meetings and the results obtained.  The second item would be customer feedback and its assessment, positive or negative of the quality system and its effectiveness.

The Management Review should also analyze the results of all quality audits.  Quality audits should be conducted for internal or external purposes and could encompass the review of the quality system, the process quality, or the product quality. They should not be confused with the quality surveillance and inspection process, which is part of the process control or product acceptance. Quality audits are often conducted by personnel not involved directly in the operation, together with and in cooperation with relevant personnel. The purpose of an audit is to evaluate and recommend changes that may be needed for improvement or corrective action. Form 5.5.2-2 can be used to report on the most critical areas of activity typically covered by a quality audit.

The status of preventive and corrective actions must be evaluated for implementation of changes that could affect quality, with goal of seeking improvements in the system.

In a manufacturing environment, the process performance and  product conformity are key to achieving superior customer satisfaction.

2.2.6.3 Review Output (Clause 5.6.3)

The findings of the Management Review  should be recorded and reports  filed  of  management involvement and recommendations.  Follow-up action items for corrective actions or preventive actions should  include measurable goals for further review at the next meeting.

The type of output desired from the Management Review is  identification of  negative performance areas and  needed improvements for the system or processes.  Identification of human resource needs or equipment requirements is the direct output of these reviews.

Changes to update the quality goals and/or the Quality Policy Statement are also examples of the Management Review output.  A typical Form for reporting is shown in Form 5.6.3-1.  As usual, fulfillment of the customer needs and requirements shall be the main goal from this Review.  Planning the resources required to achieve this, is an effective outcome from the Review.

- Annual Review of the Quality System and its Impacts on the Company and the Company’s Products and Customer Relations (Form 5.6.3-1)

- Review performance of the company against the quality systems in this Manual

- List Products and Their Quality Measures (percent passing inspection, returns, etc.)

- Product Name						Measure

- Last Year		Current Year		Best

- Process Review		Problems detected	Corrective Action	Preventive

- Customer Feedback Comments by Product

- Product Name						Comments

2.3 Resource Management (Clause 6)

2.3.1 Provision of Resources (Clause 6.1)

This clause has been expanded in the present ISO 9001:2000 revision to include all types of resources needed to implement and improve the quality management system.

The previous issue took for granted that facilities to meet the product existed and dealt with maintenance and training of the personnel at all levels.  The requirements under this clause of ISO 9001:2000 cover all the human resources required by the company to fulfill and assure that the products delivered meet all customer requirements and also enhance customer satisfaction by  improving the quality system.  This applies to the total resources of the company, human resources, facilities, materials, supplies, instruments, software, transportation equipment, environmental issues and most important the time required to provide the services and improve the system.

This clause refers not only to the personnel involved in the provision of the service or manufacturing of the product, but also to the management personnel involved in the sales, finance departments and the general management functions.

Resource availability  should be evaluated at periodic intervals, for example during Management Reviews, or prior to accepting special contracts or new projects that increase the demands for time and skills.  This process should be audited and documented to demonstrate that the company satisfies this requirement.

2.3.2 Human Resources (Clause 6.2)

2.3.2.1 General (Clause 6.2.1)

All employees can affect the quality of service or the product that a large or small company  provides its customer.  Customer satisfaction can be impaired if sales personnel do not identify properly the customer requirements and also if the product does not comply with all its specifications.  The purpose of this clause is to create a mechanism that the company can use to enhance the performance of its personnel.

The company should analyze the four competency aspects critical to the achievement of  quality goals.  They are the skills, experience, education and training of all personnel in all levels of the organization, including the management personnel.  This effort is partly satisfied using the job descriptions developed in a previous chapter.

2.3.2.2 Competence, Awareness and Training (Clause 6.2.2)

The most valuable asset your company has is the knowledge and competence of its employees.

The competence of all personnel is fundamental to the proper performance of all functions.  This includes training for people at all levels of the organization, including  management. The supplier should identify by internal audits or resource review the educational level, the skills requirements and the training needs associated with each function or process. These needs should be documented and can change or become more demanding as  tasks change. A procedure shall be established to provide training to all personnel in positions that affect quality as shown in Form 6.2.2-1.

This can be achieved by creating the job descriptions as detailed earlier in the Manual and  analyzing the gap between job requirements and the actual skills and abilities available in the company.  Adjustments can be made through re-assignments of functions or by training.  Training requires that all personnel be made aware of the importance of their function in the achievement of customer satisfaction and the improvement of the quality system.

The organization should keep records of all personnel trained; these records should be continuously audited and updated. Continuously enhancing the training should be a high priority because it will result in better performance and enhance the abilities of all personnel.

The training of new operators in safety, health and quality policies is recommended, even for small businesses. Employees should be trained in several operations, and records of each employee’s certifications should be kept. Only certified operators should perform critical quality functions. The training records do not have to be very elaborate, but they should be designed to help demonstrate during an audit that only properly qualified personnel are performing the critical quality operations.  A flow chart is shown in Figure 1.8

All employees should be required to undergo training in safety and health.  They need to be aware of potential accidents that could take place in many areas. Training in chemical handling, fire drill procedures, and emergency management are essential to protect the welfare of employees at all times. Management should make clear that health and safety is not only in the employee’s own best interest but also that of the company.

Training in quality and workmanship is particularly important. All employees must know what to do and how to do it as part of their training and certification for all operations.  Employees must know at all times what is expected of them, how to operate  equipment, how to follow set-up parameters, how to conduct routine maintenance, and when to shut down an operation (for example if the unit is out of calibration or the product is not within specifications).

It is important that the organization either contract for external training (which is sometimes more cost-effective) or develop a group of in-house trainers composed of personnel with the appropriate skills, knowledge, qualification and experience needed to design and conduct training that meets the required needs.  Whichever method is used, evaluation of training effectiveness  is a necessary follow-up step so as to verify that the training has been understood, learned and properly applied.

Based on the skill requirements of specific job categories, training and review shall be carried out on a regular basis to ensure familiarity with evolving procedures. Training will be relevant to job function, as well as general company orientation, safety, and quality training. Company orientation and on-the-job safety training are required of all new employees.  Safety training should be repeated periodically.  While safety training is not an ISO requirement, it is in every company’s interest and is highly recommended.

Employees shall receive quality training within six (6) months of starting with the company, or within six (6) months after the quality training is instituted. Skills training, where applicable, shall take place prior to allowing an employee to begin work on an assigned job. Additional training may be required when indicated in audits of job performance, product quality, service quality, corrective action reviews and preventive action reviews or other appropriate indicators. Employees shall also receive training on any new equipment prior to beginning work with that new equipment.

TRAINING

Figure 2.3

Education Skills Training

Operator 1

A

Safety

Quality

statistics

Process

Equipment  Operation

Operator Certification

Operation A

Operator Certification

Operation B

Operator Certification

Operation C

Operator 2

A + B

Operator 3

A + B + C

Safety

Quality

Process

& Statistics

Equipment Operation

Managers

Quality Management

TRAINING DOCUMENTATION FORM

- Employee Name

- Employee ID/Designation

- Employee Start Date

- Employee Job Function

- Employee Training Record

Training Module			Date

New Employee Orientation

Employee Safety Training

Renewal of Safety Training								Quality Training									Renewal of Quality Training								Skills/Competency Training

Job Function										Job Function

Job Function									Additional Training

Training Module										raining Module										Training Module										Training Module

- Trainer/Training Organization: Name:

Evaluation of effectiveness of training ____________________________________

__________________________________________________________________

__________________________________________________________________

Comments and corrective and improvement actions  ________________________

__________________________________________________________________

__________________________________________________________________

Employee Feedback: (attach separate document)

TRAINING PLANS AND DOCUMENTATION (Form 6.2.2-1)

2.3.3 Infrastructure (Clause 6.3)

This clause is also new for the ISO 9001:2000.  Although the concept was implicit in the previous issue of the Standard, this requirement has been highlighted due to its great importance in obtaining the desired results.  Infrastructure refers to  physical facilities such as buildings, offices, warehouse, equipment, instruments and the like, which are essential to provide the service or product to the customer.  This also includes office space, telephones and support equipment. Without the required infrastructure, product quality will suffer and work will not be performed as anticipated.

Infrastructure requirements are evaluated in function of the availability of necessary resources that are required for the fulfillment of the customer needs.

Some examples of infrastructure needs are  an air-conditioned room to store materials, or a clean room for the assembly of a complex and sensitive product.  Other infrastructure needs can be computer systems to maintain records or to control purchase orders and the statistical control of various parameters.

Management usually evaluates these needs during the Management Review or during quality evaluation of the system.  They are normally subject to economic analysis and may require the renting or leasing of equipment or acquisition of new facilities or equipment.

2.3.4 Work Environment (Clause 6.4)

The work environment can be a critical factor in achieving adequate quality results.  Several jobs  require special protective equipment; others  require ample cooling, due to the heat generated by a furnace or a special high temperature device. These problems may affect the quality of the product, or they can affect the performance of the operator of the equipment and sometimes the equipment calibration.

This new ISO 9001:2000 clause  could be erroneously viewed as solely focused on employee Health and Safety.  While employee health and safety  are major concerns  to be considered in any company; the intent of  clause 6.4 is to prevent any environmental conditions that could curtail or diminish the capability of  personnel or equipment to perform the  function at the desired quality level.

The most important work environment considerations are usually:

- Heat

- Noise

- Light

- Hygiene

- Humidity

- Vibration

- Pollution

- Airflow

- Cleanliness

- Ergonomic considerations

The facility requirements should be identified and personnel trained to work in those areas and monitor requirements and record compliance.  Work environment compliance should be a subject for the Management Review in order to assess the need for improvements.  The requirements are usually evaluated through the data obtained from the corrective and preventive action reviews.

Homework Assignment – Day Two

YOUR NAME: ________________________________

YOUR COMPANY:  ___________________________

Early on in the process of designing for the first time or setting out to improve your company’s quality system, it will be important to engage people throughout the company in taking stock of what policies, procedures and attitudes towards quality currently exist.  Today’s homework assignment gets you started with this assessment process.

What quality goals and policies are currently in place in your company?  How were they set and by whom?  How often are they reviewed?  How are they communicated throughout the company?

On the back of this page or on a separate piece of paper, draw an organizational chart for your company.  Include all its divisions, managers, and employees and make it clear who reports to whom.

Does your company currently have a Management Representative responsible for the achievement of quality goals?

___  Yes

___  No

(If yes) What specific responsibilities and authority does this person have?  Based on what you’ve learned at this training, do you think this role is adequately defined and implemented in your company or needs to be improved?  (Be specific)

(If no)  Who in the company should be assigned to carry out this function and why?  What specific steps need to be taken by what parties to formalize the appointment of someone to this position in the near future?

What specific steps need to be taken and by whom in your company to ensure that every employee is clear about his/her role and responsibility in quality control?  What kinds of resistance would you expect?  How might such resistance be effectively overcome?

Refer back to the flow chart you developed as part of your Day One Homework.  Put a checkmark beside those steps for which you know a description of the operation or work instructions already exist and are adequate, or a plus (+) for where they exists but needs improvement.  Put a question mark (?) beside those where you are uncertain, along with the name of the person(s) who would know.  Put an “x” next to those steps where you are certain no descriptions or instructions exist.

Conformity assessment is an activity concerned with determining that relevant requirements are fulfilled.  Typical examples of conformity assessment activities are sampling, testing and inspection, evaluation, verification, registration, accreditation and approval.