## Chapter Three:  ISO 9001:2000 Standard

## (Continued)

Session 3-1 Product Realization	  (Clause 7)

This clause is one of the most important in the Standard. It combines several requirements of the previous issue of the Standard.  The fundamental reason for existence of the organization is to deliver a service or product. Therefore product or service realization is the organization’s most important business.

The organization shall demonstrate that it knows what it takes to make a product and therefore should plan all the activities required to realize the process from beginning to end.  This clause dwells on the fundamental plan and execution requirements that will assist your company in assuring that your product or service conforms to all requirements, satisfies  customer needs and achieves improvements in the delivery process.

The basic concept of the ISO 9001:2000 is to establish a quality system through a process approach.  Various steps are linked one to the next until product or service delivery  is accomplished.  In this sequence, each step can be considered as performing an addition to the value of the product; also each step has an input and an output, a supplier and a customer in a different terminology. Therefore on each step we can have a measure of effectiveness and a value added measurement.  The quality of each step must be evaluated and improved to allow progress to the goal of customer satisfaction.

3.1.1 Planning of Product Realization  (Clause 7.1)

The main objective of this clause is to define what is required to make the product, how this is going to be achieved, the documented procedures and processes needed, and the resources available and required in this regard. These objectives are usually known. Your organization has to detail a plan to achieve them and document the process.  Validation and verification of the activities must be recorded and these results shall be stored.

The first task for your organization is to demonstrate that it knows what is required in the total process, from the initial customer requirement to the delivery and customer feedback.

These processes were presented in clause 4.1 in a simple flow diagram (Form 4.1-1).  Each of the activities requires a process description and an acceptance procedure.  Most organizations already have  these documents and they have been compiled in the Quality Manual.  At this time, it is important to review  whether there are any new processes and, if so, to include their documentation in the Quality Plan.  This Plan shall indicate all that must happen for the product or service to be successful in meeting all requirements.  Sample Form 7.1-1

For example:  If you are manufacturing solar cells, the silicon wafers have to be within a range in resistivity and the type of crystal should be of a specific type, monocrystalline “p” type or polycrystalline; this should be described in a purchasing document etc.  The chemicals must be of certain quality and so on.  The process shall be performed in certain equipment with defined parameters, etc.  This list is part of the Quality Plan.

The resultant product should be able to meet certain specifications, reliability, and the like, as defined by the Standards in use for that product with the additional customer requirements for the specific project.

This listing of documentation, specifications and parameters are compiled into a Quality Plan that demonstrates that the product is made in such a way as to meet all needs.   A typical example of a Quality Plan has been included in Appendix 2.

The Plan should also include which records are needed and shall be saved to demonstrate conformance, if requested by a customer or an inspector.

The ISO has issued Document ISO 10005, which has valuable guidelines for the creation of Quality Plans.

The Quality Plan must further define the Quality Goals and the objectives and requirements to be met by the product.  Usually the organization has all the resources at hand to meet its Quality Plan.  If there is a gap in either the equipment or the expertise of the human resources, the Quality Plan must contain a plan of action to fill this gap.  The management of the organization must be familiar with the contents of the Quality Plan, its needs and procedures in order to create the goals and objectives of the product realization process.

It is important that documentation is always kept up to date.  All operators need to know  the location of documentation for each process and the type of records that they should maintain.  Each employee could sign a format as attached in Form 7.1-2 to demonstrate his or her awareness of the requirements for the job.

WORK FLOW DIAGRAM

Form 4.1-1 (repeated)

Customer Order

Product Design Review ——>If Design required——>Design Process Steps

If No Product Design, then Manufacturing review

Purchase Components ——>Yes——>Outside Vendor/Product Assessment

If Company does not Purchase Finished Components, then

Collect Components From Inventory/Storage

Product Assembly		    Yes		Assembly Process Steps

Product Testing         Yes        Equipment Calibrated         No        Calibration Process

Yes		Test Products		Yes

Product Testing Not Carried Out

Product Packaging (Include Manual/Product Specifications)

Product Shipment/Delivery

Customer Receipt/Approval

Customer Education and Feedback

Service Agreement

Documentation of Product Quality and Management Review

Sample Documentation of the Steps in 
the Work Flow Process

Form 7.1-1

Customer Order

Product Design

Design Process Steps

Final Design Quality Assurance Approval

Purchase Components

Outside Vendor/Product Assessment

Collect Components from Inventory/Storage

Manufacture/Assembly Process Steps

Product Testing

Equipment Calibrated

Calibration Process

Test Products

Product Packaging

Product Shipment/Delivery

Customer Receipt/Approval

Customer Education

Service Agreement

Documentation of Product Quality

Marketing & Sales		Manufacturing Supervisor		Customer

Date				Date					Date

- INSTRUCTIONS FOR THE USE OF THE QUALITY

- SYSTEM PROCEDURES (FORM 7.1-2)

- The use of Quality System Procedures involves an understanding of the specific quality standards, through training and familiarizing employees with the concepts of quality systems and the specific procedures and documentation requirements of each step in the process for which an employee is responsible.

- Instructions on Where to Find Information on the 
Quality Procedures

- Information on where to find information on the Quality Procedures is available in (location) of this document.  In addition, information is available from (name of the Management Representative), and additional documents are on file in the office of the (name of office). Finally, information on specific quality procedures is available at the applicable workstations.

- _____________________________________                   __________

- Operator							Date

3.2  Customer- Related  Processes	(Clause 7.2)

The organization shall have a procedure for contract review and documentation, in order to assure that all commitments are adequately defined in writing and that the organization has the capability to deliver and meet all the contractual requirements.  Many contracts require complex systems that need new design or assembly configuration; therefore a project review, schedule and documentation are necessary to fully evaluate the contract.

The contract review should cover all aspects of the contract – the product to be delivered, performance specifications, product design (if necessary), design reviews, terms of customer acceptance, shipping schedule, method of shipment, payment schedule, installation instructions, training, and anticipated reliability, service and warranty.

3.2.1  Determination of Requirements Related to the Product   			  (Clause 7.2.1)

One of the most common problems encountered with contracts is the misunderstanding of what was ordered or misinterpretation of the customer expectations. A good communication and clarification process with the customer is essential for the good execution of the contract. A complete record of all communications is also very valuable, in case of conflict.  The person taking an order over a telephone should read back what has been written to confirm a good understanding of all the details of the order.  All contacts, discussions, memos and correspondence should be properly documented and filed. If the order is received electronically, a FAX acceptance or an electronic record of the requirement shall be sent to the customer to verify the customer need.

There are four different types of requirements that the new ISO 9001:2000 has explicitly incorporated:

- Customer specified requirements

- Necessary requirement, not specifically stated, but understood for the application

- Regulatory requirements

- Additional requirements characteristic of the organization

Any modifications or changes between the contract and the original tender documents should be clarified and recorded in writing.  All parties affected by the changes shall be notified in writing and they should approve the changes in writing, if called for in the contract.  If the contract is changed in scope or contents at any time, a written record should be made and all the supplier’s departments involved in the contract should be notified in writing to avoid confusion leading to an erroneous delivery of a product.

For example, after an order for a system is received, a layout and a drawing for the configuration are commonly submitted to the customer for approval.  These might not be acceptable and changes in configuration may be requested at that time. Since the manufacturing and ordering of components could be taking place simultaneously, prompt and accurate feedback is needed to avoid purchase of wrong components or assemblies and subsequent delay of delivery.  Communications and rapid feedback are essential for proper workflow and delivery.

3.2.2 Review of Requirements Related to the Product (Clause 7.2.2)

Product requirements have been discussed above.  It is very important that your organization review the requirements vs. the performance of its products or services.  In case of a distributor or the contract for off-the-shelf items, this is a simple review that some experienced personnel can conduct.  In case of a small company, the manager can make a quick decision.  The problem starts when the product is part of a system that has to operate in diverse conditions, sometimes  unknown. In such a case it is important to review the requirements with the engineering or development personnel to avoid mistakes.

A documented procedure is the most convenient and expeditious way to prevent these situations.  The acceptance authority should be clarified and the customer requirements meticulously analyzed.  An example of the procedure steps is noted in  Figure 3.1.

The important aspects of this clause is first to confirm the requirements (product, delivery, and installation, if needed); second to review the requirements against the product and capability of your organization and make a plan to fill the gaps if any are found, and third to provide the resources required.  If your organization does not have these resources or the product is a state of the art product, the customer and the key people in your company must know there risks associated with the order.

3.2.3 Customer Communication  (Clause 7.2.3)

In order to satisfy the customer, it is imperative to understand all the customer requests and pass them on to those in charge of delivering the product or service.  After delivery, follow-up with the customer is important to determine customer satisfaction and acceptance of the product.  This follow-up exercise applies not only to product quality, but also to delivery, invoicing, details of the instructions and the like.

A file for the contract should be established. All paper work, quality records and documentation should be filed and kept for future reference or use.  Many contracts specify a minimum period of time that the supplier must keep records proving that the quality process was fulfilled during the execution of the contract.  All contract documentation contract or orders are quality records and should be handled according to the ISO requirements in Clause 4.2.4.  The retention of records is very important for future reviews and verification of compliance. An established record keeping cycle is recommended to avoid storage of records forever.  The storage time should be pre-established as part of your Quality Manual.  If your customer or contract requires longer storage, this should be specified when the contract or order is received and approved.

Figure 3.1  NON STANDARD PRODUCT-

PURCHASE ORDER ENTRY PROCEDURE

CONTRACT ORDER, REVIEW, VERIFICATION, AND VALIDATION (Form 7.2.1-1)

Customer:

Customer Contact Information (phone, etc.) 					______

Product Name:

Unique Product Identification:							______

Order Date:

Due/Delivery Date:

Delivery Requirements/Pick-Up Requirements:

Special Requests:

Quoted Cost:

Quoted Constraints:

Contract Review:

Employee/Group Assigned Work:

Manager Responsible:

Initial Product Specification Review

Requirements

Estimated Cost

Delivery Schedule

Special Requirements

Contact Person(s)

Design/Engineering Meeting (if appropriate)

Initial Product Design/Progress Review (if appropriate)

Interim Product Design/Progress Review (if appropriate)

Prototype Review (if appropriate)

Final Design Review

Order/Contract Modifications:

3.3 Design and Development    (Clause 7.3)

The design and development process applies only to companies that provide design and development services or that design new products.  The intent of the process is to provide a documented procedure that will explain the organization of the design effort.  It should not be very elaborate or limit or restrict the creativity and ingenuity of the designer.

3.3.1 Design and Development Planning (Clause 7.3.1)

The process starts by defining the intent of the design; this includes identifying any specifications for performance, appearance, and reliability, along with any changes or new developments in shape or form. Once the intent is defined, the company shall identify the resources needed to achieve the design goal, assign qualified personnel, and commit resources. This will generate a Design Plan and schedule that should be reviewed periodically to monitor progress, address difficulties, or add new requirements, as needed. The Plan should outline the interfaces between the design and manufacturing functions and develop a phased-in production process, so that there is a smooth transition between the functions.

A well-defined goal for performance and reliability must be set-up at the beginning of the design effort.  Regular progress reviews help the team involved in the design focus on problem areas, seek support from other areas of the company and keep the customer aware of any difficulties or delays in the completion of the design.  Effective communications are indispensable for an effective Design Plan. The documentation of all calculations or any appropriate data is important for the contracted design, as well as for input to any future design effort.  Flow charts, Gantt or Pert charts are effective visual aids and graphical tools that show the status and progress of the design effort and the indicators of proper or defective planning.

The elements of the suggested process for establishing the desired control are given in Forms 7.3.1-1 and 7.3.1-2:

- The aim of the design is to:

- Define clearly the desired outcome

- Establish a simple flowchart of how to get there

- Assign responsibility for various steps

- Establish design reviews, verification and validation activities

Ideally, it will be possible to compare the new design to previous designs as the first step in identifying the changes required. In this case, the calculation of the new design can be derived from a basic known product and its performance, and can simply be expanded or changed to achieve the new design.  The creation of a schedule and a Gantt or milestone chart is very helpful for planning and resource issues.

- Procedures and responsibilities:

- Assign technical support requirements

- Determine other related efforts and support

- Specify needed documentation and purchasing support

- Define communication with and review by all related parties

The responsibility for carrying out the design effort must be made clear. All resource needs should be anticipated and addressed, in order to allow for a smooth project schedule. It is likely that other departments, such as manufacturing, purchasing, sales and/or marketing, will be involved in the outcome of the design.  Subcontractors and the customer may also be involved.   All concerned parties should participate as appropriate throughout the process. Their support at critical times may be indispensable to timely and satisfactory completion of the design process.  Regardless of the nature and extent of their participation, however, each party should always be kept aware of the status of the design.

    - Design and Development Inputs (Clause 7.3.2)

Design requirements are sometimes confusing or not well defined.  Communications are of prime importance to avoid an effort that could lead to an unwanted product. A clear outline of needs calls for links among sales, the customer and the designer. There should be no conflicting goals. Common examples of non-stated requirements are high voltage performance, operating environment, corrosion resistance, and reliability test conditions.

Information requirements:

- Review and clarify all input requirements with customers and marketing

- Review all statutory and regulatory requirements

- Take into consideration all contract reviews and needs

- Clarify conflicting requirements with customer or marketing needs

- Review lists of “non-stated requirements,” such as: legal requirements, patents, safety, industry practice and standards, etc.

- Use all previous design and development data and pertinent requirements of existing products

3.3.3  Design and Development Outputs  (Clause 7.3.3)

Once the design is completed, a performance test is calculated, based on the parameters that have been changed or a prototype is commonly made. It is important to evaluate whether the new design introduces any negative environmental effects that would violate regulatory norms.  For example, the new design may require use of a different chemical that has special disposal requirements, a new metal that emits gases as it is processed, etc.  Also, problems can be associated with handling or storage, as in the case of batteries that may be in storage for a long time and require recharging or specific storage conditions.  Some materials for assembly have limited shelf life and can only be stored for a limited amount of time under certain conditions of humidity and temperature.  All these details must be part of the design effort, including a plan for the disposal of the product at the end of its life (e.g. batteries).

Design output (outcome):

- Review the desired outcome of the design effort

- Verify that the design meets all the intended requirements of the input

- Consider how the design is going to be used and any pertinent regulations

- Document all aspects of the design and its components or the raw materials needed to provide the service or produce the product

- Review the acceptance criteria of the finished product and also of the materials and subassemblies.

- Identify factors that could be critical to customer satisfaction during operation or use (storage, handling, maintenance, disposal, etc.)

- Demonstrate that the product will not perform unwanted functions or have unwanted failures

3.3.4  Design and Development Review  (Clause 7.3.4)

The design review is a vital instrument for the proper outcome of the design. Sometimes, if the design is very small, initial and final reviews are all that is required.  However, for a new design effort or for an elaborate product or service, multiple reviews involving all the departments such as sales, marketing, manufacturing, in addition to engineering and service are generally involved and can provide a great benefit.  These general reviews are highly recommended. These reviews could include the customer and some outside contractors as required. Documentation of any changes is a critical item to be controlled.

- Design reviews details

- Establishes intermediate reviews to measure progress

- Allows a review of the plan and updating the schedule and use of additional resources if needed.  New people can be assigned to the project

- Reviews all needs, internal and external to assure success

- Seeks all involved departments’ inputs into the discussion, i.e. marketing, manufacturing, management, engineering, maintenance, service, installers, etc.

- Checks scheduling of purchase of components or subassemblies and setup of inventory requirements

- Enables customer or contractor participation

- Assures that the product meets all specified requirements

- Secure suppliers’ or subcontractors’ assurances that their products meet requirements

- Reduces cost of fabrication by involving proper manufacturing people

- Assures a timely release of new products

- Provides records and documentation of all steps and decisions.  Allows follow up of action items and list all the issues and their solution

- Allows greater customer satisfaction through participation on the design effort.

Design review

User

Needs

Design

Input

Design

Process

Design

Output

Product

Verification

Validation

##### Figure 3.2  DESIGN REVIEW

- 3.3.5 Design and Development Verification (Clause 7.3.5)

The verification process should be defined in the Quality Manual.  The verification of the design typically takes place after the process is defined and the design is completed or initial units are made.  It is a valuable step for producing a reliable product. Several tests are usually needed to confirm that the design meets all the desired parameters and specifications, such as the international standards or regulatory functions.

- Design verification

- Use an established verification plan to confirm that the design meets the outcome desired

- Perform alternative comparisons to existing products or calculations based on available products

- Undertake demonstrations and tests per existing standards

- Review the status of documentation and its completeness

- Involve the customer in the evaluation of results

3.3.6  Design and Development Validation (Clause 7.3.6)

The ideal reliability test is usually a life test under normal use.  In the case of photovoltaic products that have a life expectancy of 10-25 years, the use of accelerated tests have been well established and provide stress in the designs similar to those of actual life. These tests should be repeated as many times as required for practical purposes.

Design validation:

- Follows verification, after completion of a finished product

- Usually requires a functional test under normal operating (field) conditions

- Should involve customer or intended user

- Must meet the environmental and reliability goals for the product and be tested under established standard test conditions.

3.3.7  Design and Development Changes  (Clause 7.3.7)

Records and documentation of the product design function are valuable tools in the event of a redesign process.  Products are redesigned for varying reasons.  The verification and validation of the product should be made when changes are implemented to understand the effect of the changes in the design. This effort is greatly simplified if good records have been kept during the initial design process.

Future design modifications and changes

- Products usually require changes and modifications during their market life

- All changes should be recorded and documented and APPROVED by authorized Management Representative

- The quality system must have formal requirements for documentation and control of changes

- Design changes may require design review, verification and validation

- Changes may affect contracts or customers, who, therefore should be notified prior to implementation

FORM FOR QUALITY CONTROL OF DESIGN PROCEDURES 
(Form 7.3.1-1)

Description of the Company’s Design Process

Customer Order

Customer Requirements

Purchasing Specifications for Material/Component

Assess Cost and Price

Confirmation of Order, Requirements, and Price

Assignment to Appropriate Staff

Document Requirements and Schedule with Staff

Preliminary Design

Design Review

Review Design with Customer

Finalize Design

Final Design Review

Customer Review

Approve Design

3.3.8  Procedures to Control and Verify the Design of Products

For each design process step above, the parties involved (manager, designer, customer, etc.) should keep a record of all the appropriate comments and after reviewing the new changes, initial and date the documented change. All documentation for each product design should be kept together in a dedicated file. A simple form listing all the steps in Form 7.3.1-1 above can be used.  It should be modified as necessary for your company’s process, with the appropriate space for comments, as a checklist form.

Very often, a company may select a number of managers from various departments, including marketing, manufacturing, design engineering, process engineering, quality assurance and purchasing to create a Product Review Board.  This Board signs off on any new product or when changes in the design of existing products are made.  The Review Board is useful to make sure that all involved parties are aware of any changes before they are implemented.  Form (3.4-2) can be used to document the stages in the design review, verification, and validation process.

Design Review, Verification, and Validation form 
(Form 7.3.1-2)

Product Name:								________

Unique Product Identification:___	____________________________________

____________________________________________________________

Product Customer:							______________

Product Design Requested by:						________

____________________________________________________________

Date of Product Design Initial Request:						__

Date Product Design Expected:					______________

Manager Responsible for Product Design:						__

Product Design Assigned to:						______________

Date Design Assigned to Staff:							__

Initial Product Specification Review

Requirements

Estimated Cost

Delivery Schedule

Special Requirements

Contact Person(s)

Initial Product Design Meeting

Design/Engineering Meeting

Initial Product Design/Progress Review

Interim Product Design/Progress Review

Prototype Review

Final Design Review

Case Study #1

CONTRACT CONTROL

The contract review process should be performed with extensive analysis of the requirements of a contract and the customer needs prior to acceptance.

Sometimes in the process of doing business, contracts are signed either to gain a customer or to enter into a new market.   Every contract should be properly analyzed prior to acceptance with the aim of fully understanding the risks entailed and the investments required.

For example, a PV system manufacturer was determined to win a contract to build a PV-powered irrigation system in Southern Egypt.  This was a great opportunity to establish an oasis in the middle of a dessert.  A group of managers worked very hard for weeks to develop the contract proposal.  Their efforts were rewarded when they did, indeed, win the contract.

However, it wasn’t long after starting work on the project that the manufacturer discovered a major problem.  The contract called for the delivery and installation of a water tank as part of the irrigation system.  The customer anticipated getting a steel water tank sitting on a pad of concrete.  It turned out that the managers had not understood the formidable engineering challenge of installing a heavy tank in a remote region where there were no access roads.  It soon became clear that the tank would cost more than the company had estimated for the entire cost of the project.  The management failure to consult adequately with the company’s engineers as part of the process of drafting the contract had created a seemingly inescapable dilemma.

As it turned out, after several negotiations and reviews, the manufacturer was able to avoid going deeply into the red by a stroke of luck.  This was possible only because of an equally lax contract control on the customer’s part.  Although the customer had wanted a metal water tank on a concrete base, the contract had not specified what the tank needed to be made of.  This gave the supplier the opening to utilize an easy-to-transport plastic bladder tank that served the performance specifications of the irrigation system and honored the letter of the contract at a substantially reduced cost.

Although this case study had a happy ending, the outcome could easily have been disastrous.  The situation illustrates how the absence of formal contract review and procedures can easily lead to costly oversights and mistakes.   It also illustrates how detailed contract negotiation with the customer can create cost-saving opportunities, for an alert company.

3.4 Purchasing     (Clause 7.4)								Session II-5

Clause 7.4 of  ISO 9001:2000 requires a documented procedure to make sure that any products purchased from vendors, retailers or distributors conform to desired specifications and functions, especially those that affect the quality of the product.  The intent is to assure that your company gets what it needs, wants, and expects without incurring costly errors or delays.

### 3.4.1 Purchasing Process (Clause 7.4.1)

It is important that the purchase order for any product describe in detail your company’s requirements, including the type, class, grade and other pertinent information. When purchasing materials, costly errors can be avoided by specifying technical data and the materials intended use.

The vendor should be required to certify that the product meets its specific performance rating, either by identification of a catalogue number (standard product) or by providing evidence of testing prior to delivery. Your company should identify the title, description or quality system standard to be applied. If the purchased product or material is of a critical nature for the finished product, you should remember that the responsibility for the quality of the material that you purchased is yours.  You have to assure that what you receive is what you need.

All purchasing transactions or subsequent changes should be presented in writing. Even when agreement is reached over the telephone, proper documentation of the changes or agreements should be written up and kept for the records. All documents should contain all the relevant details of the items purchased, including part numbers, special requirements, or markings needed to handle the product.  Provisions should be made for settlement of any quality disputes.

A performance record should be kept on each subcontractor that documents product quality, delivery performance, and subcontractor reliability in supplying quality information.

In some cases, it is advisable for your company to visit the vendor’s production site, possibly accompanied by the customer. These visits will help ensure timely deliveries and proper quality.  Site visit requirements should be part of the purchase document and coordinated with the subcontractor personnel.

### Evaluation of suppliers

In order to assure that suppliers are acceptable, you must establish and document the answers to the following questions regarding each of your critical vendors (Form 7.4.1-1)

- How reliable is the vendor?

Many suppliers or vendors overlook tolerances or quality requirements. You must establish with the vendor the specification of the products or materials that you are purchasing and make the vendor’s managers aware of the impact on your product that a missed parameter can cause.

- Can they deliver what you want?

The vendor should have the equipment and the capability to deliver the products without significant trouble. Otherwise your manufacturing schedule can be at risk, and, by extension, your delivery to the customer. A plant inspection should be made to assure that the supplier or vendor has adequate capability.

- Is their quality acceptable? Do they have a quality system?

Ideally, the vendor will have ISO 9000 certification for its manufacturing plant. This will assure you that it has the proper quality system in place and that it will work together with you to maintain the required quality level. If the vendor does not have such certification, then you must explain in painstaking detail what they must do to guarantee that the product they deliver to you is adequately tested.

- Is the quoted delivery and price adequate?

Price and delivery considerations are always important, but they should not replace quality considerations as the primary guideline for a purchase. You must stress that poor quality at a low price results in higher cost, due to the increased amount of non-conforming product.

- Does the vendor have a good reputation, and have you done business before?

Previously known suppliers are usually preferred vendors. You should establish a list of preferred vendors for each of the materials or products purchased, based on delivery experience. Timeliness and quality should be measured and recorded for future buys.

- Does the vendor have adequate equipment, resources and personnel?

Adequacy of equipment is imperative for holding the desired tolerances. For example, if you buy a frame, the subcontractor should have adequate machine tools to maintain squareness during the assembly process. Trained personnel make it easier to obtain good products. For example, in the case of installation services, a trained technician can avoid costly mistakes in the installation of the battery, regulator, and the release of the system after checkout.

- Is the product off the shelf?  Does it have a shelf life? How well does the supplier handle product?

Many products are common to several suppliers of different industries. A distributor can handle these products and they may be on their shelf for a long period of time.  This is significant when a product has a limited shelf life. For example, if your product needs a primer, be aware that most primers have a limited shelf life. If you use a battery, keep in mind that most batteries self-discharge and should be maintained or delivered dry to avoid plate corrosion. Also, many of the silver pastes used for printing cell contacts have limited life and require specific temperatures for storage.  Failure of a vendor to handle products adequately can result in future failures for your products.

Inventory Control

The purchase of materials or components for manufacturing is a very important matter, not only with regard to the quality of the materials, but also with regard to delivery schedule and the service received from vendors.

The maintenance of a basic inventory of standard materials, parts and components is advisable to be able to respond to non-scheduled demands.  Usually the level of inventory is strongly dependent on the lead times required and the turnover rates for the product.  In general an on-time delivery system from the vendors or sub-contractors is favored over maintaining costly in-house inventories of materials.

The cost of inventory is a significant investment for many industries and can be particularly high for a small manufacturing company.  Therefore, inventory should be kept to a minimum without unduly affecting the day-to-day operations.  Timely delivery fulfillment is critical. Late or irregular delivery of materials or subassemblies can prove very expensive for your company because they force you to carry a higher inventory level.

Vendor evaluation

Records of the performance of all vendors shall be maintained to develop a list of approved suppliers.  These suppliers are selected by the performance in the acceptability of their products and the timely fulfillment of the delivery as described above.  These records are part of the documentation requirements by the inspectors for the ISO evaluation of your system.

    - Purchasing Information (Clause 7.4.2)

The process of purchasing depends strongly on the importance of the materials involved. If the material is an off the shelf item and it is not used for a critical assembly or application, the description should be simple and only requires a catalogue number.  Approval procedures for purchase usually consist of a check mark or some simple note or signature.

This clause focuses instead on the purchase of critical parts.  These require a very detailed description of what is required and written information that leaves no doubt of what is needed.  The information shall list all pertinent details and clearly state how the material or product will be accepted.  This also includes the description of the acceptance process at your plant or at the vendor’s plant.  The purchase documents should be very specific as to details of inspection accuracy to satisfy your needs.

An inadequate documentation process is the most frequent reason for failure or rejection of materials or worse. If the product is not caught at the receiving platform, it will result in defective products and warranty claims in the future, thereby creating a liability.

The purchasing documentation is an important element in the evaluation of the quality process. Again the responsibility for the adequate quality of materials used is yours.

All Purchase Orders for critical parts are to be approved by various managers prior to release.  Care shall be taken that the documentation is kept current and that the parts ordered have not been changed as was discussed in the design change section of the Manual. Records of the Purchase Orders for materials are maintained and performance of the vendors analyzed frequently to promote improvement in the process.

### 3.4.3  Verification of Purchased Product  (Clause 7.4.3)

This step is very important to ensure and maintain quality control  in your company.  The type of inspection or verification depends on the nature of the material received and the record of its supplier.  The intent is to assure that all materials received are in conformity with requirements.

In the case of office supplies a very simple procedure can verify that the product is what was ordered.  In the case of manufacturing materials, it is important to follow the specific instructions of the Purchase Order and its level of inspection and verification.  The activities shall be planned in advanced and they could be of a variety of levels such as:

- Conventional sampling techniques

- One hundred per cent inspection and measurement

- Certification of material quality by the supplier

- Verification of operating quality at the suppliers facility

- Hiring an independent inspector

- A combination of all these strategies

The end result of these efforts should be objective evidence that the requirements are being met.

Products received from a vendor could require tracking after receipt, in order to verify quality, quantity or performance. These data should be part of vendor or subcontractor records and should help in future purchases of a similar product.  This is especially important for suppliers of complete systems that buy most components of the system in a completed assembly, since there are limited ways to verify actual performance until the components are integrated in the final system. A failure at this point could cause major delays or expense, resulting in the need to disassemble all the other components in the factory or at the field installation.

## Case Study # 2

# PURCHASING CONTROL OF SUB-CONTRACTOR QUALIFICATIONS

Design process controls, including substantial testing of new designs, are essential to ensuring that products will meet customer expectations for performance and reliability. However, product quality can also be greatly affected by post-design factors.  For example, lax implementation methods or sloppy workmanship can jeopardize the outcome of the assembly process. Likewise, procedures for procuring components or assemblies can have a major impact on the quality of the final product.

In fact, the ultimate success or defeat of a company’s quality efforts can hinge on the proper selection and qualification of its vendor’ quality systems. As the following example shows, the acquisition of components or, in the case of solar cells, materials for use in the manufacturing process are situations in which vendor performance can become a critical issue.

A new controller was designed for Solar Home Systems with a maximum capacity of 12 Amperes for either 12-volt or 24-volt systems. The functional parameter for this unit was its ability to control the charging capacity with a switching, shunt-type circuitry, but with low power dissipation.  As a result, it did not require a large heat sink.  This performance was the result of a variable pulse design, which lowered the voltage to a level close to the short-circuit current and therefore dissipated very low power levels.  The frequency of the pulse varied with the state of charge of the battery -- zero at a low state of charge, allowing a full charge, and faster as the battery voltage progressed towards full charge. The advantage of the system was that the battery received short pulses of charge even when the voltage was full, thereby preventing problems related to lack of charging when the system is fully charged and in a float condition.

As with all shunt systems, the performance of the unit depended on the protection afforded by the blocking diode placed between the solar and the battery. Special care was taken to select a Schottky diode characterized by low leakage and low forward voltage drop, in order to avoid losses and ensure enough voltage during system operation.

The design parameters produced by the engineering team were excellent and successfully withstood extensive environmental testing, including temperature and humidity tests typically performed on solar modules. This was necessary, since one of the design requirements called for the controller to be fitted in the module junction box.  Therefore, it would have to be able to endure the same type of environmental conditions.

Once the design was completed, several manufacturing lots were made with a successful yield and performance after burn-in of a sample taken from each lot.  A substantial order was received and, after thorough analysis of the plant assembly capacity, the decision was made to go with an outside supplier rather than to expand internal capacity.  The purchasing department

led the effort and a vendor was selected based upon its experience with this type of assembly. A set of components was delivered to assess workmanship and successful product assembly. Next, the production lot was released to the selected vendor.

Samples of the production lots were tested for performance, and the lots were accepted and shipped to customers until a field failure was noticed. Repeated environmental tests indicated a deviation from the original design.  The nonconforming product was analyzed and it was soon found that, after the high temperature test, the leakage and the voltage drop across the diode were drifting with temperatures higher than what was allowable within normal parameters.  Room temperature tests were not affected. Further investigation showed that the blocking diode was at fault. Since the diode was a very critical component, the engineering department had specified the exact type of diode to be used. Further inquiries of the vendor revealed that its purchasing department had decided to use an “equivalent” unit, without noticing that the drift and leakage of the alternate unit differed from the original design.  A vendor has no authority to change components without prior approval from the contractor. Any vendor should have the same quality system level as the contracting company, in order to assure the proper results. Also, no changes should be made or “equivalent” supplies used without testing the consequences of the change, however minor their anticipated impact on performance.

Further problems developed in a subsequent delivery due to the encapsulation process. All the terminals and connections were encapsulated to prevent corrosion of the controller due to humidity. The location of a calibration potentiometer was such that it allowed the adjustment of the set points after encapsulation. A batch of units went to stock before it was noticed that the level of encapsulation was below the specified mark. This allowed water penetration and accumulation with the resulting corrosion and failure of the terminals of the adjustment device. Although the system did not fail outright, the battery ran unprotected for several weeks before the defect was noticed.

Tighter control of the encapsulation process and inspection was necessary to avoid further recurrence of the problem. The drawings needed to be clarified because, although properly dimensioned, they did not warn the manufacturer of the danger of not meeting the proper level of encapsulation.

The performance of the unit after these initial hurdles has been excellent, with returns of less than 0.1% in the last 12 years.

We save lots of money because we use fewer specifications and less paperwork

Component

Cost Chart

5% savings

Assembly Cost

Assembly Yield

They gave us these diodes and many have failed!!

I have an idea!! Why don’t we specify the diodes with a better rating ?

Figure 3.4   Why Purchasing Specifications ?

Next Month:

Assembly Line Results

Test Department

Good 10 %

10 %

100 %

INCREASE

Rejects

90%

FORM FOR QUALIFYING OUTSIDE VENDORS/PRODUCTS 
AND TRACKING AND VERIFICATION 
(Form 7.4.1-1)

NAME OF VENDOR

1) IS VENDOR CERTIFIED TO ISO 9000 QUALITY STANDARDS, OR EQUIVALENT?

Yes

No

2) PREVIOUSLY PURCHASED PRODUCTS FROM THIS VENDOR?

Yes (if Yes, answer Questions 3 and 4)

No (if No, skip to Question 5)

3) EXPERIENCE WITH PREVIOUSLY PURCHASED PRODUCTS

Parts previously supplied:							______

Quantities of parts previously supplied:

Audits previously performed:							______

Failure rate and acceptance testing: 					______

4) COMMENTS ON BUSINESS PRACTICES OF VENDOR (service, delivery, warranty, etc.)

5) INFORMATION GATHERED FROM OTHER ORGANIZATIONS THAT HAVE USED THIS VENDOR

6) REFERENCES ON VENDOR (Better Business Bureau, etc.)

7) DOCUMENTATION FROM VENDOR (warranty, service agreement, references, product specifications, etc.)

3.5  Production and Service Provision	 (Clause 7.5)

This important clause deals with the product or service the company provides to its customers, which is the main goal of the business.  Whatever is provided must be of good quality and must satisfy all the requirements of the customer.  This clause also deals with the delivery and post-delivery activities, which are also  critical  in maintaining customer satisfaction.

3.5.1 Control of Production and Service Provision (Clause 7.5.1)

This requirement addresses the need for controlling the processes that critically affect product or service quality. The standard that deals with the need for planning to ensure constant controlled conditions was discussed in section 2.2.4. There is no specific requirement for operations that involve no risk to the quality of the end product or service. In such non-critical operations, adequate training of a non-skilled operator, or a brief description of the steps in the process will suffice. This is usually summarized in simple work instructions.

In order to determine which operations are critical, you must have a full knowledge of the requirements and specifications that the product or service must fulfill.  After a design or development function as described previously is completed or based on previous experience, a documentation package is presented with interim specifications which can be your present procedures.  The critical areas are those that contribute more strongly to noncompliance conditions if they are not performed properly and those that have higher complexity.

For all critical operations, written specifications and process procedures are required.  These must detail clearly what to do, how to do it, and the equipment required together with the set-up parameters for its use. Only equipment that has been approved for the task should be used. To avoid costly errors, fully qualified and certified operators shall carry out these operations. The operator should know the reason for the requirements and the operation’s impact on the quality of the product.  All critical operations should be monitored and measured and reported for compliance and the measurement equipment properly maintained and calibrated.

To control critical processes satisfactorily, your company must take the following steps:

a) Information, documentation and work instructions:

- Document work instructions and the sequence of processes.

- Provide up-dated process specifications accessible to all operators

- Monitor and frequently audit documented procedures to assure inclusion of the latest approved revisions or changes (Form 7.5.1-1)

- Evaluate and document the impact of any process change on quality

b) Equipment related issues:

- Use suitable equipment

- Use only equipment that has been approved for manufacturing

- Monitor and control all process parameters

- Review and list all necessary equipment compliance with codes, standards, quality plans and documented procedures

c) Monitoring and measurement:

- Monitor maintenance and proper operation of equipment

- Establish a clear workmanship standard and representative expectation for the output in a measurable form of data, shape, color and the like, with the aids of test equipment or color charts and similar tools.

- Provide statistical data to show in measurable terms that the operation is under control conditions. Data should be displayed whenever possible.

- Provide training as necessary to maintain always qualified operators in all critical steps

Implementation and record keeping:

- Generate a system to maintain quality records

- Set up maintenance records that document scheduled maintenance requirements

- Keep records of all critical parameters and calibrations

- Keep record of all non-planned maintenance requirements and their downtime effect or losses

- Try to maintain a very simple and clear system

Process control is a basic requirement that originates from the knowledge of the requirements and specifications.  The processes to obtain the product or service are based on the experience on what is critical for the product to meet all requirements.  This is usually demonstrated through the verification and validation process, which shows the inputs and outputs required from each step in the provision of the product.  The need for training and assistance for the operators is based on these experiences.  The auditors rely on these records to be able to verify that the processes are under control at all times.   REMEMBER:  the aim of the ISO standard is to maintain the process and quality under controlled conditions. The sole purpose of paperwork is to verify that the controls are in place, and that the specifications and process documentation are up to date and accessible to the operators.

PROCESS CONTROL  (Form 7.5.1-1)

- Date______________

- Process/Operation: 						Revision #	______

- Work Instructions (attach, if necessary): 						______

_______________________________________________________________________

_______________________________________________________________________

_______________________________________________________________________

- Process Input: 								_____________

- Staff Requirements:									__

- Training Requirements:								_______

_______________________________________________________________________

- Equipment Requirements:								_

______________________________________________________________________

- Quality Monitoring/Testing: 								_

______________________________________________________________________

- Storage/Handling Requirements:							______

______________________________________________________________________

______________________________________________________________________

- Process Output: 									______

_______________________________________________________________________

###### Figure 3.5 Process Control

1

Work Instructions and Processes

3

Flow Chart

2

Process Engineering

Document Process

9

What do you do?

How do you do it?

Data

11

Output

Yield

8

10

Operator

Maintenance Log

12

Calibrated Equipment

flow meters

Calibration Stickers ickers

7

6

Furnace

5

Temperature Controller

File

Operator

Auditor

Inspection and testing

This clause also requires establishment and maintenance of procedures for inspection and testing activities to assure that the product manufactured meets performance requirements.  The quality system should spell out all the inspections and tests that are to be conducted and documented before, during, and after the manufacturing process begins, along with the steps that are necessary for the release of the product to shipment. (Forms 7.5.1-2 and 7.5.1-3)

The inspection process can be divided into three stages: incoming materials or receiving inspection; in-process inspection; and final inspection and test. 	 Each stage of inspection and its requirements shall be identified in the Quality Plan and shall be performed on all materials and parameters that could affect quality to guarantee that the end product will meet customer requirements. The testing requirements and how they will be carried out are to be clearly defined. The operators responsible for the inspection or testing must be given the appropriate equipment and training.

Incoming Inspection

- The supplier should have written specifications for all raw materials used, or detailed acceptance criteria in the purchase order.

- The quality plan and documented procedures should address any specific requirements not covered in the material specifications.

- No material should be released for use before incoming inspection.

- If an emergency release of material is required, the material should be positively identified to allow for recall of the finished product, in case a subsequent problem is found with the quality of the original material after the product is finished. Examples could be the absence of a UV inhibitor, or an unknown change in plastic formulation.

Consideration should be given to the certification and inspection of the subcontractor and its past quality performance and audit of its established quality system. The need for incoming inspection could be waived in such circumstances; but should be identified previously in the Quality Plan and documented accordingly.

In-process Inspection and Testing

- The product should be tested or inspected to verify that it meets the required quality procedures as described in the Quality Plan before further processing. For example: testing the resistivity of diffused layer of wafers prior to further processing.

- The product should be held until the required inspection and tests are completed. If the product is released before acceptance under established procedures, it should be marked appropriately so it can be recalled and preclude shipment of a non-conforming product.

- All non-conforming materials should be treated according to the non-conforming material procedures and marked appropriately, as well as removed from the production line.

- Individual operators may be responsible for checking their own work. This flexibility is essential for small businesses where duplication of efforts is to be avoided.

- Records should confirm that operators or inspectors have performed the necessary tests at the intermediate process steps.

FORM FOR MONITORING PRODUCTION PROCESS 
(Form 7.5.1-2)

1) NAME/DESIGNATION OF PRODUCT

2) UNIQUE ASSIGNED TRACKING ID (alpha/numeric)

3) Procedures and Instructions for Quality Functions

Review Procedures for Production Quality System for Your Company (sections 
	IV) QUALITY SYSTEM PROCEDURES; 
	IX) CONTROL OF CUSTOMER-SUPPLIED MATERIALS
	X) PRODUCT TRACKING

4) ATTACH ORIGINAL DOCUMENTATION ON EQUIPMENT USED IN EACH STEP

5) Monitoring of Process Conditions

Process Step:

Acceptable Range	Current Measure	Date

Temperature

Humidity

Barometric Pressure

Cleanliness

Process Step:

Acceptable Range	Current Measure	Date

Temperature

Humidity

Barometric Pressure

Cleanliness

Process Step:

Acceptable Range	Current Measure	Date

Temperature

Humidity

Barometric Pressure

Cleanliness

6) Monitoring of Product

(Use Form 7.5.3-1 page 106 of this Manual, “Product Tracking and Verification”)

Procedure and Schedule for Equipment Maintenance 
(Form 7.5.1-3)

1) Equipment Item:

2) Date Purchased: 					New:			Used:

3) 	Attach Copy of Original Equipment Documentation

4) Maintenance Schedule: (Weekly, Monthly, After “X” Cycles, etc.):

5) Name of Operator:

6) Maintenance:

Date		Who Performed	Completed?	Action/Repair		Notes

Final Inspection and Testing

Final inspection is your company’s last opportunity to assure that all specifications and parameters are met. This process should be documented to the customer’s satisfaction as shown in Form 7.5.1-4   It is also important to verify that all the inspections, tests, and paperwork have been performed on the product before it is released. This could be achieved by a simple, but consistent methodology. A check mark in a routing slip is all that is necessary to verify compliance.

The final inspection and test could be specified within any of the following:

- A quality plan

- A sampling plan

- A procedure

- An instruction

- An inspection and test plan

- The customer’s order

An inspection authority should be empowered to make sure that all the contract details are fulfilled.  Records should indicate when the product is released. All documentation shall be kept to demonstrate that the Quality Plan has been satisfied.

A contract or special order may also have specific shipping requirements. A responsible authority must be sure that all shipping-related paperwork, packaging, markings and identification of product conform to the contract.  All records related to the contract should be kept for a specified time and be made available for customer review upon request.

DOCUMENTATION FOR RECEIVING, IN-PROCESS, AND 
FINAL INSPECTION AND TESTING  (Form 7.5.1-4)

1) NAME/DESIGNATION OF PRODUCT

2) UNIQUE ASSIGNED TRACKING ID (alpha/numeric)

RECEIVING

3) PRODUCT RECEIPT				BY (initials)		DATE/TIME

VISUAL INSPECTION

Accepted

Rejected

REASON FOR REJECTION AND DISPOSITION OF THE PRODUCT:

4) IN-PROCESS

QUALITY TEST

Pass Performance Check

Pass Quality Check

Rejected

REASON FOR REJECTION AND DISPOSITION OF THE PRODUCT:

4) FINAL

QUALITY TEST

Pass Performance Check

Pass Quality Check

Rejected

REASON FOR REJECTION AND DISPOSITION OF THE PRODUCT:

5) POSITIVE RECALL (When products are released prior to verification: e.g., Urgent Release)

REASON FOR RELEASE PRIOR TO VERIFICATION (customer request, etc)

IDENTIFICATON AND DOCUMENTATION OF PRODUCT/CUSTOMER

NON-CONFORMITY EVENT/DATE OF RECALL

Servicing

Servicing, also part of this clause, includes delivery and post-delivery activities.  This section requires that the supplier establish and maintain documented procedures for performing the servicing function and verifying that it meets specified requirements.

Suppliers frequently offer installation services for their products. Also, the supplier may offer additional services, such as maintenance, or may agree to conduct a performance test of the installed equipment, etc.  In such cases, procedures should be in place to document the result of these services. (Form 7.5.1-5)

As example, a supplier may commit to testing a solar system after installation for power output, orientation, potential shadows, proper wiring, status of storage battery system, and the like.  A document should be filed and a report issued with the findings and any correction made to the installation. A similar case occurs when a servicing organization is responsible for verification of the performance and soundness of the system after it has been in use for a long period. This inspection can help detect corrosion issues, early module or equipment failures, water penetration, and problems with the encapsulant in the modules.

In cases where a warranty claim is involved, a written record and report are indispensable to back the claim to the supplier and to get the system back into proper working order.

### Planning and Scheduling Servicing Activities

The plan depends on the type of arrangement between the supplier and the customer, for example on whether the supplier is a service-only (installation and repair) organization or whether it is also the provider of the product to be serviced.

### Spare Parts Availability and Management

This is a very important issue, especially if the system is installed at a remote site where access to the end user involves a long and inconvenient journey.  The points where batteries are exchanged are often useful points of access for service and maintenance.

### Post delivery activities.- Instructions and Records

Good record keeping is especially important should warranty claims or complaints about product performance arise.   One example is when the interconnects in a module fail. This limits the power output of the module, but is not a catastrophic failure.  Previous measurements are key to being able to detect the type of failure mode.  Therefore, good records and adequate instructions on what measurements to take are essential for early detection and can prevent the supplier from having to replace the entire module.

The basic tools needed to control the quality of servicing are the following:

- Personnel education and training

- Records of product failures and non-conformance

- Instrumentation and proper instrument calibration

- Audits of services and reports to Management

The service department must be organized to issue reports of their findings that become part of the quality record.

SERVICING DOCUMENTATION FORM (Form 7.5.1-5)

1) CUSTOMER NAME

2) CUSTOMER LOCATION

3) PRODUCT NAME

4) PRODUCT UNIQUE ID

5) DATE PRODUCT PURCHASED/INSTALLED

6) SOLD BY/INSTALLED BY

7) SERVICING INFORMATION/EDUCATION PROVIDED TO CUSTOMER?

8) SERVICING HISTORY

Scheduled Servicing (attach agreement and schedule)

Date:		Technician:

Date:		Technician:

Date:		Technician:

Date:		Technician:

Date:		Technician:

Unscheduled Servicing

Date:		Technician:

Problem:

Resolution:

Customer Acceptance (signature & date)

Date:		Technician:

Problem:

Resolution:

Customer Acceptance (signature & date)

Date:		Technician:

Problem:

Resolution:

Customer Acceptance (signature & date)

(if product is replaced, document the date and new Unique Product ID)

9) REPORT AND DOCUMENTATION FILED WITH SERVICE AND QUALITY MANAGERS

SERVICE MANAGER/DATE

QUALITY MANAGER/DATE

3.5.2 Validation of Processes for Production and   Service Provision                                        (Clause 7.5.2)

Tests are made to materials in progress in the manufacturing area as we discussed in the previous section.  The idea is to eliminate from the manufacturing cycle those products that do not meet the requirements previously designated as necessary to generate a product that meets the specifications desired.  These tests consist basically of analysis by inspection or testing of the output of certain critical processes.

The problem with testing some products is that sometimes either the test is destructive or it cannot be performed in real time.  These defects may affect product reliability later in life, several months after installation.  Although the final factory tests were satisfactory, they could not reveal the latent defect.

The purpose of this clause is to deal with what to do when it is not possible to monitor or inspect a problem.  The solution is to qualify the procedure by validation of the process.  The emphasis is to analyze all the parameters that surround the process, that is:  the equipment, its parameters, the materials, the qualifications of the personnel, the environmental conditions, and the procedures and preservation of the product.

Once all these parameters are optimized by test and validated from results, the process is documented and the inspection is performed about the future conditions that should be the same as those validated.  If the conditions change, for example a new piece of equipment is introduced, then the process must be revalidated.

This type of problem is readily visible in the interconnects for solar cells and their degree of reliability under extreme temperature cycling.  This also applies to the performance and reliability of the metallization of the solar cells and the variables surrounding the interconnect attachments.

In other industries, the results of a weld and the strength of a concrete slab after casting are prime examples of processes that require controls on process, since products cannot be tested without destroying the pieces.  Testing the interconnects and the solar cell bond strength are also destructive tests.

Thus, the records of the process used in these examples are very important to demonstrate that the process was under control.  In the case of the solar cell interconnects, the problem was detected 4 years after field deployment. The records and design parameters were used to find the solution to the problem and to reduce the stress that promoted those early failures.

3.5.3 Identification and Traceability	(Clause 7.5.3)

The need for product identification is common in manufacturing. The supplier should establish a system that can identify the product and aspects of its status that are essential to maintaining quality and avoiding costly mistakes. This identification system should also identify changes in documentation and revision levels. You can use Form 7.5.3-1 or 7.5.3-2, depending on whether your company provides installation services.

For example, in a solar cell factory, cleaned wafers must be identifiable from new uncleaned wafers and also from diffused wafers. This is usually achieved by some combination of:

- Job card entries

- Tagging

- Computer tracking

- Batch numbers

Some contracts may require further traceability of the raw materials used in the final product.  In this case, the supplier shall establish procedures to document the flow of product and identify the materials used. This identification can be achieved through the use of:

- Material tags with identification numbers of lots or quality

- Batch processing data

- Individual data in computerized form

- Visual inspection data

There are other important details to be taken into account in product identification and traceability, such as how to:

- Segregate non-conforming materials and mark them properly

- Be able to demonstrate that only approved material is permitted to progress from one operation to the next

- Record and maintain unique identification numbers for products

- Maintain quality records and tracking records as required by the customer contract

The supplier should identify meaningful parameters and traceability requirements and evaluate what is truly important for the quality of the product. This section does not require tracing and identifying everything (unless the contract spells out this need).  Paperwork and data should be kept to a minimum and limited to significant information. What is “significant” should be established during the design phase of the product or service.

Figure 3.6 gives examples of the types of tags, identifiers, routing slips and process records that are typical for a manufacturing process.

Inspection and test status

Most businesses have some method of identifying the status of products or services provided.  The ISO 9001:2000 standard requires that suppliers implement the following controls:

- Establish a quality system for identification of the inspection status of a product during all stages of manufacture. (Form 7.5.3-1)

- Classify product as:

- Not inspected

- Inspected and approved (by a previously described criteria)

- Inspected and rejected

- Inspected and rejected and put on hold for further action

- Inspected and released under authorized concession.

- Establish procedures for maintaining records of inspection and individual(s) responsible for releasing conforming product for further action.

- Be able to demonstrate that only products that meet the specific quality requirements have been released at the various stages of manufacture.

- Establish a recording and reporting system for the critical parameters of the product.  Data should be posted so that all operators and managers become aware of the performance and control of these parameters.  The statistical section of this Manual shows ways you can chart average parameters, as well as deviations and/or drifts in the manufacturing process.  These data are essential to keeping the process under control or implementing product improvements.

Reason for rejection

Engineering

Disposition

Corrective Action

Calibration Record

Unit

Date

Operator

Deviation Found

Routing Sheet

Lot #

Step 1

Step 2

Step 3

Data Step 1.

Data Step 2.

Data Step 3.

Date

Next Step

Received by

Date

Received

Passed

Reject

Operator

Time

Test Record

Module #

Recorded Data

Voltage Peak

Current Peak

Peak Power

Power at NOTC

Operator

Date

V

A

W

W

Figure 3.6  SAMPLE TAGS AND LABELS

PRODUCT TRACKING AND VERIFICATION 
(Form 7.5.3-1)

(Copy to Remain with Product/Shipment)

1) NAME/DESIGNATION OF PRODUCT

2) UNIQUE ASSIGNED TRACKING ID (alpha/numeric)

Product Tracking

3) LOCATION	RECEIVED BY	          DATE/TIME		  DOCUMENTED

(ex. Receiving)	(initials)            (ex. 99-10-01, 08:35)	       (ex. signed receipt)

Product Verification

4) PRODUCT RECEIPT				BY (initials)		DATE/TIME

A) VISUAL INSPECTION

Accepted (go to B)

Rejected

REASON FOR REJECTION:

B) CHECK ORDER

Product Matches Order/Accepted

(go to C)

Product Does Not Match Order/

Rejected

C) QUALITY TEST

Pass Performance Check

Pass Quality Check

Rejected

REASON FOR REJECTION:

PRODUCT TRACKING* 
(Form 7.5.3-2)

(ID shall be Permanently Affixed to the Product)

1) NAME/DESIGNATION OF PRODUCT

2) UNIQUE ASSIGNED TRACKING ID (alpha/numeric)

Product Tracking

PRODUCT DEVELOPMENT

3) LOCATION	ACCEPTED BY	DATE/TIME		DOCUMENTED

(ex. Warehouse)	(initials)		(ex. 99-10-01, 08:35)	(ex. initialed form)

PRODUCT DELIVERY

4) PRODUCT DESCRIPTION

5) DELIVERY FROM/LOCATION

6) DELIVERY TO/LOCATION

7) A) DELIVERY AGENT

B) PICK-UP DATE/TIME

8) A) RECEIVED BY								 _____	         B) RECEIPT DATE/TIME

C) Documentation/Manuals/Installation Instructions

D) DELIVERY OF SAFETY INFORMATION/INSTRUCTIONS

INSTALLATION

9) A) INSTALLED BY

B) INSTALLATION DATE/TIME

10) INSTALLATION TEST			PASS			FAIL	___

11) CLIENT ACCEPTANCE: Signature/Date

## Note: For companies that provide installation services.

3.5.4 Customer Property          (Clause 7.5.4)

On many occasions, the customer supplies a product to the supplier as part of the end product to be provided.  The supplier shall maintain documented procedures for the verification, control, storage, maintenance and handling of the customer-supplied product. If any damage or failure occurs with the product, it should be recorded and the customer should be advised of the problem. (See Form 7.5.4-1)

An example of this could be customer-supplied batteries for use in a finished system. The batteries should be kept properly in a vented area, with adequate temperature control, and charged as required during the time of storage.  Failure to do so will result in premature battery failure after the finished products are delivered.

The product supplied by the customer should be inspected for quality, and a relevant procedure submitted to the customer before delivery. The supply of an acceptable product is the responsibility of the customer, even if the product has been accepted and verified by the supplier.

The product should be identified by serial numbers and controlled storage conditions to avoid mixing them with similar product in the supplier’s warehouse or with material supplied by other customers.

Similarly, your company might use a service provided by the customer.  In one instance, a customer purchased a system for a radar beacon in Alaska. The supplier provided details of sizing, installation, battery enclosure, depth burial to avoid freezing, and the like, but the site preparation was left up to the customer.  After a few months of operation, the system was in trouble and the customer called the supplier. After several hours of telephone exchange, the supplier realized that while the system had been installed in a forest reserve with trees 16 meters tall, the system was installed at a height of only 9 meters. In the month of December, the sun in Alaska barely clears the horizon, thus the system was not getting enough sunlight to generate energy adequate to sustain the load. Changing the height of the system the next year resolved the installation problem. In this case the customer’s installation was clearly defective and caused the system to fail.  Better communications resolved the problem in this case, which arose due to customer-supplied service.

Similarly the intellectual property of the customer must be protected and verified as useful for a service or an application of equipment.  All data shall be evaluated and protected to preserve customer confidentiality.

CUSTOMER-SUPPLIED PRODUCT TRACKING AND VERIFICATION 
(Form 7.5.4-1)

(Copy to Remain in Customer File)

1) CUSTOMER NAME

2) NAME/DESIGNATION OF PRODUCT

3) PRODUCT/ORDER FOR WHICH THIS IS DESIGNATED

4) UNIQUE ASSIGNED TRACKING ID (alpha/numeric)

Product Tracking

5) LOCATION	RECEIVED BY	DATE/TIME		DOCUMENTED

(ex. Receiving)	(initials)   (ex. 99-10-01, 08:35)	(ex. signed receipt)

Product Verification

6) PRODUCT RECEIPT				BY (initials)		DATE/TIME

A) VISUAL INSPECTION

Accepted as is (go to B)

Not Accepted

NOTE PROBLEMS:

B) CHECK ORDER

Product Matches Order/Accepted

Product Does Not Match Order/

Not Accepted

C) QUALITY TEST

Pass Performance Check

Pass Quality Check

Rejected

REASON FOR REJECTION:

7) SPECIAL CONDITIONS FOR STORAGE AND/OR HANDLING

8) REPORT TO CUSTOMER ANY PRODUCT THAT IS LOST, DAMAGED, OR UNSUITABLE FOR USE

3.5.5 Preservation of Product          (Clause 7.5.5)

This ISO clause requires that the commercial functions of product handling, storage, packaging, preservation and delivery be controlled by documented procedures, in order to avoid problems in these areas and to identify requirements that will eliminate error in the performance of these functions.

### Handling

Many products require specific procedures to avoid risk, damage or problems in the handling of raw materials, the handling of the product throughout the manufacturing process, or the handling of the final product in the stockroom prior to shipping.

Examples of such problems include the improper handling of semiconductors sensitive to electrostatic discharges, the storage of electrical or electronic equipment in high humidity or dusty environments, overstocking product to the extent that the weight could damage the shape or boxes, the mishandling of acids or liquids of corrosive nature, the handling of metals with improper equipment, etc.

Handling procedures shall be simple and clearly explained.  They should describe the method of handling, tools or precautions required during the handling, movement of the product through the plant or, after completion acceptance, through storage and shipment.

### Storage

Many products have specific storage requirements before use or shipment of the finished product.  A good example is the storage of batteries in a systems integration operation.  Batteries require periodic recharging to prevent plate corrosion.  The room the batteries are stored in requires ventilation, because of the corrosive liquids involved. Furthermore, specific temperature ranges must be maintained in order to avoid rapid deterioration of the units.

As another example, most plastics or laminating materials used in manufacturing PV modules have shelf lives that cannot be exceeded, if a quality product is desired. Sometimes a cycle of “first in, first out” is required for perishable materials, or order quantities have to be restricted and delivery schedules carefully set so as to avoid shelf life problems.

Problems that can arise in the location of different chemicals should be taken into account in storage facility layout. Attention to temperature needs is also necessary for safe storage of volatile and sensitive products.

The manufacturer should issue norms for storing raw materials and finished products to avoid any damage to products or employees, due to improper storage conditions.

### Packaging

The packaging of finished products is of great relevance for the manufacturer of PV products.  Regulators are semiconductor-based units that are sensitive to humidity and electrostatic problems.  Batteries are highly corrosive devices.  There are rules and regulations for their handling during the shipping process, including special crating and labeling. Sometimes when shipping batteries, the electrolyte must even be shipped separately in glass containers.

PV products are sometimes fragile.  This is in the case of cells and wafers.  In other cases, PV products are heavy and consequently, crating should have protection against mechanical damage from other metal components – for example, when a system containing several modules in an assembly is shipped together with metal hardware.

In all cases, procedures should be established for the shipping and marking of products in order to avoid costly mistakes due to breakage, damage or product deterioration.

### Preservation

Quality control methods are required for the preservation of product during storage or use in the factory.  In the case of chemicals, the segregation of acids from bases during storage is very important.  And again, the handling and storage of batteries and laminating materials requires special preservation, such as temperature control and cycling of use to prevent damage.

Delivery

This includes the proper labeling and insurance of the product for delivery to the customer and the necessary written instructions on the proper method of shipment. Other requirements were discussed in the previous sections on handling.

The container used for normal delivery should be an approved and tested container that protects the product from damage en route, such as rough handling by the shipping company.  Markings are important to identify dangerous or fragile products.

### Case Study # 3

# BATTERY REQUIREMENTS FOR STAND-ALONE OR AUTONOMOUS SYSTEMS

All stand-alone PV systems require a device that stores the energy produced during the day so it can be used when needed.   This is particularly important in the case of rural home systems that are used at night or transmission systems that must be available at all times.

There are various types of storage devices.  The most common is a set of batteries designed to be able to hold energy for many days, so that the system can function even during winter periods when the amount of solar energy received is very low.  Storage capacity can vary, depending on the anticipated usage charge and usage conditions and on how critical the system is.  The cost of batteries – the most short-lived component of a solar system – is also an important factor.

Storage times range from 30 days, in the case of navigational aids like marine buoys, to three days in the case of remote home systems.  Telecommunication systems typically have a storage time of up to seven days, depending on the location site and the criticality of the operation.

The type of battery most frequently used is the lead acid battery.  The following two examples illustrate battery-related issues of concern for suppliers and system integrators.

## Example #1:  Transportation Requirements

The location of the system should be a major concern for the system installer or integrator, especially in the case of large telecommunication systems in which the battery size demands the use of a liquid electrolyte. A few years ago, several sets of such systems were sold that were to be located on mountaintops. The lack of attention to their location had a negative impact on the performance of the systems.

The customer failed to mention that there was no access to the locations by road. The supplier simply assumed such access.  The supplier did not know that the equipment was to be transported up the mountains on the backs of donkeys.  No special preparation was made for the transportation of the large batteries, which were shipped filled with acid, instead of shipped dry-charged with the electrolyte kept in separate containers.  Several instances of lost electrolyte went unnoticed and the batteries went into service at sub-par conditions, diminishing the life of the systems.  In other instances, the electrolyte spilled and burned animals and operators handling the cargo, creating a safety concern for the system integrator.

## Example #2:  Storage of Battery Systems

Many applications involve multiple systems that are installed over a lengthy period of time and that require storage of systems that have been delivered ready to be installed.  In such

situations, a strong warning is warranted, since the storage of battery systems is limited to a few weeks at a time.  After a few weeks, the electrolyte -- liquid or gel -- starts to sulfate the plates of the battery and a film develops that affects future life and capacity.   In one case, a group of 50 navigational aid systems were shipped to a customer in the Middle East.  After deployment, several problems developed with the charge retention (capacity) of the batteries.  Several trips were made to analyze the solar system size, the controller operation and settings, as well as the installation.  It was subsequently discovered that the systems had been kept in a warehouse for more than six months, awaiting the arrival of other equipment.  The temperature of the warehouse and, consequently, of the batteries rose as high as 50 º C.  These temperatures accelerated the self-discharge and activity of the electrolyte.

The customer did not pay attention to the warning labels that were placed on the systems and in the manuals, which specifically instructed that batteries be recharged during extensive periods of storage, especially when exposed to high temperatures.

The lesson to be learned here is that, in order to deliver a sound system, the designer should not only take into consideration the insulation and weather for the area, but also the transportation, handling and storage of the units from the time they are delivered through the actual time of commissioning by the customer.   See form 7.5.5-1 for example.

SAMPLE  COMPONENT/PRODUCT HANDLING FORM 
(Form 7.5.5-1)

1) PRODUCT NAME:

2) UNIQUE ID:

Activity		Procedure				Conditions

Handling	(e.g., Use gloves when handling)	(e.g., Minimize vibration)

Storage	(e.g., Do not stack)			(e.g., Maintain temp.10-35°C)

Packaging	(e.g.,Use bio-degradable pkg)  (e.g., Minimize dust in packing area)

Delivery	(e.g., Customer education)	(e.g., Verify operating environment)

____________

Signature of Reviewing/Inspecting Manager			Date

3.6 Control of Monitoring and Measuring Devices

(Clause 7.6)

Throughout the production process, the supplier utilizes equipment for inspection, testing, and measurement.  In order to assure that the product meets its requirements, it is necessary to establish and maintain calibration and maintenance schedules for all measuring and test equipment.  This ensures that the measurements are correct and within acceptable accuracy requirements.  A record of this process should be kept in a file for future reference. (Form 7.6-1)

The use of equipment requires that any measurement uncertainty is known and consistent with requirements.  Any hardware and software used to test the product should be checked regularly to verify the accuracy and repeatability of their test results.  Records should be kept of this process.

The standard requires that the calibration be performed at specified intervals, that the equipment is adjusted as necessary, that the equipment cannot be readjusted by unauthorized personnel and that the equipment is kept and operated within an environment adequate to achieve accurate measurements.

Control Procedure

- Identify the equipment that needs to be calibrated. Only equipment that controls critical parameters or measurements required by the product specifications needs to be calibrated.

- Establish and maintain a list of each piece of equipment that requires calibration.

- Demonstrate the calibration status of each piece of critical equipment. Records should be kept on when equipment was calibrated, who did it, the acceptance criteria, what the result was and if it was acceptable, plus the next date for calibration.

- Identify and document the accuracy of required measurements and the accuracy of the equipment in use to perform those measurements. Demonstrate that the equipment in use has better accuracy or that it at least meets the accuracy requirement.

- Establish calibration methods traceable to a certifiable standard to maintain the accuracy of tests and measurements. Establish frequency of calibration and monitor records of calibration. Equipment or meters shall not be used beyond the effective calibration date.

- Set up a certificate of calibration and tag each unit or equipment with the date that recalibration is due, the accuracy of the calibration and the conditions of operation.

- Make sure that the conditions of operation are within the environmental conditions of the calibration; otherwise correction factors must be utilized or the equipment must be calibrated under the anticipated usage conditions (e.g. in the case of high temperature or high humidity beyond the range of the calibration).

- Use of secondary standards is permissible when the measurement requires less accuracy than that provided by the unit calibrated to a certifiable standard.

- If any test equipment is found to be operating outside the calibration or accuracy required by the product, an effort must be made to find when the unit went out of calibration, and how the product shipped during that period could be affected. The product should be listed as potentially noncompliant to the specification. A non-conforming product review must be done to decide what action should be taken.

- Provide adequate storage and handling of calibration equipment so that accuracy and fitness for use can be maintained.

The calibration process can be expensive. Every effort should be made to minimize the number of units that require external calibration with traceable references, because these may not be readily available in all locations. You may use a subcontractor that has this type of capability or cooperate with other companies that have the same requirements to share the expense. Neither expense nor difficulty waives the need for accuracy in testing, but the accuracy should not be overstated beyond the need or the capability of the available equipment.

The process of calibration requires proper documentation procedures and maintenance of records for backing up the measurements performed at the company or at the field.  Auditors are particularly sensitive in regard to the calibration process. All records can become critical for demonstrating that specifications and requirements have been met.

PROCEDURES FOR QUALITY CONTROL 
OF MEASUREMENT AND TEST EQUIPMENT 
(Form 7.6-1)

1) Equipment Designation:

2) Date of Calibration:

3) Employee/Contractor Performing the Action:

4) What is the Item used to Test/Measure:

5) What is the Measurement Tolerance for the Process:

6) What is the Measurement Uncertainty:

7) Number of Tests/Cycles since Last Calibration:

8) Return of the Item to its Stored Capacity?	Yes		No

9) Date Item Returned to Storage:

10) Location of Item Storage:

(Note: This is only applicable to those businesses where measuring or testing equipment is used to check that what they are providing meets the customers’ needs. Also, this applies only to equipment that can affect quality.)

Homework Assignment – Chapter Three

YOUR NAME: ________________________________

YOUR COMPANY:  ___________________________

Today’s homework assignment is intended to help you think about a plausible quality management situation in your own company and about what you will do when you return to your company after this training to promote the adoption of a quality management system to qualify for the PV GAP Mark or Seal.

- Please describe the key elements of a quality system you would set up to deal with customer complaints about one of your company’s products?  (The complaints might have to do with the product’s performance, with its condition upon receipt after shipping or with after-sale service).

- If the quality system you described in #1 were in place, how would your company benefit and what advantages would the system give your company relative to its competitors?

- What are the key messages you will take back to your company from this training? What specific things will you do in the next thirty days to communicate what you have learned here with others in your company and start the process of change?