QUALITY MANAGEMENT

IN PHOTOVOLTAICS

##### Appendices

- How Expensive is it for a PV Manufacturer to Set-up

and Operate a Quality Management System?

- Example of a Standard Quality Plan

- Publications Issued by IEC TC 82

- IEC Standards of  Possible Use for PV

- PV GAP Recommended Standards (PVRS)

- Blank Product Quality Assessment Specification (B-PQAS)

- IECQ Supervising Inspectorates

- Provisional List of Testing Laboratories

- Glossary of Terms

- Bibliography

Appendix 1:

How Expensive is it for a PV Manufacturer to Set-up and Operate a Quality Management System?

The Global Approval Program for Photovoltaics (PV GAP) was initiated by the worldwide PV Industry, supported by financial institutions and by government agency funding programs to establish an easy recognition system for quality PV products. PV GAP created a PV Quality Mark for PV components and a PV Quality Seal for PV Systems.  Manufacturers will be licensed to display the Mark and/or Seal on their products if they fulfill the approval requirements. In order to minimize expenses, PV GAP is at present relying on IEC’s Quality Assessment System for Electronic Components (IECQ).

The manufacturer who is able to display the PV GAP PV Quality Mark or Seal on a product derives obvious benefits.

- The manufacturer’s quality product is distinguished from products of unknown quality not having the PV Quality Mark or Seal.

- Besides the marketing advantage of raising the customer’s confidence in the products,  products displaying the PV Quality Mark or Seal have another important advantage.  They are accepted in every country, while products not qualified to display the Mark or Seal must present credentials in every country.  This is illustrated by the World Bank decision (http://www.worldbank.org/energy) to advise its clients to accept products displaying the PV GAP Mark and Seal while those not having the Mark or Seal must be re-certified country by country according to each country’s requirements.

The following is required for IECQ approval and PV GAP licensing of the PV GAP Quality Mark and or Seal:

### PV GAP

Requirements to receive IECQ approval:

- Quality Manufacturing System (ISO 9001:2000),

- Product manufactured and tested according to International Standards (IEC or PVRS) by the manufacturer testing facility (included in his ISO 9001:2000 certification) or by an accredited  (ISO/IEC 17025) Testing Laboratory

- “Periodic Audit” of the manufacturer and the product

PV GAP function:

- Based on approval PV GAP licenses the use of the Mark and/or Seal

The obvious question is how expensive is it for a manufacturer to obtain PV GAP

approval to display the PV Quality Mark and/or Seal on its products? Can a small or medium size company afford it?

Apart from testing expenses, the costs to establish the needed requirements for a manufacturer depends on the size of the company. For a smaller company it will be less and for larger ones more.

CASE 1:

Assumptions:

- As an example, let’s consider a small/medium PV cell and module manufacturing company which produces 1 MWp/year and has 50 employees Some manufacturers obtain test certificate for their products from an accredited (ISO/IEC 17025) testing laboratory (in case of modules according to IEC 61215 for crystalline and IEC 61646 for thin film standards), but let’s assume that the manufacturer starts from scratch, has no approved manufacturing quality system and has to set up the Quality Manufacturing (ISO 9001:2000) system in its manufacturing operation and must have its modules tested at an accredited testing laboratory.

- Another assumption is, that the sales price of the module is US $ 3.00 per Wp.

First Year expenses:

1)	ISO 9001:2000: Setting up the Quality Manufacturing System.

The PV GAP Quality Control Training Manual provides the necessary information to help a manufacturer set up the ISO 9001:2000 system in their operation. An  estimated 2 man months are approximately needed in order to install the system, including the 5-day training course. The expense for this would vary from country to country.  Taking Germany as an example, it is estimated that this would cost Euro 16,500 (about US $15,000).

A comparison, (based on the experience of consultants who helped to set up the ISO 9001:2000 system in a company of that size), indicates that a Consulting organization would charge the manufacturer about US $ 25,000 to perform this service.

- The manufacturers ISO 9001:2000 certification

The European co-operation for Accreditation has published a guideline of the suggested auditor days based on the number of employees. This is shown in the following chart: As can be seen the number of “auditor days” depends on the size of the company (number of employees).

| Certificated entity: # Number of employees | Initial assessment (Auditor Days) | Initial assessment (Auditor Days) |
| --- | --- | --- |
|  | Total | On-site minimum |
| Less than 5 | 2 | 1 |
| 5 – 9 | 2.5 | 1.5 |
| 10 – 19 | 3 | 2 |
| 20 – 29 | 4 | 2.5 |
| 30 – 59 | 6 | 4.5 |
| 60 – 100 | 7 | 5 |
| 100 – 250 | 8 | 6 |
| 250 – 500 | 10 | 7 |
| 500 - 1000 | 12 | 9 |

Based on information received from VDE of Germany this would convert into US Dollars according to the following chart:

| Certificated entity: ## Number of employees | # US $ |
| --- | --- |
| Up to 20 | 3,800 |
| 21 – 50 | 5,000 |
| 51 – 100 | 7,000 |
| 101 – 250 | 8,000 |
| 251 - 500 | 11,000 |

The ISO 9001:2000 certification will cost the manufacturing company with 50 employees about US $ 5,000.

- Module testing in an accredited Testing Laboratory.

A guideline, based on the information published in December 2001 on the web site of the Arizona State University’s testing laboratory (www.asu.edu/east/pte), is that complete type testing of a PV module will cost about US $ 25,000.

The IECQ approval process utilizes a “Product Quality Assessment Specification” (PQAS) (described in detail in Chapter 5.4). PQAS also includes the concept of structural similarity.  In the case of modules, this means, for example, that only one size has to be tested, an important saving for the manufacturer.

- License fee to be paid to PV GAP US $ 750

#### Total expense for the first year (considering a 1 MWp  / year production):

If an outside consultant is used: US $55,750 or a total of US $ 0.056 per watt

If the ISO 9001:2000 system is set up in-house: US $ 45,750 or a total of    US $ 0.046 per watt

2nd year expenses

- ISO 9001:2000: The quality manufacturing system is in place

No expense

- ISO 9001:2000 annual surveillance

The European co-operation for Accreditation published a guideline of the suggested auditor days based on the number of employees. This is shown in the following chart:

| Certified entity: # Number of employees | Annual surveillance (Auditor Days) | Annual surveillance (Auditor Days) |
| --- | --- | --- |
|  | Total | On-site minimum |
| Less than 5 | 1 | 0.5 |
| 5 - 9 | 1 | 0.5 |
| 10 – 19 | 1 | 1 |
| 20 – 29 | 1.5 | 1 |
| 30 – 59 | 2 | 1 |
| 60 – 100 | 2 | 1.5 |
| 100 – 250 | 2.5 | 2 |
| 250 – 500 | 3 | 2 |
| 500 - 1000 | 4 | 3 |

In Germany, the Annual Surveillance for a company with 50 employees would cost

US $ 1,700

- Module testing in an accredited Testing Laboratory.

According to the PVRS 2, if the manufacturer makes no changes in the module, no re-testing is required.

- License fee to be paid to PV GAP US $ 750

Total expense for year 2: US $ 2,450

#### Total expense for the second year (considering a 1 MWp  / year production):

Will add a negligible ………………………………………… US $ 0.0025 per watt.

BENEFITS:

On the other side of the coin are the benefits the manufacturer will get by installing and operating the IECQ approval system and the PV GAP licensing of the PV Quality Mark/Seal

THE REWARDS OF QUALITY

| Leveling the playing field | - Customers can differentiate between quality products and products of poor or unknown quality. - The same quality standard is used globally. |
| --- | --- |
| Reliability and durability of PV products | - The reputation of PV products is important. - Financing of PV products will become easier. |
| Customer satisfaction | - Market expansion will be a natural result. |
| Participation in tenders | - Tenders require quality products. - Globally uniform quality standards make it possible for tenders to specify the same testing. |
| Opening export markets | - Reciprocity of testing will open export markets. |
| Customer recognition of quality products by the “PV Quality Mark and Seal” | - The “PV Quality Mark and Seal” will help all companies, large or small, differentiate their products from products of inferior quality. |
| Cost reduction by manufacturing improvements | - The reduction of inline rejects and the finished product non-conformity has been reduced by 9 % in most companies that have established a quality system. |
| Cost reduction for system companies buying PV GAP marked products | - Purchasing components from a company that is approved to use the Mark and Seal do not have to retest these products, because they are identified as good quality and meeting all specified requirements. - Significant savings and confidence in the products purchased gives a significant advantage |

The potential cost savings from a well established Quality Management System are a minimum of 9 % of the total cost of the product (see section 6.4.3 of the Manual), therefore if the cost is $ 2.00 per watt the cost savings should be $ 0.18 per watt for a net savings of $ 0.12 per watt after the approval is achieved and the system installed.

Appendix 2:  Example of a Standard Quality Plan

TITLE:

# MANUFACTURING COMPANY

OF PHOTOVOLTAICS PRODUCTS

NUMBER

PAGE  1     OF  3

STANDARD QUALITY PLAN

FOR PHOTOVOLTAIC CELLS/MODULES AND PHOTOVOLTAIC SYSTEMS

- OBJECTIVE

To describe the Quality Control Plan to ensure the quality of photovoltaic Cells, Modules and Systems at all stages of manufacture. Testing, storage and packaging including that of input materials and components fabricated in-house and or bought outside.

- SCOPE

This Quality Plan is a Standard Quality Plan for:

Photovoltaic Cells/Modules and Photovoltaic Systems.

It shall be applicable to the following areas:

- Control of all Incoming Materials/Components- Direct or Indirect

- Control of Critical Manufacturing Processes

- Control of Bought-out Items

- Control of Assembly and Final Testing if Finished Products

Tests include Routine and Type Test

- Control of Handling, Storage and Packaging

- Control of Final Inspection

- Control of Inspection, Measurement and Test Equipment

- Any special requirements of the Customer which require changes in

electrical/ mechanical characteristics and or test specification as additional instructions given against specific PO / Contract.

- CONTENTS

This Standard Quality Plan describes the inspection carried out for all materials, components, semi-finished products at key stages of manufacturing, including final inspection and testing, storage, preservation, and packaging at works and at site, and calibration of gauges, measuring and test equipments.

The various documents referred to in this Standard Quality Plan are internal documents, the details of which are available for perusal at this Division.

APPROVED: Boss on 3/2/2000

REVISIONS  01. Renamed Company

PREPARED:

Helper

ISSUED:

In Charge

DATE:

3/2/2000

# TITLE:

# MANUFACTURING COMPANY

OF PHOTOVOLTAICS PRODUCTS

NUMBER

PAGE  2     OF 3

- IMPLEMENTATION

The implementation of the Standard Quality Plan is mandatory for all Standard / catalogued products as per plan indicated.

STANDARD QUALITY PLAN

FOR PHOTOVOLTAIC CELLS / MODULES AND PHOTOVOLTAIC SYSTEMS

| Sl. No. | Material or component tested / inspected | Characteristics Tested / inspected | Quantity tested/ Inspected | Reference document / acceptance norm | Format or record | Implementing department |
| --- | --- | --- | --- | --- | --- | --- |
| A. A.1 | PHOTOVOLTAIC CELLS & MODULES Incoming materials and Components |  |  |  |  |  |
| a.1.1 a.1.2 | Silicon wafers, Screens,Silver Paste, PV Module Components (Glass, EVA, Tedlar, Aluminum frame, Gasket, Scrim, Terminal Box, Soft Solder, Interconnects, Bus Bars, Silicone Adhesive Gases, Chemicals, Consumable materials | Appearance, Physical Parameters, Dimensional Appearance, Physical Parameters, Dimensional | Sample Sample | BOM No. 1111 BOM No 1112 | Q In Q In | QC & Stock QC & Stock |
| A.2 | Critical Processes & semi-finished products |  |  |  |  |  |
| a.2.1 a.2.2 | Etching, Diffusion, Coin-stack Edging, Screen Printing, Firing. Semi-finished Products Diffused Silicon Wafer | Manufacturing Process Control Manufacturing Process and Test Control | 100 % 100 % | SPC- 5001 to SPC- 5010 SPC- 5011 and STC- 5015 | P.M.I. P.M.I. | Prodn. QC Prodn. QC |
| A.3 | Assembly and Final Testing of Finished Products |  |  |  |  |  |
| a.3.1 a.3.2 | Final Testing of Photovoltaic Cells and Modules Routine Acceptance and Special Tests Final Inspection of Finished Products | Testing Instructions Per Quality Management Work Instructions | 100 % 100 % | STC- 5016 to STC- 5020 QMS-FPT101 to FPT104 | P.M.I. Q.M.I. | Prodn.QC QC SPC |

| Sl. No. | Material or component tested / inspected | Characteristics Tested / inspected | Quantity tested/ Inspected | Reference document / acceptance norm | Format or record |  | Implementing department |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A.4 | Handling and Packaging |  |  |  |  |  |  |
| a.4.1 | Handling, Storage and Packaging Instructions | As per Handling and Packaging Instructions: FPS 5021 to FPS 5027 |  |  |  | P.M.I. | Prodn & QC Final QC |
| A.5 | Inspection, Measurement and Test Equipment |  |  |  |  |  |  |
| a.5.1 | Calibration | Per Procedures of Operating and Test Equipment. Work Instructions |  |  |  | P.M.I. | Prodn & QC |

- PHOTOVOLTAIC SYSTEMS

b.1	Bought-out Materials and Components

| b.1.1 | Mechanical Items: Module Mounting Structures, Junction Boxes, Hardware, Cubicle and Painting | Dimensional, Visual, Electrical Parameters | Sample | BOM No: HS-1-500 To 520 PS No: Pur.HS 1-500 to 520 | Test Plan: HS1-500 | QC-Sys HS |
| --- | --- | --- | --- | --- | --- | --- |
| b.1.2 | Electrical Items: Charge Controller, Inverter, Battery Charger, Luminaries with Electric’s, Batteries, Switch Gear, Cables, Cooling or Exhaust Fans. | Dimensional, Electrical Parameters | Sample | BOM No: HS-1-500 To 520 PS No: Pur.HS 1-520 to 530 | Test Plan: HS1-510 | QC-Sys HS |
| b.1.3 | Electronic Items: Diodes, Transistors, Power Devices, LEDs, Populated PCBs | Dimensional, Visual, Electrical Parameters and solder ability | Sample | BOM No: HS-1-500 To 520 PS No: Pur.HS 1-520 to 530 | Test Plan: HS1-510 | QC-Sys HS |
| b.1.4 | Inspection of Packing for Mechanical, Electrical and Electronic Items, Spares, etc. | Dimensional and Visual | Sample | BOM No: HS-1-500 To 520 PS No: Pur.HS 1-540 | Test Plan: HS1-540 | QC-Sys HS |

APPROVED: Boss on 3/2/2000

PREPARED

Helper

ISSUED

In Charge

NUMBER

DATE

3/2/2000

PAGE  3      OF 3

TITLE:

# MANUFACTURING COMPANY

OF PHOTOVOLTAICS PRODUCTS

Appendix 3:

Publications issued by IEC TC 82

| Publication |  |  |  |
| --- | --- | --- | --- |
| IEC 60891 Ed. 1.0 |  |  |  |
| Procedures for temperature and irradiance corrections to measured I-V characteristics of crystalline silicon photovoltaic devices |  |  |  |
| IEC 60891 Amd.1 Ed. 1.0 |  |  |  |
| Amendment No. 1 |  |  |  |
| IEC 60904-1 Ed. 1.0 |  |  |  |
| Photovoltaic devices. Part 1: Measurement of photovoltaic current-voltage characteristics |  |  |  |
| IEC 60904-2 Ed. 1.0 |  |  |  |
| Photovoltaic devices. Part 2: Requirements for reference solar cells |  |  |  |
| IEC 60904-2 Amd.1 Ed. 1.0 |  |  |  |
| Amendment 1 |  |  |  |
| IEC 60904-3 Ed. 1.0 |  |  |  |
| Photovoltaic devices. Part 3: Measurement principles for terrestrial photovoltaic (PV) solar devices with reference spectral irradiance data |  |  |  |
| IEC 60904-5 Ed. 1.0 |  |  |  |
| Photovoltaic devices - Part 5: Determination of the equivalent cell temperature (ECT) of photovoltaic (PV) devices by the open-circuit voltage method |  |  |  |
| IEC 60904-6 Ed. 1.0 |  |  |  |
| Photovoltaic devices - Part 6: Requirements for reference solar modules |  |  |  |
| IEC 60904-6 Amd.1 Ed. 1.0 |  |  |  |
| Amendment 1 |  |  |  |
| IEC 60904-7 Ed. 2.0 |  |  |  |
| Photovoltaic devices - Part 7: Computation of spectral mismatch error introduced in the testing of a photovoltaic device |  |  |  |
| IEC 60904-8 Ed. 2.0 |  |  |  |
| Photovoltaic devices - Part 8: Measurement of spectral response of a photovoltaic (PV) device |  |  |  |
| IEC 60904-9 Ed. 1.0 |  |  |  |
| Photovoltaic devices - Part 9: Solar simulator performance requirements |  |  |  |
| IEC 60904-10 Ed. 1.0 |  |  |  |
| Photovoltaic devices - Part 10: Methods of linearity measurement |  |  |  |
| IEC 61173 Ed. 1.0 |  |  |  |
| Overvoltage protection for photovoltaic (PV) power generating systems - Guide |  |  |  |
| IEC 61194 Ed. 1.0 |  |  |  |
| Characteristic parameters of stand-alone photovoltaic (PV) systems |  |  |  |
| IEC 61215 Ed. 1.0 |  |  |  |
| Crystalline silicon terrestrial photovoltaic (PV) modules - Design qualification and type approval |  |  |  |
| IEC 61277 Ed. 1.0 |  |  |  |
| Terrestrial photovoltaic (PV) power generating systems - General and guide |  |  |  |
| IEC 61345 Ed. 1.0 |  |  |  |
| UV test for photovoltaic (PV) modules |  |  |  |
| IEC 61646 Ed. 1.0 |  |  |  |
| Thin-film terrestrial photovoltaic (PV) modules - Design qualification and type approval |  |  |  |
| IEC 61683 Ed. 1.0 |  |  |  |
| Photovoltaic systems - Power conditioners - Procedure for measuring efficiency |  |  |  |
| IEC 61701 Ed. 1.0 |  |  |  |
| Salt mist corrosion testing of photovoltaic (PV) modules |  |  |  |
| IEC 61702 Ed. 1.0 |  |  |  |
| Rating of direct coupled photovoltaic (PV) pumping systems |  |  |  |
| IEC 61721 Ed. 1.0 |  |  |  |
| Susceptibility of a photovoltaic (PV) module to accidental impact damage (resistance to impact test) |  |  |  |
| IEC 61724 Ed. 1.0 |  |  |  |
| Photovoltaic system performance monitoring - Guidelines for measurement, data exchange and analysis |  |  |  |
| IEC 61725 Ed. 1.0 |  |  |  |
| Analytical expression for daily solar profiles |  |  |  |
| IEC 61727 Ed. 1.0 |  |  |  |
| Photovoltaic (PV) systems - Characteristics of the utility interface |  |  |  |
| IEC 61829 Ed. 1.0 |  |  |  |
| Crystalline silicon photovoltaic (PV) array - On-site measurement of I-V characteristics |  |  |  |
| IEC 61836 TR2 Ed. 1.0 |  |  |  |
| Solar photovoltaic energy systems - Terms and symbols |  |  |  |
| IEC/PAS 62111 Ed. 1.0 |  |  |  |
| Specifications for the use of renewable energies in rural decentralised electrification |  |  |  |

Information from IEC Web Page:   www.iec.ch		Web page generated: 4 January 2002

| IEC TC 82 |  |  | # Work in Progress |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Project Reference |  |  | Project Leader |  |  |  |
| IEC 61215 Ed. 2.0 |  |  | Project Leader : J.h. Wohlgemuth |  |  |  |
| Crystalline silicon terrestrial photovoltaic (PV) modules - Design qualification and type approval |  |  |  |  |  |  |
| IEC 61727 Ed. 2.0 |  |  | Project Leader : S. Chalmers |  |  |  |
| Photovoltaic (PV) systems - Characteristics of the utility interface |  |  |  |  |  |  |
| IEC 61730-1 Ed. 1.0 |  |  | Project Leader : S. Jochums |  |  |  |
| Photovoltaic module safety qualification - Part 1: Requirements for construction |  |  |  |  |  |  |
| IEC 61730-2 Ed. 1.0 |  |  | Project Leader : W. Wiesner |  |  |  |
| Photovoltaic module safety qualification - Part 2: Requirements for testing |  |  |  |  |  |  |
| IEC 61836 TR Ed. 2.0 |  |  | Project Leader : G. Howell |  |  |  |
| Solar photovoltaic energy systems - Terms and symbols |  |  |  |  |  |  |
| IEC 61853 Ed. 1.0 |  |  | Project Leader : J. Wohlgemuth |  |  |  |
| Performance testing and energy rating of terrestrial photovoltaic (PV) modules |  |  |  |  |  |  |
| IEC 62078 Ed. 1.0 |  |  | Project Leader : W. Wiesner |  |  |  |
| Certification and accreditation program for photovoltaic (PV) components and systems - Guidelines for a total quality system |  |  |  |  |  |  |
| IEC 62093 Ed. 1.0 |  |  | Project Leader : F. Wouters |  |  |  |
| Balance-of-system components for photovoltaic systems - Design qualification and type approval |  |  |  |  |  |  |
| IEC 62108 Ed. 1.0 |  |  | Project Leader : R. McConnell |  |  |  |
| Concentrator photovoltaic (PV) receivers and modules - Design qualification and type approval |  |  |  |  |  |  |
| IEC 62109 Ed. 1.0 |  |  | Project Leader : Mr. T. Zgonena |  |  |  |
| Electrical safety of static inverters and charge controllers for use in photovoltaic (PV) power systems |  |  |  |  |  |  |
| IEC 62116 Ed. 1.0 |  |  | Project Leader : Mr. I. Tsuda |  |  |  |
| Testing procedure - Islanding prevention measures for power conditioners used in grid connected photovoltaic (PV) power generation systems |  |  |  |  |  |  |
| IEC 62124 Ed. 1.0 |  |  | Project Leader : Frank Wouters |  |  |  |
| Photovoltaic (PV) stand-alone systems - Design qualification and type approval |  |  |  |  |  |  |
| IEC 62145 Ed. 1.0 |  |  | Project Leader : J. Wohlgemuth |  |  |  |
| Crystalline silicon PV modules - Blank detail specification |  |  |  |  |  |  |
| IEC 62234 Ed. 1.0 |  |  | Project Leader : G. Howell |  |  |  |
| Safety guidelines for grid connected photovoltaic (PV) systems mounted on buildings |  |  |  |  |  |  |
| IEC 62253 Ed. 1.0 |  |  | Project Leader : P. Kremer |  |  |  |
| Direct coupled photovoltaic pumping systems - Design qualification and type approval |  |  |  |  |  |  |
| IEC 62257 Ed. 1.0 |  |  | Project Leader : A. Schmitt |  |  |  |
| Specifications for the use of renewable energies in rural decentralised electrification |  |  |  |  |  |  |
| IEC 62257-1 Ed. 1.0 |  |  | Project Leader : A. Schmitt |  |  |  |
| Recommendations for small renewable energy and hybrid systems for rural electrification - Part 1: General introduction to rural electrification |  |  |  |  |  |  |
| IEC 62257-2 Ed. 1.0 |  |  | Project Leader : A. Schmitt |  |  |  |
| Recommendations for small renewable energy and hybrid systems for rural electrification - Part 2: From requirements to a range of electrification systems |  |  |  |  |  |  |
| PNW 82-263 Ed. 1.0 |  |  | Project Leader : D.G. Infield |  |  |  |
| Maximum Power Point Tracking |  |  |  |  |  |  |
| PWI 82-1 Ed. 1.0 |  |  |  |  |  |  |
| Photovoltaic electricity storage systems |  |  |  |  |  |  |
| Information from IEC Web page: www.iec.ch Web page generated: 4 January 2002 |  |  |  |  |  |  |

Appendix 4:

IEC Standards of Possible Use for PV

# BATTERIES

IEC 61044 Ed. 1.0 	Opportunity-charging of lead-acid traction batteries

IEC 61056-1 Ed. 1.0 	Portable lead-acid cells and batteries (Valve regulated types) - Part 1: General requirements, functional characteristics - Methods of test

IEC 61056-2 Ed. 1.0	Portable lead-acid cells and batteries (Valve regulated types) - Part 2: Dimensions, terminals and marking

IEC 61056-3 Ed. 1.0	Portable lead-acid cells and batteries (Valve-regulated types) - Part 3: Safety recommendations for use in electric appliances

IEC 60254-1 Ed. 3.0 	Lead-acid traction batteries - Part 1: General requirements and methods of test

IEC 60254-2 Ed. 3.0 	Lead-acid traction batteries - Part 2: Dimensions of cells and terminals and marking of polarity on cells

IEC 60896-1 (1987-01)	Stationary lead-acid batteries - General requirements and methods of test. Part 1: Vented types.

IEC 60896-1 Amd.1 Ed. 1.0	Amendment No. 1

IEC 60896-1 Amd.2 Ed. 1.0	Amendment No. 2

IEC 60896-1 Ed. 1.0 	Stationary lead-acid batteries - General requirements and methods of test. Part 1: Vented types

IEC 60896-1-am1 (1988-01)	Amendment No. 1

IEC 60896-1-am2 (1990-12)	Amendment No. 2

IEC 60896-2 (1995-11)	Stationary lead-acid batteries - General requirements and test methods - Part 2: Valve regulated types.

IEC 60896-2 Ed. 1.0 	Stationary lead-acid batteries - General requirements and test methods - Part 2: Valve regulated types

IEC 60095-1 Ed. 5.0 	Lead-acid starter batteries. Part 1: General requirements and methods of test

IEC 60095-2 Amd.1 Ed. 3.0 	Amendment No. 1

IEC 60095-2 Amd.2 Ed. 3.0 	Amendment No. 2

IEC 60095-2 Ed. 3.0 	Lead-acid starter batteries. Part 2: Dimensions of batteries and dimensions and marking of terminals

IEC 60095-4 Amd.1 Ed. 1.0	Amendment No. 1

IEC 60095-4 Ed. 1.0 	Lead-acid starter batteries. Part 4: Dimensions of batteries for heavy trucks

IEC 60952-1 (1988-07)	Aircraft batteries. Part 1: General test requirements and performance levels.

IEC 60952-1 Ed. 1.0 	Aircraft batteries. Part 1: General test requirements and performance levels

IEC 60952-2 (1991-03)	Aircraft batteries - Part 2: Design and construction requirements.

IEC 60952-2 Ed. 1.0 	Aircraft batteries - Part 2: Design and construction requirements

IEC 60952-3 (1993-07)	Aircraft batteries - Part 3: External electrical connectors.

IEC 60952-3 Ed. 1.0

Aircraft batteries - Part 3: External electrical connectors

# CABLING / WIRING

IEC 60610 (1978-01)	Principal aspects of functional evaluation of electrical insulation systems: Aging mechanisms and diagnostic procedures.

IEC 60611 (1978-01)	Guide for the preparation of test procedures for evaluating the thermal endurance of electrical insulation systems.

IEC 60614-1 (1994-03)	Conduits for electrical installations - Specification - Part 1: General requirements.

IEC 60614-1-am1 (1995-09)	Amendment No. 1

IEC 60614-2-1 (1982-01)	Specification for conduits for electrical installations. Part 2: Particular specifications for conduits. Section One: Metal conduits.

IEC 60614-2-1-am1 (1993-10)	Amendment No. 1

IEC 60614-2-5 (1992-11)	Specifications for conduits for electrical installations - Part 2: Particular specifications for conduits - Section 5: Flexible conduits.

IEC 60811-1-1 (1993-10)	Common test methods for insulating and sheathing materials of electric cables - Part 1: Methods for general application -Section 1: Measurement of thickness and overall dimensions - Tests for determining the mechanical properties.

IEC 60811-1-2 (1985-07)	Common test methods for insulating and sheathing materials of electric cables - Part 1: Methods for general application - Section Two: Thermal aging methods.

IEC 60811-1-2-am1 (1989-11)	Amendment No. 1

IEC 60811-1-4 (1985-07)	Common test methods for insulating and sheathing materials of electric cables - Part 1: Methods for general application - Section Four: Test at low temperature.

IEC 60811-1-4-am1 (1993-08)	Amendment No. 1

EC 60811-4-1 (1985-10)	Common test methods for insulating and sheathing materials of electric cables - Part 4: Methods specific to polyethylene and polypropylene compounds - Section One - Resistance to environmental stress cracking - Wrapping test after thermal aging in air - M

IEC 60811-4-1-am2 (1993-08)	Amendment No. 2

IEC 60812 (1985-07)	Analysis techniques for system reliability - Procedure for failure mode and effects analysis (FMEA).

IEC 60998-1 (1990-05)	Connecting devices for low voltage circuits for household and similar purposes. Part 1: General requirements.

IEC 60998-2-1 (1990-05)	Connecting devices for low voltage circuits for household and similar purposes. Part 2-1: Particular requirements for connecting devices as separate entities with screw-type clamping units.

IEC 60998-2-2 (1991-11)	Connecting devices for low-voltage circuits for household and similar purposes - Part 2-2: Particular requirements for connecting devices as separate entities with screwless-type clamping units.

IEC 60998-2-3 (1991-11)	Connecting devices for low-voltage circuits for household and similar purposes - Part 2-3: Particular requirements for connecting devices as separate entities with insulation piercing clamping units.

IEC 60998-2-4 (1993-05)	Connecting devices for low-voltage circuits for household and similar purposes - Part 2-4: Particular requirements for twist-on connecting devices.

IEC 60998-2-5 (1996-01)	Connecting devices for low-voltage circuits for household and similar purposes - Part 2-5: Particular requirements for connecting boxes (junction and/or tapping) for terminals or connecting devices.

# CHARGE CONTROLLER

IEC 60439-1 (1992-12)	Low-voltage switchgear and controlgear assemblies - Part 1: Type-tested and partially type-tested assemblies.

IEC 60439-1-am1 (1995-11)	Amendment No. 1

IEC 60439-1-am2 (1996-12)	Amendment No. 2

IEC 60439-2 (1987-11)	Low-voltage switchgear and controlgear assemblies. Part 2: Particular requirements for busbar trunking systems (busways).

IEC 60439-2-am1 (1991-09)	Amendment No. 1

IEC 60439-3 (1990-12)	Low-voltage switchgear and controlgear assemblies. Part 3: Particular requirements for low-voltage switchgear and controlgear assemblies intended to be installed in places where unskilled persons have access for their use - Distribution boards.

IEC 60439-3-am1 (1993-10)	Amendment No. 1

IEC 60439-4 (1990-12)	Low-voltage switchgear and controlgear assemblies. Part 4: Particular requirements for assemblies for construction sites (ACS).

IEC 60439-4-am1 (1995-12)	Amendment No. 1

IEC 60439-5 (1996-03)	Low-voltage switchgear and controlgear assemblies - Part 5: Particular requirements for assemblies intended to be installed outdoors in public places - Cable distribution cabinets (CDCs) for power distribution in networks.

IEC 60529 (1989-11)	Degrees of protection provided by enclosures (IP Code). Applies to the classification of degrees of protection provided by enclosures for electrical equipment with a rated voltage not exceeding 72,5 kV. Has the status of a basic safety publication.

#### DC SAFETY

IEC 61204 (1993-02)	Low-voltage power supply devices, d.c. output - Performance characteristics and safety requirements.

# INVERTER

IEC 60146-1-1 (1991-04)	General requirements and line commutated convertors - Part 1-1: Specifications of basic requirements.

IEC 60146-1-1-am1 (1996-07)	Amendment No. 1 to IEC 146-1-1.

IEC 60146-1-2 (1991-04)	General requirements and line commutated convertors - Part 1-2: Application guide.

IEC 60146-1-3 (1991-04)	General requirements and line commutated convertors - Part 1-3: Transformers and reactors.

### LAMPS

IEC 60400 (1996-06)	Lampholders for tubular fluorescent lamps and starterholders.

IEC 60400-am1 (1997-04)	Amendment No. 1

IEC 60598-1 (1996-12)	Luminaires - Part 1: General requirements and tests

IEC 60598-2-1 (1979-01)	Luminaires. Part 2: Particular requirements. Section One: Fixed general purpose luminaires.

IEC 60598-2-1-am1 (1987-01)	Amendment No. 1

IEC 60598-2-23 (1996-04)	Luminaires - Part 2: Particular requirements - Section 23: Extra low voltage lighting systems for filament lamps.

IEC 60598-2-3 (1993-04)	Luminaires - Part 2: Particular requirements - Section 3: Luminaires for road and street lighting.

IEC 60598-2-3-am1 (1997-04)	Amendment No. 1

IEC 60598-2-4 (1997-04)	Luminaires - Part 2: Particular requirements - Section 4: Portable general purpose luminaires

IEC 60598-2-5 (1979-01)	Luminaires. Part 2: Particular requirements. Section Five: Floodlights.

IEC 60598-2-5-am1 (1987-01)	Amendment No. 1

IEC 60598-2-5-am2 (1993-04)	Amendment No. 2

IEC 60598-2-7 (1982-01)	Luminaires. Part 2: Particular requirements. Section Seven: Portable luminaires for garden use.

IEC 60598-2-7-am1 (1987-01)	Amendment No. 1

IEC 60598-2-7-am2 (1994-08)	Amendment No. 2

IEC 60598-2-8 (1996-07)	Luminaires- Part 2: Particular requirements - Section 8: Handlamps.

IEC 60081 (1984-01)	Tubular fluorescent lamps for general lighting service.

IEC 60081-am1 (1987-01)	Amendment No. 1

IEC 60081-am2 (1988-01)	Amendment No. 2

IEC 60081-am3 (1992-07)	Amendment No. 3

IEC 60081-am4 (1993-09)	Amendment No. 4

IEC 60081-am5 (1994-12)	Amendment No. 5

IEC 60810 (1993-10)	Lamps for road vehicles - Performance requirements.

IEC 60810-am1 (1994-07)	Amendment No. 1

IEC 60924 (1990-07)	D.C. supplied electronic ballasts for tubular fluorescent lamps - General and safety requirements.

IEC 60924-am1 (1993-03)	Amendment No. 1

IEC 60925 (1989-06)	D.C. supplied electronic ballasts for tubular fluorescent lamps - Performance requirements.

IEC 60925-am1 (1996-05)	Amendment No.1

IEC 60095-1 Amd.1 Ed. 5.0 	Amendment No. 1

IEC 60095-1 Amd.2 Ed. 5.0 	Amendment No. 2

IEC 61162-1 (1995-`11)	Maritime navigation and radio communication equipment and systems - Digital interfaces - Part 1: Single talker and multiple listeners. Contains the requirements for data communication between maritime electronic instruments, navigation and radio communication.

## Appendix 5:

## PV GAP Recommended Standards

October 2001

| Ref. | Title | Remarks | Availability |
| --- | --- | --- | --- |
| PVRS 1 | Photovoltaic Stand-Alone Systems – Design Qualification and Type Approval First edition 1997-10 44 pages | Covers components (except lamps) and indoor testing of systems. Based on JRC-Ispra testing experience. | Available: USD 20 incl. mailing |
| PVRS 2 | Crystalline silicon terrestrial photovoltaic (PV) modules Blank detail specification First edition 1998-12 14 pages | Makes IEC 61215 certifiable. | Available: USD 20 incl. mailing |
| PVRS 3 | Thin-film terrestrial photovoltaic (PV) modules Blank detail specification First edition 1999-05 15 pages | Makes IEC 61646 certifiable. | Available: USD 20 incl. mailing |
| PVRS 4 | Photovoltaic (PV) stand-alone systems, with a system voltage below 50V. Blank detail specification Third draft 2000-10 11 pages | Draft which is intended to make IEC 82/259/CD, June 2001, (with any further amendments, and as published by the IEC in due course), certifiable. One of the documents on which IEC 82/259/CD is based is NREL/TP-520-27031 (see PVRS 9). | Available as draft, free of charge |
| PVRS 5 | (reserved for future use) | Reserved for a future specification for SHS batteries/accumulators. | Not available at present |
| PVRS 6 | Charge controllers for photovoltaic (PV) stand-alone systems with a nominal system voltage below 50 V. Blank detail specification First edition 2000-06 12 pages | Makes GEF/WB China Renewable Energy Development (RED) Project Specification certifiable (see PVRS 6A). | Available: USD 20 incl. mailing |
| PVRS 6A | Annex - Specification and testing procedure First edition 2000-06 17 pages | Reproduced by kind permission of the Project Management Office (PMO), World Bank/GEF Assisted China Renewable Energy Development Project, State Economic and Trade Commission, Beijing, People's Republic of China - May 1999, Beijing | Free of charge |
| PVRS 7 | (reserved for future use) | Reserved for a future specification for SHS lighting system. | Not available at present |
| PVRS 8 | Inverters for photovoltaic (PV) stand-alone systems Blank detail specification First edition 2000-06 13 pages | Makes GEF/WB China Renewable Energy Development Project Specification certifiable (see PVRS 8A). | Available: USD 20 incl. mailing |
| PVRS 8A | Annex - Specification and testing procedure First edition 2000-06 17 pages | Reproduced by kind permission of the Project Management Office (PMO), World Bank/GEF Assisted China Renewable Energy Development Project, State Economic and Trade Commission, Beijing, People's Republic of China - May 1999, Beijing | Free of charge |
| PVRS 9 | Procedures for Determining the Performance of Stand-Alone Photovoltaic Systems 2000-01 34 pages | This is document NREL/TP-520-27031, September 1999, which was prepared by authors from NREL, Sandia National Laboratories, Florida Solar Energy Center, Southwest Technology Development Institute, and Photovoltaics for Utility Scale Applications. The procedures cover complete outdoor system testing and provide a common approach for evaluating whether a given PV system is suitable to perform the function for which it was designed and manufactured to accomplish, and whether it will provide adequate power to run the load. This is one of the documents on which the IEC TC 82 Committee Draft referred to under PVRS 4 (see above) is based. | Free of charge |

© PV GAP 2000

With Permission from PV GAP Web site: www.pvgap.org

Appendix 6:

Example of a Blank Product Quality Assessment Specification

CRYSTALLINE SILICON TERRESTRIAL PHOTOVOLTAIC (PV) MODULES

Qualification Approval under the IEC

Quality Assessment System for Electronic Components (IECQ)

1	General

- Scope

This PV GAP Recommended Specification is a blank detail specification applicable to crystalline silicon terrestrial photovoltaic (PV) modules of assessed quality. PV modules according to this specification are provided for operation in Solar Home System applications. However, as PV modules, they are also suitable for other applications, as described in the manufacturer’s data sheets.

This specification references IEC 61215 requirements and methods of test to be used in detail specifications derived from this specification, and lists the technical criteria that are necessary and sufficient to assess the quality of the PV modules in accordance with the IECQ Qualification Approval procedure described in QC 001002-3 clause 3.

- Normative references

The following normative documents contain provisions, which, through reference in this text, constitute provisions of this PV GAP Recommended Specification.

IEC 61215 : 1993, Crystalline silicon terrestrial photovoltaic (PV) modules — Design qualification and type approval

IEC 61345 : 1998, UV test for photovoltaic (PV) modules

QC 001002-2: 1998, Rules of Procedure of the IEC Quality Assessment System for Electronic Components (IECQ) — Part 2, Documentation

QC 001002-3: 1998, Rules of Procedure of the IEC Quality Assessment System for Electronic Components (IECQ) — Part 3, Approval procedures

QC 001005, Register of Firms, Products and Services approved under the IECQ System, including ISO 9000

1.3		Front page of detail specification

The layout of the front page of detail specification shall be as follows (see key on following page)

| (1) | PQCxxxAAxxxx (2) Edition: 199x/200x Page 1 of x |
| --- | --- |
| Electronic components of assessed (3) quality in accordance with : IEC 61215: 1993 | (4) |
| Detail specification for crystalline silicon terrestrial PV modules of assessed quality Type : (5) Construction: (6) |  |
| Outline drawing (7) Dimensions in millimeters | Applications : (8) |
| Electrical characteristics (see 2.1 and 2.2) (9) |  |
| Mechanical characteristics (see 2.3) (10) |  |
| Information about manufacturers who have components approved according to this detail specification is available in the current QC 001005. |  |

Key to front page:

The numbers between brackets on the front page correspond to the following indications, which should be given.

Identification of the detail specification

- The name of the National Standards Organization under whose authority the detail specification is published and, if applicable, the organization from whom the detail specification is available.

- The IECQ logo and the number allotted to the completed detail specification by the National Standards Organization or the IECQ National Authorized Institution (PQCxxx is the IECQ provisional specification reference allocated earlier by the IECQ Secretariat to the blank detail specification. AA is the country identifier, e.g. FR for France, DE for Germany, IN for India, US for United States of America).

- The number and the year of availability of the IEC standard concerning test and measurement procedures for the PV modules and/or sectional specification; also national reference, if different.

- If different from the IECQ number, the national number of the detail specification, date of issue and any further information required by the national system, together with any amendment numbers.

##### Identification of the PV module

- Type: for example, glass front, or non-glass

- Construction: frame material, junction box, glass type, sealing and other typical construction details.

- An outline drawing with main dimensions, which are of importance for interchangeability. Alternatively, this drawing may be given in an annex to the detail specification, but (7) should always contain an illustration of the general outer appearance of the PV module.

- Typical field of applications.

- Electrical characteristics.

- Mechanical characteristics.

2	Characteristic values of the PV module

- Electrical characteristics

|  |  | Limits 5% |  |  |
| --- | --- | --- | --- | --- |
| Maximum power (Pmax) W Maximum power voltage (Vpmax) V Maximum power current (Ipmax) A Short-circuit current (ISC) A Open-circuit voltage (VOC) V Current-temperature coefficient A/°C Voltage-temperature coefficient V/°C Power-temperature coefficient %/°C Nominal operating cell temperature (NOCT) °C Maximum system operating voltage (Vsmax) V (see 10.3 of IEC 61215) |  |  |  |  |
| NOTE These data represent the performance of typical modules as measured at their output terminals, and do not include the effect of such additional equipment as diodes and cabling. The data are based on measurements made at standard test conditions (STC) which are: - Illumination of 1kW/m2 at air mass solar reference spectrum of AM 1,5 (except for the temperature coefficient characteristics, where IEC 60904-1, referenced in 10.4 of IEC 61215, states that irradiance shall be at least 800W/m2) - Cell temperature of 25°C or as otherwise specified (on curves). |  |  |  |  |

- Current-voltage characteristics

| Output current (A) |  | Curves for temperature 25°C (other temperatures may be mentioned, in addition) |  |
| --- | --- | --- | --- |
|  |  | Output voltage (V) |  |

Figure 1 — Current-voltage characteristics

- Mechanical characteristics

Output cable, connector, or terminals

Mass (kg)

- Additional information (not for inspection purposes)

For example

- optional protective backplate,

- mounting hardware kits.

3	Qualification Approval Procedure

- General

The Qualification Approval procedure shall be in accordance with QC 001002-3, clause 3, with fixed sampling (3.1.4 a) of QC 001002-3).

NOTE 1 Qualification Approval is preceded by organization approval of the manufacturer (QC 001002-3, clause 2). Organization approval covers all of the quality system requirements, the quality system being described in a Quality Manual and complying with ISO 9001:2000. Therefore details such as the address of the place of manufacture do not belong to the detail specification, which is a publicly available specification for anyone to use.

NOTE 2 Other types of IECQ product/process approval, namely Capability Approval, Technology Approval (including Manufacturing Line Approval) and Specialist Subcontractor Approval are more concerned with the process aspects. Therefore, in this blank detail specification for Qualification Approval, such aspects are not covered.

- Primary stage of manufacture

Formation of the contact metallization on the solar cell. The manufacturer of the solar cells, on whom the overall quality and reliability of the PV module is strongly dependent, shall have a quality system that meets the requirements of ISO 9002 or ISO 9001 or hold IECQ manufacturer’s approval (see QC 001002-3 clause 2) for the manufacturing site.

- Subcontracting

Subcontracting of the primary and subsequent stages of manufacture is permitted, under the conditions laid down in QC 001002-3 clauses 2 and 3.

3.4		Technical requirements

Sampling, testing, pass criteria and classification of major visual defects shall be in accordance with IEC 61215.

NOTE	This blank detail specification cannot add requirements that do not exist in IEC 61215. This is because IEC 61215 was prepared by an IEC technical committee (IEC TC 82) in which all IEC National Committees may participate.

Reporting, and issuing of the Qualification Approval certificate, shall be in accordance with QC 001002-3 clause 3.

- Increased severity

Detail specifications derived from this blank detail specification may make the severities of test, the end-of-test requirements, or the sampling levels, more severe. These severities, or requirements, can never be made less stringent.

- Preparation of detail specifications

An individual manufacturer may prepare a detail specification from this blank detail specification and submit it to the responsible national organization for verification of compliance to the IECQ rules and the allocation of a number. After allocation of the number, the manufacturer may commence Qualification Approval testing under the surveillance of the IECQ Supervising Inspectorate (SI). After completion of the testing, the manufacturer or the responsible organization shall publish the detail specification.

A group of manufacturers may act together to produce a common detail specification with one of them accepting the responsibility for the submission for verification and the allocation of a number, the procedure being the same as above.

The above actions may be in co-operation with one or more customers.

Detail specifications may also be prepared by a responsible organization or by an IEC technical committee.

- Product identification and traceability

The minimum period referred to in 2.3.2 of QC 001002-3, italic subclause heading 4.8, for maintenance of records, shall be five years.

4	Quality conformance inspection

Qualify conformance inspection comprises the tests stated in table 1:

- group A: lot-by-lot (100% inspection) tests;

- group C: periodic tests.

All tests of table 1 are mandatory. Where a subgroup contains cumulative tests, the order of the tests is mandatory. Specimens subjected to tests denoted as destructive (D) shall not be released for delivery.

Table 1 — Test schedule for quality conformance inspection

| Subclause number and Test of IEC 61215 | Destructive or non-destructive | Conditions of test | Performance requirements |
| --- | --- | --- | --- |
| GROUP A INSPECTION (100%) |  |  |  |
| Subgroup A1 | ND |  |  |
| - Visual inspection |  |  | As in 10.1 of IEC 61215 |
| 4 Marking |  | Check that marking is present | Clear and indelible. |
| — Dimensions (gauging) |  |  | See the outline drawing in item (7) of the cover page of this specification and any related tables. |
| Subgroup A2 | ND |  |  |
| - Performance at STC |  |  | See 2.1 and the manufacturer’s data sheet with the calibration reference |
| 10.3 Insulation test |  |  | As in 10.3 of IEC 61215 |
| GROUP B INSPECTION covering additional important characteristics |  |  |  |
| none |  |  |  |

| Subclause number and Test (see Note 1) of IEC 61215 | D or ND | Conditions of test | Sample size and acceptance criterion (see Note 1) |  |  | Performance requirements |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | p | N | c |  |
| GROUP C INSPECTION (periodic) |  |  |  |  |  |  |
| Subgroup C1 |  |  | 24 | 1 | 0 | or change in product |
| - Measurement of temperature coefficients | ND |  |  |  |  | See 2.1 |
| 10.5 Measurement of NOCT | ND |  |  |  |  | See 2.1 |
| 10.6 Performance at NOCT | ND |  |  |  |  | See 2.1, first five items |
| 10.7 Performance at low irradiance | ND |  |  |  |  | See manufacturer’s data sheet |
| 10.8 Outdoor exposure test | D | After, repeat tests 10.1, 10.02, 10.3 of IEC 61215 |  |  |  |  |
| 10.9 Hot-spot endurance | D |  |  |  |  |  |
| GROUP C INSPECTION (periodic) |  |  |  |  |  |  |
| Subgroup C2 | D |  | 12 | 2 | 0 |  |
| 10.10 UV test |  | IEC 61345 |  |  |  | See clause 7 of IEC 61345. |
| - Thermal cycling 10.12 Humidity freeze |  | 50 cycles After, repeat tests 10.1, 10.2, 10.3 of IEC 61215 |  |  |  | See 10.1, 10.2, 10.3 of IEC 61215 See 10.1, 10.2, 10.3 of IEC 61215 |
| - Robustness of terminations - Twist |  |  |  | 1 1 | 0 0 | See 10.14.5 of IEC 61215 See 10.15.4 of IEC 61215 |
| Subgroup C3 | D |  | 12 | 2 | 0 |  |
| - Thermal cycling |  | 200 cycles |  |  |  | See 10.11.5 of IEC 61215 |
| Subclause number and Test (see Note 1) of IEC 61215 | D or ND | Conditions of test | Sample size and acceptance criterion (see Note 1) |  |  | Performance requirements |
|  |  |  | p | n | c |  |
| Subgroup C4 | D |  | 12 | 2 | 0 |  |
| - Damp heat 10.16 Mechanical load |  | 1 000h |  | 1 | 0 | See 10.13.4 See 10.16.4 |
| 10.17 Hail |  |  |  | 1 | 0 | See 10.17.5 |
| NOTE 1 - In this table: | p n c | = periodicity (in months) = sample size = acceptance criterion (permitted number or non-conforming items) |  |  |  |  |

5	Modifications likely to affect Qualification Approval

For the modifications listed below, the Qualification Approval tests in IEC 61215, shall be repeated as indicated :

a)	Modification to cell technology

- For modifications such as

- metallization materials and/or process,

- anti-reflective coating,

- diffusion process,

- order of cell process, and

- change of manufacturing site of the solar cells

Repeat

- thermal cycling, 200 cycles (10.11),

- humidity freeze (10.12),

- damp heat (10.13),

- outdoor exposure (10.8), and

- hot spot endurance (10.9).

b)	Modification to encapsulation system

- For modifications such as

- different materials,

- different additives,

- different primer or method of priming, and

- modification of encapsulation process (that is, different time, temperature, pressure, etc.)

Repeat

- thermal cycling, 200 cycles (10.11),

- UV (10.10, IEC 61345)/ thermal cycling, 50 cycles (10.11) / humidity freeze (10.12) sequence,

- insulation test (10.3), and

- outdoor exposure (10.8).

c)	Modification to superstrate

- For modifications such as

- different material,

- different thickness,

- different additives, and

- different preparation process,

Repeat

- UV (10.10, IEC 61345)/  thermal cycling, 50 cycles (10.11) / humidity freeze (10.12) sequence,

- twist test (10.15),

- mechanical load test (10.16),

- insulation test (10.3),

- hail test (10.17), and

- damp heat (10.13) (if non-glass)

d)	Modification to backsheet/substrate

- For modifications such as

- different thickness,

- different additives, and

- different preparation process,

Repeat

- UV (10.10, IEC 61345)/  thermal cycling, 50 cycles (10.11) / humidity freeze (10.12) sequence,

- robustness of terminations (10.14), and

- damp heat (10.13) (if non-glass),

If there is a change from superstrate to substrate design or from substrate to superstrate design, the entire qualification test sequence in IEC 61215 shall be conducted.

e)	Modification to frame and/or mounting structure

- For modifications such as

- cross section of frame

- different framing material, and

- elimination of frame altogether,

Repeat

- mechanical load test (10.16),

- outdoor exposure (10.18),

- UV (10.10, IEC 61345)/  thermal cycling, 50 cycles (10.11) / humidity freeze (10.12) sequence, if plastic material is used, and

- hail test (10.17), unless tempered glass is used as superstrate.

f)	Modification to junction box/electrical termination

For modifications such as

- different material,

- different design, and

- different method of attachment

repeat

- robustness of terminations (10.14),

- thermal cycling, 200 cycles (10.11), and

- insulation test (10.3).

g)	Lower or higher efficiency cells in identical package and identical cell process

Repeat

- hot-spot endurance (10.9),

- thermal cycle, 200 cycles (10.11), and

- damp heat (10.13).

6	Modifications that do not require re-testing

Provided that all structural components, materials used and processes (including cell process) remain the same, the following modifications shall not require re-testing:

- fewer cells in module;

- smaller cells in module, as long as each cell has the same number of interconnects and equivalent numbers of solder bonds per unit area.

7	Marking

Marking of the PV module shall be in accordance with clause 4 of IEC 61215. The detail specification may also require that, in addition, the following be marked:

- maximum power (W);

- maximum power current (A);

- open-circuit voltage (V);

- short-circuit current (A);

- NOCT (°C).

Marking of the package may also be required, for example

- manufacturer’s name, logo or trade mark;

- model/type number;

- serial number.

8	Documentation

For each delivery a manufacturer’s or distributor’s Declaration of Conformity according to QC 010002-2 shall be included. The name and the site of the manufacturer of the solar cells shall be stated on the Declaration of Conformity.

9	Annexes

Annexes may be included if necessary, to show more details on PV module dimensions, mounting, terminal dimensions, etc.

Appendix 7:

###### IECQ Supervising Inspectorates (SIs)

| BSI Product Services Mr. LA Pillinger Maylands Avenue GB HEMEL HEMPSTEAD HP2 4SQ United Kingdom | TP + 44 1442 278 676 TF + 44 1442 278 621 Len.Pillinger@bsi-global.com |
| --- | --- |
| China Electronic Product Reliability and Environmental Testing Research Institute (CEPREI) Mr. Zhang Jian 110 Dongguan Zhuang Road, Box 1501-33 GUANGZHOU 510 610 China | TP + 86 20 872 36606 TF + 86 20 872 36230 info@ceprei.org http://www.ceprei.org |
| Gosstandart of Russia Mr. A A Biktchentaev 9 Leninsky Prospect 117049 MOSCOW Russian Federation | TP + 7 095 2364044 TF + 7 095 2376032 Eltech@gost.ru |
| Istituto Italiano del Marchio di Qualità (IMQ S.p.A.) Mr. F Pezzoli Via Quintiliano 43 I 20138 MILANO Italy | TP + 39 02 5073 395 TF + 39 02 5099 1524 fabio.pezzoli@imq.it |
| Korea Testing Laboratory Korea Institute of Industrial Technology Dr JK Ham 222-13 Kuro-dong Kuro-gu SEOUL 152-053 Korea (Republic of) | TP + 82 2 86014 75 TF + 82 2 86014 56 jkham@ktl.re.kr |
| LCIE/SNQ Mr. P Guillon 33 avenue du Général Leclerc BP 8 F 92266 FONTENAY-AUX-ROSES CEDEX France | TP + 33 1 40955539 TF + 33 1 40955530 philippe.guillon@lcie.fr |
| Reliability Center for Electronic Components of Japan Mr. N Shiono Shin-Daiichi Building 6F 3-4-13 Nihonbashi, Chuo-ku TOKYO 103-0027 Japan | TP + 81 3 32726420 TF + 81 3 32722926 shiono@rcj.dp.u-netsurf.ne.jp |
| SEV Mr. H Berger Luppmenstrasse 1 Postfach CH 8320 FEHRALTORF Switzerland | TP + 41 1 9561314 TF + 41 1 9561321 heinz.berger@sev.ch |
| STQC Directorate Mr. KC Chhabra Ministry of Information Technology Electronics Niketan 6 CGO Complex NEW DELHI 110 003 India | TP + 91 11 436 2381 TF + 91 11 436 3083 stqc_cert@hotmail.com |
| Underwriters Laboratories Inc Mr. J. A. Chiaramonte 1285 Walt Whitman Road MELVILLE NY 11747-3081 USA | TP + 1 631 2716200 TF + 1 631 439 6024 joseph.a.chiaramonte@us.ul.com |
| VDE Testing and Certification Institute Dr. G. Dreger Merianstrasse 28 D 63069 OFFENBACH Germany | TP + 49 69 8306535 TF + 49 69 8306636 pi.dreger@vde.com |

From IECQ Web Site :  www.iecq.org

Appendix 8:

## Provisional List of Testing Laboratories (as of Feb 1, 2002)

A.-   Laboratories Accredited by IECQ and listed by PV GAP

| Arizona State University Photovoltaic Testing Lab. Tempe Arizona, USA Phone: 1-480 727 1220 FAX: 1-480 727 1223 Ptl@asu.edu www.asu.edu/east/ptl | Accredited to ISO/IEC 17025 | Modules of all types | Capability: All types of tests for IEC 61215 and IEC 61646 |
| --- | --- | --- | --- |

B.- Other accredited Laboratories in the process of being listed by PV GAP

| JRC-ISPRA European Solar Test Installation Phone: 39 332 789 196 FAX: 39 332 789 268 | Accredited to ISO/IEC 17025 | Modules of all types | Capability: All types of tests indoors and outdoors reliability test |
| --- | --- | --- | --- |
| FSEC Florida Solar Energy Center Phone: 1 321 638 1500 FAX: 1 321 638 1010 | Accredited to ISO/IEC 17025 | Small Home Systems | Capability: All types of tests outdoors for Stand Alone PV Systems. |
| BPPT/LSDE Indonesia Phone: 62 21 316 9749 FAX: 62 21 316 9765 | Accredited to ISO/IEC 17025 | Modules of all types | Capability for testing modules, components, systems and applications |
| ETDC Bangalore, India Phone: 91 80 839 4252 FAX: 91 80 839 1804 | Accredited to ISO/IEC 17025 | Modules of all types | Capability for testing modules. |
| CEPREI, Guangzhou, China Phone: 86 20 872 37204 FAX: 86 20 872 36230 | Accredited to ISO/IEC 17025 | Modules | Capability for testing modules. |
| Intertek Testing Services, Boxborough, MA, USA Phone: 1 607 753 6711 FAX: 1 607 756 9891 | Accredited to ISO/IEC 17025 | Some components | Capability for testing some systems components. |
| TÜV Rhineland, Cologne, Germany Phone: 49 221 806 2711 FAX: 49 221 806 1350 | Accredited to ISO/IEC 17025 | Modules and system components | Capability for testing modules, components, systems and applications |

C.- Laboratories Approved for ISO 9001, not accredited under ISO/IEC17025

| GENEC, France | ISO 9001 approved |  | Modules and system testing. Outdoor facilities and BOS testing |
| --- | --- | --- | --- |
| JET, Japan | ISO 9001 approved |  | Modules and system testing. Appliances and inverter testing. |
| Fraunhofer Inst. Germany | ISO 9001 approved |  | Modules and system testing. |
| ECN, Netherlands | ISO 9001 approved |  | Modules and system testing. |
| CANMET, Canada | ISO 9001 approved |  | Modules and system testing. |
| NREL, Colorado, USA | ISO 9001 approved |  | Module and Cell testing |
| Sandia Lab., USA |  |  | BOS testing |

D.- Laboratories with appropriate equipment but not yet accredited

| IT Power, UK |  |  | PV test and training Center |
| --- | --- | --- | --- |
| CIEMAT, Spain | Expects approval for ISO 9001 in 2002 |  | Modules and system testing. |
| CIEFES, Senegal |  |  | Starting development |
| CDER, Morocco |  |  | Installing equipment for systems and battery testing. |

Appendix  9:

Glossary of Terms

Acceptance tests:  Tests performed on an ongoing piece-by-piece or statistical sampling basis of the as-manufactured and / or installed product or system for the purpose of verifying the acceptability of the current manufacturing and / or installation processes and materials.

Accreditation:  The procedure by which an authoritative body gives formal recognition that a body or person is competent to carry out specific tasks.

(As) needed/ (as) appropriate/(as) necessary: These indicate requirements that may or may not apply to your business.

Approval:  Permission for a product, process or service to be marketed or used for stated purposes or under stated conditions

Audit:  A systematic and independent examination to determine whether or not quality activities and related results comply with planned arrangement, and whether or not these arrangements are implemented effectively and are suitable to achieve objectives.

Auditor: An individual who is tasked with performing audits.

Certification:  The procedure by which a third party gives written assurance that a product, process, or service conforms to specified requirements.

Certify:  Third party gives written assurance that a product, process or service conforms to specified requirements

Component:  A constituent part of a product.

Conformity evaluation:  The systematic examination of the extent to which a product, process, or service fulfills specified requirements.

Continual improvement:  Recurring activity to increase the ability to fulfil requirements

Contract:  Agreed-upon requirements between a supplier and customer transmitted by any means.

Corrective action: An action taken to eliminate the causes of an existing situation of nonconformity, defect or other undesirable condition in order to prevent recurrence.

Customer:  An individual or organization that purchases and receives a commodity, product or service provided by the supplier.

Deficiency:  Sub-standard or non-complying, relative to objective standards.

Document:  Information and its supporting medium.  For example: record, specification, procedure document, drawing, report, standard.  The medium could be paper, magnetic, electronic or optical computer disc, photograph or the like.

Documentation:  Furnishing or authenticating with documents; evidence of a process; conformity to historical or objective facts or standards.

IEC: International Electrotechnical Commission

IECQ: IEC Quality Assessment System for Electronic Components

ISO 9000: A group of standards, under the International Organization for Standardization (ISO), which specify requirements for quality systems and which provide guidance to aid in the interpretation and implementation of the quality system.

Inspection:  Conformity evaluation by observation and judgment accompanied, as appropriate, by measurement, testing, or gauging.

Management representative:  The organization's representative who shall have defined authority for ensuring that a quality system is established, implemented, and maintained in accordance with the standard, and reporting on the performance of the quality system to management for review and as a basis for improvement of the quality system.

Manager:  The person who is exercising authority, taking responsibilities, making decisions, and fulfilling similar managerial functions on behalf of the business.

Non-conformity:  Any instance of failure to meet a specified requirement.

Objective evidence:  Information that can be proven true, based on facts obtained through observation, measurement, test, or other means.

Organization:  A company, corporation, firm, enterprise or institution, or part thereof, whether incorporated or not, public or private, that has its own functions and administration.

Photovoltaic:  Solid-state, semiconductor-based solar electric technology that directly converts light energy into electricity.

Preventive action:  An action taken to eliminate the causes of a potential non-conformity, defect or other undesirable situation, in order to prevent occurrence.

Procedure:  A specified way to perform an activity; a description that details by whom, with what, when, where, and how processes are carried out.

Process:  A series of actions or activities directed to a planned or specific result or product that transform inputs into outputs.

Product:  The result of activities or processes.

Project:  A unique process consisting of a set of coordinated and controlled activities with a start and finish dates, undertaken to achieve an objective conforming to specific requirements, including the constraints of time, cost and resources.

PV GAP: Global Approval Program for Photovoltaics: a world-wide organization for promoting satisfactory photovoltaic products, and systems.

Qualification Test:  A test performed on a randomly selected set of components or a calibration performance of unknowns usually by means of inter-laboratory comparisons.

Quality:  The totality of characteristics of an entity that bear on its ability to satisfy stated and implied needs.

Quality assurance:  All the planned and systematic activities implemented within the quality system, and demonstrated as needed, to provide adequate confidence that an entity will fulfil requirements for quality.

Quality control: Operational techniques and activities that are used to fulfil requirements for quality.

Quality improvement:  Actions taken throughout the organization to increase the effectiveness and efficiency of activities and processes, in order to provide added benefits to both the organization and its customers.

Quality manager:  A management representative with responsibility for the quality system, its implementation, and its maintenance.

Quality Management System:  Management system to direct and control an organization with regard to quality

Quality manual: The document in which is compiled the organization's procedures and documentation for quality and quality improvement programs.

Quality plan:  The document setting out the specific quality practices, resources and sequence of activities relevant to a particular product, project or contract.

Quality policy:  The overall intentions and direction of an organization with regard to quality as formally expressed by top management.

Quality system: The organizational structure and procedures by which an organization's processes are carried out, writing down how things are done and recording the results to show how things were done.

Quality team:  An organization's team of management and staff responsible for developing and reviewing the organization's quality systems.

Registrar: An independent, third-party body that audits organizations for accreditation/ certification and registers those qualified organizations.

Review:  The broad overview of the activities relevant to the situation under study. Here, used in three situations: management review, contract review, and design review.

Service:  The result generated by activities at the interface between the supplier and the customer and by supplier internal activities to meet the customer needs. Also the non-tangible result of a process or effort (e.g., testing, maintenance, repair, consulting, etc.)

Shall:  This means a requirement has to be followed whenever it occurs in the Manual.

Should: (may/can) These items are normally used to suggest or recommend a course of action. They are never used to indicate a requirement that must be followed.

Specified requirements: Product requirements prescribed by the customer and agreed to by the supplier.  Also, requirements prescribed by the supplier that are perceived as satisfying a market need.

Subcontractor:  Any organization from which you purchase products or services, or both. You normally refer to them as your vendors.

Supplier: Organization or person that provides a product

Supply chain:  A set of interrelated resources and activities that accepts inputs from suppliers, adds value to those inputs, and produces outputs for customers.

Tender:  An offer made by a supplier in response to an invitation to satisfy a contract award to provide a product or service.

Training:  Processes to introduce, expand, and/or improve the knowledge and skills of individuals and teams in a systematic way.

Vendor:  An outside provider of a product or service.

Verification: Confirmation, through the provision of objective evidence that specified requirements have been fulfilled.

Validation:  Confirmation by examination and recording of physical evidence that the requirements for a specific intended use or application have been fulfilled.

PHOTOVOLTAIC TERMS:

Photovoltaic effect:  The generation of direct current voltage by the absorption of light.

Photovoltaic system:  An installed assembly of modules and other system components that generate and supply electricity suitable for connection to an electrical load.  The component list and system configuration varies as a function of the application.

Distributed generating system:  The facility and equipment comprising an electricity generation system that is connected and operated in parallel with a utility distribution system.  Owner system may or may not be connected to the electricity transmission system.

Grid-connected operation:  The operating mode in which a photovoltaic system generates electricity in parallel with a utility supply authority.

Stand-alone operation:  The operating mode in which a system’s load is electrified solely by the system and not in parallel with supply authority or other electricity generator.

Balance of system:  The parts of a system other than the array of photovoltaic modules, including switches, controls, meters, power conditioning equipment, support structure for the array, batteries and energy storage if any.  Symbol:  BOS

Appendix 10:

Bibliography

- ISO 9001 for Small Businesses: What to do. Advice from ISO/TC 176, International Organization for Standardization, Geneva, Switzerland. Second Draft, November 2001

- Publicizing Your ISO 9000 or ISO 14000 Certification,  International Organization for Standardization, Geneva, Switzerland 1997.

- Global Approval Program for Photovoltaics (PV GAP) Reference Manual. PV GAP Recommended Standard PVRS 2 Amendment 1, December 1998.

- Manufacturing Quality Process Manual for Photovoltaic Components and Systems Version1.5, June 30,1999.

- Photovoltaic Test Laboratory Quality Manual.  DRAFT Document, Version 1.1, February 22,1999.

- PV Manufacturers Quality Control Training Manual. PV GAP First Edition, 2000

- ISO 9000:2000, Quality management systems-Fundamentals and vocabulary

- ISO 9000-3: 1997, Quality management and quality assurance standards — Part 3: Guidelines for the application of ISO 9001:1994 to the development, supply and maintenance of software.

- ISO 9001: 2000, Quality Management systems-  Requirements

- ISO 9004:2000, Quality Management systems-Guidelines for performance improvements

- ISO 9004-4: 1993, Quality management and quality system elements – Part 4: Guidelines for quality improvement.

- ISO 10005: 1995, Quality management – Guidelines for quality plans.

- ISO 10006: 1997, Quality management – Guidelines to quality in project management.

- ISO 10007: 1995, Quality management – Guidelines for configuration management.

- ISO 10011-1:1990, Guidelines for auditing quality systems — Part 1: Auditing.

- ISO 10011-2:1991, Guidelines for auditing quality systems — Part 2: Qualification criteria for quality systems auditors.

- ISO 10011-3:1991, Guidelines for auditing quality systems — Part 3: Management of audit programmes.

- ISO 10012-1:1992, Quality assurance requirements for measuring equipment — Part 1: Metrological confirmation system for measuring equipment.

- ISO 10012-2:1997, Quality assurance requirements for measuring equipment — Part 2: Guidelines for control of measuring processes.

- ISO 10013:1995, Guidelines for developing quality manuals.

- ISO/TR 13425:1995, Guidelines for the selection of statistical methods in standardization and specification.

- ISO 9000:2000 New Requirements-28 New Requirements, Checklist and compliance guide

Jack Kanholm.  AQA Co. 2001

- ISO 9001:2000 Explained (Second Edition) Cianfrani et al. ASQ Quality Press, 2001

- ISO 9001:2000 for Small and Medium Businesses, H.C.Monnich Jr. ASQ Quality Press,2001

- QC 001001:1998, Basic Rules of the IECQ, IEC.  www.iecq.org.

- QC 001002:1998, Rules of Procedure of the IECQ. www.iecq.org

- QC 001004, IECQ Specifications List  www.iecq.org

- QC 001005, Register of IECQ Approvals  www.iecq.org

- “IECQ in Brief”.  IECQ website (www.iecq.org)

- IEC Web Site :  www.iec.ch

- ISO Web site:  www.iso.ch

- PV GAP Web site: www.pvgap.org

- World Bank Web site: www.worldbank.org/energy

Quality Management in Photovoltaics – Manufacturers Quality Control Training Manual (PV GAP Reference No: QM 2.0)

EA Guidelines on the application of EN 45012 (European co-operation for Accreditation – http://www.european-accreditation.org) September 2000-rev.00

Private communication (October 2001): Mr. A. Bergman, VDE Testing and Certification Institute, Offenbach, Germany

EA Guidelines on the application of EN 45012 (European co-operation for Accreditation – http://www.european-accreditation.org) September 2000-rev.00

PV GAP Reference Manual, Version 1.1, 1998. ©

Reprinted with the permission of PV GAP