## Chapter Four: ISO 9001:2000 Standard (Continued)

###### Chapter 4  Measurement, Analysis, and Improvement            Clause 8

4.1 General concepts        			(Clause 8.1)

While this clause is new in the ISO 9001:2000, it involves several concepts and factors that were implicit in the previous issue of the Standard.  The main purpose of this clause is to establish a Plan to review requirements and to check the performance of the execution of clauses 4.1 and 7.0, that is, documentation control  and  product realization processes.  This clause requires that the data be collected, measured, analyzed, and presented to the management of the company.  The management is committed to a formal follow up on progress and  failures of the quality system and a review of customer satisfaction and continuous improvement.

The main concept is to have management reviews, establish preventive and corrective actions, and instill the quality improvement goals throughout the organization.  This also opens the forum for review of the resources, training and infrastructure that may be necessary to meet customer requirements.

Measurements are best presented through statistical methods and graphs, although many other types of analysis are also applicable.  In order to present the available statistical methods most commonly used, the ISO TC 176 offers a Standard ISO/TR 10017:1999, which is still applicable although  developed for ISO 9001:1994.

Although the Standard does not discuss the statistical methods, we include below a brief explanation of those most commonly used.

4.1.1 Statistical Data Analysis

Quality improvements require measures of status and comparisons after corrective action is taken. This is usually achieved through comparison of data collected in the company’s various activities.  The data collection process is the first step for any analysis.  Receiving of materials, processing steps, equipment malfunctions, testing of the product, and handling at the shipping dock -- all these activities produce some sort of data that should be collected systematically to obtain a clear picture of what’s going on.

The personnel doing this frontline work usually gather the data – e.g. a sales person who gets feedback from the customers, a customer service clerk who observes promised versus actual shipping dates, or operators on the manufacturing line who handle product at various stages in the manufacturing process.  Gathering data is an important function of each employee.  Subsequent data analysis can take various forms and be carried out through a variety of methods, but  the Management Representative is usually responsible for making sure that the responsible managers or process engineers analyze the data.

If there are non-conforming products or results, data should indicate increased activity in rejects or rework. This data should be analyzed and the cause of the problem investigated.  Several methods of analysis can be used to form a decision about how best to resolve the problem.  Continued reworking or sorting only perpetuates quality losses.  Optimally, data collection and analysis can yield a basis for preventive actions that prevent defects from occurring.  Analyzing the data by statistical methods is indispensable, in correlating the corrective actions taken and the resolution of the root cause.

Some non-numerical methods commonly used to determine the best course of action to be taken include:

- Benchmarking

- Brainstorming

- Cause and effect diagrams

- Flowchart or process descriptions

In addition, various numerical methods are available for analyzing the data. The techniques more frequently used for numerical analysis are:

- Pareto charts

- Histograms

- Scatter diagrams

- Control charts

- Run charts

- Percent charts

# Description of Non-Numerical Methods

Benchmarking

This technique compares the company’s processes and performance to those of other companies, recognized as leaders in the same field or other similar fields. The optimal comparison is to the best in class in your own area. This could lead to goals and plans that will impact your marketing strategies.  For example, one might compare field life performance to warranty and find that a marketing advantage could be obtained if you offered a better warranty.

However, data on process yield or data on plant equipment performance and uptime of specialized equipment are more sensitive and less likely to be shared among competing companies.  For this reason, benchmarking may be used to compare your performance against companies in other industries that use similar technologies.  For example, in solar cell manufacturing, it is common to benchmark against semi-conductor companies, due to similarities in materials, processes and equipment.

This technique could be utilized to evaluate the merits and costs of new technologies and how they have been implemented in other companies, as well as to evaluate the benefits and costs (e.g. for maintenance and training) of acquiring new equipment used in other industries.

Brainstorming

This process is useful when a team needs to resolve a problem and there is no clear initial understanding of the problems or issues involved. The team gets together with a facilitator with the aim of generating as many ideas as possible in a sequential manner from all participants.  One person’s idea can stimulate ideas from others. The process is open, with no criticism of the ideas.  The purpose is to generate as many ideas as possible. The ideas are posted for everyone to see and evaluate. When there are no more ideas or suggestions, critical evaluation begins.  Ideas are clarified and discussed and a selection of the best is made for future action.

#### RULES FOR BRAINSTORMING

BOX 4.1

- Generate a large number of ideas.

- Unbridle the imagination.

- Don’t criticize.

- Make sure everyone participates.

- Record all ideas.

- Let ideas incubate.

This technique is often utilized to discuss potential project needs for a company, to prioritize the implementation sequence, or, to review the impact of new personnel policies on employees.

Cause and Effect Diagram

This Diagram is also called a  “Fishbone Chart”.  Defining the problem starts the process.  Next, several causes are identified that could influence quality results.   For example, operator skills, machine repairs, materials used, plant environment, etc.   For each cause, one or more contributing factors is listed (e.g. under plant environment,  such factors as ambient temperature, humidity, dirt, etc. might be identified.)

The impacts of these factors are then analyzed.  Those known to have stronger influence are selected for future action and problem resolution.

##### CAUSE AND EFFECT DIAGRAM

##### Figure 4.1

ENVIRONMENT

MACHINE

MATERIAL

EFFECT

OR

PROBLEM

### MEASUREMENT

### METHODS

### OPERATORS

# Flowchart and Process Description

A flowchart is a pictorial description of the steps in a process.  The benefit of using a flowchart is to be able to identify each step and to determine at what exact point it is most likely that a problem is occurring.  A new process or changes in the flow may remove the problem and improve the quality of the product.   Once a thorough analysis is made, a new flowchart is created.  ISO requirements for documentation and process change should be met.

Flow charts are very often used to review the impact on the work process of a proposed new method or change in work sequence.  Sometimes a change in vendor means that new materials control procedures need to be phased in.  A sample flow chart follows.

###### SAMPLE FLOW CHART

Figure 4.2

RECEIVE CELLS

ATTACH

INTERCONNECTS

PULL

TEST SAMPLE

CREATE STRINGS

AND

MATRIX

STOCK

OR

SHIPPING

WIRE AND CLEAN

INSPECT

AND

TEST UNIT

LAMINATE

OR

ENCAPSULATE

INTERCONNECT MATERIAL

ACCEPT

INSPECT

REVIEW

LOT QUALITY

REJECT

GLASS & LAMINATE MATERIALS INSPECTION

BACK TO VENDOR

ACCEPT

GOOD VALUE

LOTS

REJECT

LOW

VALUE LOTS

MEETS

CONFOMING

SPEC?

NO

YES

ROUTE

TO REWORK

FRAME AND

PACK

Description of Numerical Methods

# Pareto Charts

Pareto charts are used to present data or conditions related to the type of failure  occurring. For example, in the manufacture of solar cells, you may have a large number of rejects; the data should indicate what type of rejects you have (e.g. broken cells, chipped cells, or electrical rejects).

If your process is under control, an increase in broken wafers indicates that the quality of silicon wafers may be affecting the process, or that some process has gone out of control unnoticed.

A Pareto chart can be used to analyze not only the type and number of failures, but also their cost implications.  Losses of cells at attaching interconnects might be higher in numbers than losses at the module level, if the interconnect pull strength is not good; however, analysis might reveal that failure of one unit at the module level is more expensive than a loss at the beginning of the line.

The discrepancy values for a module line might look like those in the following table. The applicable Pareto charts are shown at the bottom of the page.

| LEGEND | % | TYPE of DISCREPANCY | QTY. | COST |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A | 40 | LAMINATION DEFECTS | 200 | 4400 | 22 |  |
| B | 24 | JUNCTION–BOX DEFECTS | 120 | 168 | 1.4 |  |
| C | 24 | CELL ALIGNMENT | 120 | 2400 | 20 |  |
| D | 8 | MATRIX ALIGNMENT | 40 | 2880 | 72 |  |
| E | 4 | GLASS DEFECT | 20 | 1440 | 72 |  |

##### SAMPLE PARETO CHARTS

Figure 4.3

The above tables indicate that, although failure mode B occurs with greater frequency, the category D and C failure modes are more critical because the cost of these failures are higher.

# Histograms

The histogram is used to review process patterns or variations.  Test results in solar cell manufacturing are usually good data to plot in histogram form.  Such results should be near a normal distribution, but, in general, they skew to the left, if plotted against efficiency.  On occasion, the results could show a bimodal distribution.  In solar cell test results, this could indicate an anomaly in the process (e.g. a difference in temperature between tubes in a diffusion furnace, a non-uniform condition across a belt in a belt furnace, etc.)  Bimodal distributions will be representative of a mixed distribution or two distributions superimposed. Maintaining such data is a good way to improve quality in a manufacturing line.

The following are examples of several types of distribution plotted in histogram format:

##### SAMPLE HISTOGRAMS

##### Figure 4.4

Performance Charts

Bar chart graphics are frequently used to present the past and present performance of a unit or of factory output and to compare it with expected performance. Charts are made based on prior experience (in our example, over three years) and the anticipated goals for the manufacturing step or department. The figures show month-to-month actual performance over the last three years for the cell line and the module assembly. These charts can give plant management a quick overview of the degree to which performance is or is not measuring up to expectations. Corrective action may be necessary, if performance is not adequate.

PERFORMANCE CHARTS Figure 4.5

Target 95%

Target 95%

Scatter Diagrams

Scatter diagrams are used to find the correlation between two different sets of data. The data seem like a cloud of points. Sometimes they are scattered throughout the page and do not show any relationship, but on many occasions they show a marked trend and statistical correlation. This can be used to optimize process parameters by changing some of the parameters and observing the outcome data.  One example could be the correlation between furnace temperature and the sheet resistivity of the junction formed; another may be the correlation between contact series resistance and firing temperatures.

The relationships can be negative slope, positive slope, or curvilinear.  The numerical correlation can be strong or weak, depending on the scatter around a trend line in the data points.  Several examples follow.

SCATTER DIAGRAMS

Figure 4.6

Scatter diagrams are valuable tools for process improvement.  The engineering department typically uses them to analyze data during proposed process changes.  If one variable (X) is changed, the performance of variable (y) can be analyzed.  Experiments are usually designed around more than two variables, requiring more complex analysis.

# STEPS TO MAKE A CONTROL CHART

TABLE  4.1

1.  GATHER AND CONVERT DATA

2.  CALCULATE AVERAGE CONTROL LIMITS

3.  DETERMINE CHART SCALE

4.  PLOT CHART

5.  ANALYZE AND INTERPRET

###### STATISTICAL TOOLS FOR PROBLEM SOLVING

Figure 4.7

### PROBLEM SOLVING STEPS			TOOLS TO BE USED

STEP 1. Identify & Select the Problem

Problem Census

Interviews

Guided Discussions

Focus Groups

STEP 2. Analyze Causes and Effects of the Problem

STEP 3. Generate Potential Solutions

STEP 4. Select Best/Feasible Solutions

STEP 5. Implement a Solution or Strategy

### STEP 6. Evaluate Results of the Solution

Fishbone Diagram

Flowcharting

Critical Incident Analysis

Storyboarding

Brainstorming

Rapid Prototyping

Quality Circle

Scenario Building

Decision Tree

Nominal Group Technique

Criteria Rating

Weighted Voting

Force Field Analysis

Brainstorming

Survey

Data Base

Interview

Control Charts

The use of histograms opens the door for the normal distribution curve and the statistical control of data. Data is usually organized so that the process average is calculated. The data is then monitored on a daily basis and the averages of the data plotted in a continuous chart. This chart will indicate the variation around the average and how much the daily averages deviate from the established mean. After a number of days of plotting daily actuals,  the upper limit and the lower limit of the average daily data can be found. This is indicative of the amount of deviation that the process has for the particular parameter or attribute being plotted.

This type of plot indicates the status of the process and whether it is improving or deteriorating over time. The smaller the variance, the better the control.  Another feature is the indication of trends in manufacturing, either by shifting of the average or by increasing or decreasing the deviation.

The ideal case is when the deviation is less than a few points and the average remains unchanged, when no changes are introduced in the process.  If an improvement is introduced, its impact should improve the average or narrow the upper and lower limits for the parameter.

You can use this tool to demonstrate that the process is under control, or to gauge the capability of the process and evaluate the improvements introduced in the line.

CONTROL CHARTS

Figure 4.8

Attribute Data and Indications of Controls

Trends

Shift

The use of control charts is usually the duty of the operators.  They gather the data and immediately generate the control chart, a very graphical analysis that enables them to observe whether the process is under control (within control limits) or not.  If the process departs from its normal range, the issue is brought to the attention of the operation managers and engineers, and its resolution is achieved in a shorter period of time than  would occur in the absence of such a statistical tool.

In summary, all of the above methods are part of an effort called Statistical Process Control (SPC). Sampling plans are usually utilized instead of 100 % inspection in many operations. The size of the sample  and the analysis of product failure rate could be indicative of the correctness of the sampling plan.  The ideal plan would detect all failures, but this may not be a realistic expectation.  Therefore, the sampling plan is usually verified against the final test and acceptance test, and, ultimately, against customer complaints and field returns. The objective is to conduct a systematic review to determine the proper level of sampling, in combination with verification against other information, to achieve the required quality.

Statistical analysis goals and procedures should be updated whenever new or improved data becomes available.  Other business activities that benefit from the applications of statistical processes are:

- Market research

- Product design

- Process optimization by design of experiments

- Stock and inventory controls

In conclusion, statistical process control is a very valuable tool that must be utilized in every area targeted for improvement in your company’s Quality Plan.

4.2 Monitoring and Measurement                  (Clause 8.2)

This clause concerns the collection of data for different areas of interest such as customer satisfaction, processes, products, and all the other functions associated with the operation of your business.  The note includes monitoring first as a less intense effort, and then measurement for numerical data, which can be compared over  time to evaluate progress made with the quality system.

4.2.1 Customer satisfaction		(Clause 8.2.1)

This is the main thrust of the ISO 9001:2000 standard.  The customer is the direct recipient of your product or your services.  Your company survives because it has customers. It is everyone’s desire that the customer is satisfied with the product that you have delivered.  The problem arises in that it is often not easy to find out what perception the customer has.  Thus, it is important to ask the customer or to make surveys to find out possible weaknesses in your company.

Many times, information received is misused or nothing is done about it.  We often treat the “status quo” as all we can do.  The purpose of this clause is to eliminate that feeling, help you organize and plan activities to obtain direct or indirect customer feedback, analyze the information, and make an action plan to resolve issues.  As a top company priority, Management Reviews shall devote considerable time to analysis of customer satisfaction and the necessary action to resolve these problems.

There are various methods for gauging customer satisfaction. Some data sources are:

- Customer complaints

- Customer returns

- Questionnaires or surveys

- Warranty claims

- Direct customer contact

- Benchmarking data

- Industry reports or studies

- Consumer organizations

Statistical analysis of the data is very important. Usually it helps to select some large customers and work together with them on major issues.  Follow up and attention to complaints until their resolution, helps eliminate a large percentage of complaints and also allows the customer to feel more closely related to your company.

The Action Plan and the Management Review provide documents that shall be recorded and will be audited by the ISO inspector during your official audit report. The ISO standard requires two types of audits -- internal and external.

4.2.2 Internal Audit				(Clause 8.2.2)

The purpose of this clause is to verify through “quality audits” that the procedures for planning and implementing your Quality System are being followed and planned results achieved. These audits are also necessary to determine the effectiveness of the Quality System; if the planned results are not achieved, the activities should be corrected and improved. The process of implementing a Quality System is a long one and involves hard work on the part of a lot of people.  If the system is implemented without periodic checking, procedures will tend to be changed, abandoned and soon forgotten.  Quality management requires that there be an auditing method to verify that procedures continue to be kept up to date and followed.

The Internal Audit

An internal audit is your company’s self-audit.  Internal audits should be organized and planned by the quality organization and should be properly documented to show that they have been carried out in a systematic fashion and that reports have been issued for review by the management team.  Audits are generally implemented by personnel who do not work in the operation being audited. The audit consists of the following activities:

- Select the process to be reviewed

Review process flow, activities and flow

Plan which part of the process will be audited

Recognize the input and the output typical of the process

Recognize responsibility and authority of all members

- Plan and execute activities

Reviewing procedures

Observing whether procedures are being followed

Talking with and observing the people carrying out these procedures

Looking at the actual records of the operation vs. the requirements

- Check all the process performance and achievement of goals

- Report and take corrective or preventive action if necessary.  A report should be written for the area manager and the Management review. (5.6.2)

Form 8.2-1 is useful for planning an audit.  Refer to the self-audit exercise on the next page to see the most commonly asked questions in an internal audit.  The results of the audit will be brought to the attention of the responsible manager for the operation being audited. If corrective action is required, the responsible manager should initiate this in a timely manner, update the procedures to reflect new knowledge and best practices, or train personnel if they are not following the procedures properly.

The goal of the audit is to confirm, in an objective manner, that the processes are being followed correctly and that everyone is doing what ought to be do. The auditor should also review whether employees have adequate training and understanding so as to perform their jobs well. Corrective or preventive action is a normal outcome of an audit.

The audit should be recorded on a form similar to Form 8.2-2.  To avoid a conflict of interest, people not involved in the operation ideally should perform the audit. Many alternatives should be tried, even using personnel from a different company to obtain objectivity in the review. The frequency of auditing shall be planned and related to the importance of the operation being audited and the company’s experience with problems in that area as shown by previous audits.

# SELF-AUDIT EXERCISE

BOX 4.2

Documentation:

- Is your quality management system documented?

- Quality manual

- Flowcharts

- Standard operating procedure

- Does your documentation system prevent the use of obsolete procedures / processes?

- Do you keep quality records? Are they legible, identifiable and easy to access?

- Do you have a documentation control system to maintain all documents and specifications up to date?

Purchasing, materials and supplies:

- Do you have a formal process to select and evaluate suppliers?

- Do you have a list of approved vendors?

- Do you ensure that key services and materials are only purchased from approved sources?

- Do you have specifications for your materials and are these up-dated?

- Do you have a control procedure for the release of Purchase Orders and their content?

- Do you have control of incoming materials and their conformance with the specification and delivery schedule?

- Do you keep records and statistics on the performance of each vendor re quality and delivery expectations?

Manufacturing:

- Is there a procedure to establish and document new manufacturing processes?

- Do you have written instructions or charts to cover all operations, especially those critical to quality?

- Do you have a formal training program?

- Do all operators performing a task receive training and have knowledge of the task and its impact on quality of the product?

- Do you keep records of quality at different stages and plot deviations of parameters critical for each operation?

- Does each employee know equipment operating parameters and preventive maintenance requirements?

- Does the operator know where to find the documentation for the processes?

Nonconforming material or product:

- Does each operator know the procedure and records needed for the disposition of nonconforming products?

- Is the nonconforming product identified properly and isolated from standard products?

- Is the operator aware of the notification process and does he or she issue reports on nonconforming products immediately?

- Are repairs and rework documentation completed and product re-inspected before release?

- Is a procedure in place for investigation of nonconformance and corrective action taken?

The audit report should include a review of the operation to assure that the previous corrective actions were made and improvements in the operation have been properly documented.  Internal audits should include procurement and inventory reviews.  Internal audit records are reviewed carefully during external quality audits and the certification process.

INTERNAL AUDITS

Figure 4.9

1

Management Representative

Manufacturing

2

3

4

5

Procedures

Operator Actions

Review Records

Quality Awareness

Safety Awareness

Non-Conforming Product

Procedures

Operator Actions

Review Records

Quality Awareness

Safety Awareness

Non-Conforming Product

5

Procedures

Operator Actions

Review Records

Quality Awareness

Safety Awareness

Non-Conforming

Product

Questions

Calibration & Inspection

Order/Contracts

Maintenance

Inventory

Non-Conforming Products

Internal Audit Team

Dept 5

Dept 4

Dept 3

Dept 2

Dept 1

Dept 1

Non-Compliance

The most common reasons for non-compliance during audits have been summarized in the September 1992 issue of Computer Technology Review.  They are as follows:

|  | Percentage |
| --- | --- |
| Document Control | 25 |
| I Inspection & Testing | 20 |
| Quality Records | 18 |
| Process Control | 16 |
| Management Responsibility | 10 |
| Training | 10 |
| Purchasing | 10 |

MOST COMMON REASONS FOR NON-COMPLIANCE

TABLE 4.2

### Procedure for Corrective or Preventive Action Required

- Notify findings and identify nonconformity

- Analyze of nonconformity by statistical means

- Clarify section responsible for nonconformance

- Investigate the root cause of nonconformity

- Evaluate  and plan corrective action

- Implement corrective action

- Record results of corrective action

- Evaluate results and confirm effectiveness of action taken

9.   If the action was not effective, repeat the process until nonconformity is resolved

AUDIT PLANNING 
(Form 8.2-1)

1) NAME OF MANAGEMENT REPRESENTATIVE RESPONSIBLE FOR INTERNAL AUDITS (note: auditor must be independent from the function being audited):

2) SCHEDULE OF INTERNAL QUALITY AUDITS

YEAR

AUDIT PERIOD	1ST QTR	2ND QTR	3RD QTR	4TH QTR

CONTRACTS

PROCUREMENT

RECEIVING

DESIGN

MANUFACTURING

TESTING

PACKING/WAREHOUSE

SHIPPING

INSTALLATION

SERVICE

RECORDS

3) AUDITS PERFORMED AND REVIEWED

AUDITOR				GROUP			GROUP MANAGER

CONTRACTS

PROCUREMENT

RECEIVING

DESIGN

MANUFACTURING

TESTING

PACKING/WAREHOUSE

SHIPPING

INSTALLATION

SERVICE

RECORDS

4) RESULTS OF INDIVIDUAL AUDITS SHOULD BE APPENDED TO THESE RECORDS AND COPIES SHOULD BE FILED WITH THE PERMANENT QUALITY RECORDS

INTERNAL AUDIT REPORT

Form 8.2-2

Internal Audit Number ____________________			Date ________________

Auditor ______________________________ Department Audited ___________________

Process Audited ______________________

Operator talked to: ___________________________________	Date ___________

Department __________________________________________

Reports To: __________________________________________

Department Manager: __________________________________

Operation/ Activity Audited:___________________________________________________

Document/ Specification/ Process Number: ______________________________________

Observation Found:_________________________________________________________

_________________________________________________________________________

________________________________________________________________________

Compliance Status: ___ Satisfactory ____Concern _____Non Compliance ____Not Available

Are Documented Instructions Available? ___________________________________________

Has the Operator been trained for this Activity? ______________________________________

Are the Instructions been followed? _______________________________________________

Has previous Corrective Action been taken? ________________________________________

Is the available Documentation up to date (revision level)? ____________________________

Has the Operation Manager been advised of status? __________________________________

Manager Signature ___________________________	 Date ____________________

Auditor Signature ____________________________ 	Date ________________________

Comments __________________________________________________________________

Corrective Action requested____________________________________________________

Preventive Action requested____________________________________________________

4.2.3 Monitoring and Measurement of Processes	(Clause 8.2.3)

This clause is new and should not  be confused with the effort of monitoring and measurement of parameters in a manufacturing line for the product realization.  Instead, this clause goes beyond such effort and includes the need to monitor and measure the manufacturing parameters.

The whole Standard is based on a sequence of processes that were explained in paragraph 1.5.3 of this Manual.  This clause attempts to monitor and measure the success or failure of the sequence of processes that integrates the organization.  Several processes cannot be measured in the commonly known parameters, but they can be monitored and evaluated by the degree of failures to achieve goals.  One example can be the failure to deliver product at the date promised by the sales department to the customer.  This can be due to deficiencies in the manufacturing (product realization) cycle or could be the result of a wishful plan from the salesperson in order to obtain the order.  If this condition repeats itself, the process of booking the orders must be reviewed and a realistic schedule proposed.

The most typical interpretation of the monitoring and measurement activity corresponds to the design effort and the product realization effort.  A plan has to be developed to provide management with key measures that indicates the health of specific efforts that are critical to the success of the organization.  Usually you can identify hundreds of parameters and data that could be of relative importance. The challenge is to select the important parameters  for key processes that limit the ability to satisfy customer requirements.

The use of statistical techniques helps to provide these measurements and analyze the monitoring details.  The relevant procedure(s) should be identified and incorporated into the Quality Manual and into the items in the Management Review process.  If some of the data or monitoring are found to be irrelevant to the goal of controlling and improving the processes efficiency, they should be discontinued; but any modifications to the monitoring and measurement procedure in the Manual should only be made after approval by the proper authority.

4.2.4 Monitoring and Measurement of Product		(Clause 8.2.4)

The measurement and monitoring of product characteristics at various stages in the manufacturing cycle or product realization was discussed at length in Chapter 3 of this Manual.

The intent of the present clause is to monitor that all critical parameters listed in the Quality Plan are met and that there is recorded evidence that the monitoring and measurement took place for all products or services.  The various steps in the process are to be monitored to assert the fulfillment of the Quality Plan.  The purchased components, their incoming inspection and certification, the necessary output parameters characteristic of all steps in the realization process, all have to be listed and the method of acceptance documented.  Sometimes, a dedicated person monitors that the adequate steps and measurements have been taken.  Often, the next operator in line is the one that performs that function.  Whichever procedure your company adopts as the most adequate and practical, shall be listed in the Quality Manual and monitored for the responsibility of accepting the materials.

The identity of the person charged with monitoring, shall be noted in a routing ticket or an acceptance document.  For intermediate steps this could also imply that if materials are accepted for further processing without having completed all the required acceptance procedures, those materials have to be identified for potential removal if the specified parameters are not met.

The most critical step for monitoring and measurement takes place at the final test and inspection of the product.  This  is the last stage where you have control of the product before its release to the customer.   Records of all tests made and the parameters observed are part of the records that show the product meets the customer requirements.  An authorized Management Representative or a designated person is usually responsible for the final release of the product and exercises the release authority.

Sometimes the results of the final test are not yet available when the product needs to be released.  In these cases, the product can be released with the customer authority and under conditions of a recall if the need for further testing, adjustments or failure to meet the specifications is detected.  This can happen when a voltage regulator unit is released without a final burn in process, and it is later found that some components show instability in the output voltage that requires adjustments.

The main requirement of this clause is to maintain positive records that the product has met all the specified requirements during the manufacturing cycle and prior to final release.

4.3 Control of Nonconforming Product		(Clause 8.3)

There will be times when things don’t work as planned and some quality problems arise.  At such times, actions should be taken to identify and address these problems as soon as possible.  However, in the interim, some product may be nonconforming and not meet intended specifications or performance parameters. (Form 8.3-1)

This section of the Standard requires that procedures be established for handling and identifying corrective action and for handling and disposition of nonconforming product.  Information about product deviation, corrective actions, and proper disposition of the product should be recorded.  The customer and other affected parties, internal or external, should be notified. A flow chart for handling the nonconforming product is shown in Figure 4.10.

This section emphasizes that you must first identify the non-conforming product. This may be accomplished by inspection or testing, either during manufacturing or during the final test. It might also happen as a result of a customer complaint or a warranty claim.

Second, your company must segregate the non-conforming product and identify it by means of labels or tags or by placing it in a separate physical location. This will prevent errors caused by mixing this product with good materials.  While awaiting disposition, nonconforming products should be stored apart from the area where good materials are being handled.

The material or product should be reviewed.  The decision about its disposition should be made by the Management Representative, together with others with responsibility and authority to make this decision. In some cases, the customer must be notified and will participate in this decision.

The nonconforming product can be disposed of in various ways, for example, as follows:

- Rejected or scrapped.

- Accepted and used as is, with some concession from the customer or user.

- Used as is for another application of a lower grade.

- Reworked or repaired by an acceptable procedure with retest and inspection after repair.

Whatever decision is taken, records should be kept. A logbook note or some identification sign shall be kept for future records, in order to recall the actions taken in case similar problems arise in the future.

# HANDLING  NON-CONFORMING PRODUCT

###### Figure 4.10

Operator

Good Product

Failure Analysis

Report

Scrap material

Tag as separate lot

Nonconforming Product Detected

Nonconforming Product Separated

Process Control Engineering

Review Board

Corrective Action

Approvals

Documentation

Continue in flow process

Rework, if possible, as a separate lot

THIS AREA MUST

BE SEPARATE FROM

NORMAL MANUFACTURING

AREA

FORM FOR PRODUCT STATUS, DISPOSITION, AND 
CORRECTIVE/PREVENTIVE ACTION 
(Form 8.3-1)

1) PRODUCT NAME:

2) UNIQUE ID:

3) Tracking Product Status

(if “Status” is “Fail,” complete #4, below)

LOCATION		RECEIVED BY	DATE/TIME		STATUS

(ex., Receiving)	(Name or initials)	(ex., 99-10-01, 08:35)	(ex., pass/fail)

______

4) PRODUCT NON-CONFORMITY

Reason for Non-Conformity

Product Removed To (location)

Disposition of Non-Conforming Product:

Reworked to meet specifications

Accepted with or without repair by concession

Re graded for alternative applications

Rejected or scrapped

5) AUTHORIZED SIGNATURE/DATE

6) CORRECTIVE ACTION

Handling customer complaints or product non-conformities

Investigation of the cause of product non-conformities and documenting the results

Determine corrective action

Apply controls to ensure that action is taken and is effective

7) AUTHORIZED SIGNATURE/DATE

8) PREVENTIVE ACTION

Assess and use information from process audits, customers, records, service

Determine steps needed to deal with any problem requiring preventive action

Initiate preventive action and application of controls to ensure effectiveness

Submit management reviews documentation of actions

9) AUTHORIZED SIGNATURE/DATE

4.4 Analysis of Data			                           (Clause 8.4)

Data analysis is one of the most useful activities of your Quality System.  It is noted as one of the basic quality principles from which is derived the ISO 9000:2000 quality process.

Data gathering is a normal technique in the discussion of various sections of this Standard, but data is meaningless unless it is analyzed, interpreted and utilized to provide future corrective or preventive actions.

While the Standard mentions four critical areas of action, these shall be interpreted as a minimum.  Other areas of activity should also use data to benefit in their pursuit of improvements.  One clear example is the product development area.  Decisions on the type and characteristics of a product should be based on marketing and competitor data, which indicate what customers want and need.

An additional goal of data analysis is to determine, by objective and measurable means, the effectiveness and performance of your Quality System and to identify areas for improvement.

The specific four areas that the Standard singles out for data analysis are:

- Customer satisfaction

- Conformity to product requirements

- Characteristics and trends of processes and products and possible improvements

- Suppliers

Customer satisfaction has been differentiated from conformity to product requirements so as to focus on and analyze what the customer wants or desires. The customer may be knowledgeable as to the characteristics of the specifications and the requirements but  may want to see a different product, even if it does not meet those requirements.  This usually offers an opportunity to design a new product and gain a new market area, although it may not be for a product that meets existing norms.  A good example is the development for a marine module for the United States Coast Guard, which had very specific requirements that no other module could meet, even though the other modules met normal specifications for a photovoltaic module.

Data analyses can help evaluate:

- Product performance deviations

- Training effectiveness

- New processes

- Evaluating Customer complaints

- Missed delivery dates

- Customer satisfaction levels

- Critical process performance

- Supplier selection and materials improvement

Statistical methods, such as those described earlier, offer important tools for analyzing data.  Different methods can be applied, depending on the type of data, but the results of analysis must be oriented towards elimination of nonconforming product or product improvement.  Achieving both these objectives will improve customer satisfaction as well as the operational performance of your company.

There cannot be continuous improvement without continuous data analysis.

4.5  Improvement				                           (Clause 8.5)

4.5.1 Continual Improvement		(Clause 8.5.1)

The Quality Management System may not always be perfect in its implementation.  Many loopholes and problems can develop.  These can lead to nonconforming products which; demand corrective actions.  Many of these are due to defective training, and they can be avoided if your organization has better training or different type of training. Some are related to product issues that result in customer complaints or lack of satisfaction.  Many of these issues can be approached by preventive actions, that will be discussed below in a separate section.

In order to enhance the effectiveness of your Quality System, the management of the organization has to analyze the goals of the Quality Plan, continuously monitor progress towards these goals and adjust goals as needed.  This cannot be viewed as an arbitrary goal setting exercise.  It is a serious undertaking coming out of the Management Review for which resources will be made available.  The continuous enhancement of goals requires continuous review and data analysis to assess progress made throughout the year.  Achievement of small steps in the quest for improvements is the basic principle on which this effort is based.

The Quality System must be perfected to eliminate nonconformance through use of corrective and preventive actions.  The intent to continually improve the System should be part of your company’s Quality Policy.

4.5.2  Corrective Action			(Clause 8.5.2)

Corrective action is a useful tool in all quality systems.  The need for change arises when products are not meeting the desired performance of a service or manufacturing organization.  The corrective action called for in the Standard goes beyond product or service analysis.  It encompasses all the processes of the organization; starting in sales and marketing and ending at analyzing customer satisfaction with the delivery. For example a delayed shipping date can make for a very unhappy customer, even though your product meets all requirements and is in conformance with contract specifications.

Corrective action processes are usually undertaken when a condition of nonconformity occurs with certain frequency.  An occasional event does not usually trigger corrective action.  This is why data use and analysis are important first steps prior to initiating such action.  Usually the need for corrective action arises from internal nonconformity, warranty claims, product returns, customer complaints, internal audits or problems encountered with a supplier.

A documented procedure shall be part of the Quality Manual to indicate how to implement and carry out corrective actions, and how to approve the resultant requests for action.  The first step is to determine the root cause of the nonconformance and the underlying source that creates the problem.  Data analysis can indicate the following root causes:

- Inadequate procedures or documentation

- Inadequate process control

- Defective or inaccurate equipment controls

- Poor scheduling

- Insufficient training or human resources

- Inaccurate facilities and working conditions

Data can be obtained from the following sources:

- Inspection records of product and raw materials

- Operator observation during monitoring

- Internal audits

- Customer complaints regarding product or service

- Supplier changes or problems with existing ones

- Management review of quality goals

Corrective action process requires that you identify the root cause of the problem as well as various trials for its correction.  Analysis of the results of these trials will indicate if the root cause has been clearly identified and corrected and thus justify the recommended course of action.  A typical technique used is the design of experiments.  This technique is very valuable for complex problems where several variables can affect the results and is frequently utilized to optimize process variables.  Care should be taken to avoid solving one problem and generating another one further along in the process.

The analysis of the verification shall be used to assert if the solution is appropriate to the problem and if action is warranted.  The necessary action should be recorded for future audits and potential correction in the future.  One example is the need to replace a large piece of equipment that requires capital investment and for which no capital is presently available.  The corrective action results can be a very important means to justify the proposed investment.

If the corrective action requires changes in your company’s Quality Manual, process documentation or changes in the procurement specifications, these shall be approved by authorized personnel and taken in accordance with the documentation procedures described in Clause 4.2.3 (Chapter 2.1.3).  Once corrective action is implemented, a validation process regarding the actions taken is needed.  Records should be established to confirm that these actions were effective.

All these actions should be part of the Management Review process and part of the record of its activities.  If the corrective action originated due to a customer complaint, the customer should also be notified of the action and a follow up procedure established to confirm the solution of the problem and to re-establish the customer satisfaction.

4.5.3  Preventive Action			(Clause 8.5.3)

Preventive action is frequently confused with corrective action.  Both deal with the same problem: avoidance of nonconformance and improvement of customer satisfaction.  The difference is the point of departure for these two actions.  Corrective action is taken after the fact; that is, it takes place as a result of a recognized problem.  Preventive action is an action taken prior to a problem in order to prevent the problem from occurring.  A simple and useful comparison is the difference between fire prevention and putting out fires.

Quality improvement activity is usually enhanced by preventive actions. This  requires a thorough knowledge of the processes and the product, a vision of what could go wrong, and what should be done in order to prevent this from happening.  Several techniques help prevent problems.   Most of them are based on analysis of failures and their consequence.  A clear example is offered by the automotive industry, which has various types of tires and limits on the capability of their use for specific vehicles.  Accidents can be prevented when proper tire size and quality are used, although this might be more expensive.

The most common preventive action technique is the FMEA (failure mode effect analysis).  This technique, used in aerospace and the automotive industry, evaluates the potential consequence of a failure and compares its relevance to the effectiveness of a change to prevent the failure.

The preventive action procedure shall be documented in your Quality Manual and form part of the Management Review process.  Resources are usually needed and must be provided, whenever justified by the data.  Verification and validation of the results are also needed and records must be kept of these results.

Some of the preventive actions that require continuous attention are:

- Planned preventive maintenance of equipment

- Improved control equipment

- Improved training of operators

- Monitoring of operator activities to prevent mistakes

- Systematic improvement of facilities to allow easier operating conditions

- Review of schedule and plant capacity to allow upgrades before demands increase

- Implementation of new products and technology to satisfy new customer needs

The continuous improvement process is best achieved through preventive action, which is more in tune with the main goal of ISO 9001:2000:

“Complete Customer Satisfaction”